# 3GPP TSG SA WG5 Meeting #153                 	           S5-240305

# Sevilla, SPAIN, 29 Jan - 3 Feb 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0491 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Add message flow for TSC traffic charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TSN_CH |  |  |  |  |  | Date: |  |  | 2024-1-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | SMF may support the converged charging for time sensitive communication traffic via 5GS bridge. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To specify the basic principle and message flows for time sensitive communication traffic charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How to perform charging for time sensitive communication traffic via 5GS bridge is not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.3, 5.2.1.x(new), 5.2.2.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2] - [42]	Void.

[43]	3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

[44] - [50]	Void.

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[53] 	Void.

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55-56] 	Void.

[57]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[58]	3GPP TS 32.291: " Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[59] - [69]	Void.

[70]	3GPP TS 28.202: " Telecommunication management; Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[71] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]	3GPP TS 22.115: "Service aspects; Charging and billing".

[102]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[103] - [199]	Void

[200]	3GPP TS 23.501:"System Architecture for the 5G System".

[201]	3GPP TS 23.502:"Procedures for the 5G System".

[202]	3GPP TS 23.503:"Policy and Charging Control Framework for the 5G System; Stage 2".

[203]	3GPP TS 23.316: "Wireless and wireline convergence access support for the 5G System (5GS)".

[204]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[205] - [299]	Void

[300] - [399]	Void.

[400] - [499]	Void.

[500] - [599]	Void.

[600]	3GPP TS 23.228:"IP Multimedia Subsystem (IMS); Stage 2".

[601]	3GPP TS 29.512:" 5G System; Session Management Policy Control Service; Stage 3".

Next change

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

5GC	5G Core Network

5GS	5G System

ABMF	Account Balance Management Function

AF	Application Function

AMF	Access and Mobility Management Function

ATSSS	Access Traffic Steering, Switching, Splitting

AUSF	Authentication Server Function

BD	Billing Domain

CAG	Closed Access Group

CCS	Converged Charging System

CDF	Charging Data Function

CGF	Charging Gateway Function

CHF	Charging Function

CIoT	Cellular Internet of Things

CP	Control Plane

CTF	Charging Trigger Function

DNN	Data Network Name

DS_TT	Device side TSN translator

FBC	Flow Based Charging

IoT	Internet of Things

GPSI	Generic Public Subscription Identifier

GUAMI	Globally Unique AMF Identifier

MA	Multi-Access

MBS	Multicast/Broadcast Service

MB-SMF	Multicast/Broadcast Session Management Function.

MB-UPF	Multicast/Broadcast User Plane Function

MPTCP	Multi-Path TCP Protocol

MTC	Machine-Type Communications

N3IWF	Non-3GPP InterWorking Function

NE	Network Element

NEF	Network Exposure Function

NF	Network Function

NPN	Non-Public Network

NRF	Network Repository Function

NSSF	Network Slice Selection Function

NW_TT	Network side TSN translator

OCF	Online Charging Function

OCS	Online Charging System

PCC	Policy and Charging Control

PCF	Policy Control Function

PEI	Permanent Equipment Identifier

PNI-NPN	Public Network Integrated Non-Public Network

QBC	QoS flow Based Charging

QFI	QoS Flow Identifier

SCP	Service Communication Proxy

SCC	Satellite Communication Customer

SDF	Service Data Flow

SMF	Session Management Function

SMNO	Satellite Mobile network Operator

SNPN	Stand-alone Non-Public Network

SSC	Session and Service Continuity

SSP	Satellite Service Provider

SUPI	Subscription Permanent Identifier

TNAN	Trusted Non-3GPP Access Network

TNAP	Trusted Non-3GPP Access Point

TSC	Time sensitive communication

TSN	Time sensitive networking

UDM	Unified Data Management

UDR	Unified Data Repository

UPF	User Plane Function

Next change

5.2.1.x	TSC charging

5.2.1.x.1	General

The charging scenarios for the TSN domain are described in TS 32.282 [43].

The traffic data of time sensitive communication is transferred on the 5GS bridge between TSN end station and TSN system, or between TSN end stations, as shown in Figure 5.28.1-1 in TS 23.502 [201].

-	For traffic between TSN end station and TSN system, UE/DS_TT and TSN system transfers the TSN stream via the UPF/NW-TT on the 5GS bridge.

-	For traffic between TSN end stations, UE/DS_TT and another UE/DS_TT transfer the TSN stream for UE-UE communication, via the UPF/NW-TT on the 5GS bridge.

The charging for TSC traffic is based on SCUR scenario.

The charging information is collected by SMF per UE per PDU session, reported to C-CHF, and optionally forwarded to B-CHF when 5GS bridge information is included.

The C-CHF is the UE CHF and B-CHF is the tenant CHF used for TSN bridge, as described in Annex G TS 32.240 [1].

B-CHF or billing domain aggregates the charging information per 5GS bridge.

Next change

5.2.2.x	PDU session charging – TSC charging

5.2.2.x.1	General

This clause specifies the message flows for the PDU session establishment, modification, and release when TSC is supported, based on clause 5.28 in TS 23.501 [200] and clause 4.3 in TS 23.502 [201].

5.2.2.x.2	PDU session establishment

The following Figure 5.2.2.x.2-1 describes the PDU session establishment for time sensitive communication, based on Figure 4.3.2.2.1-1 TS 23.502 [202] description:

Figure 5.2.2.x.2-1: PDU Session Establishment for Time Sensitive Communication

1. UE request for PDU session establishment to support time sensitive communication on the 5GS bridge.

2 to 6. PDU session request/authentication/authorization procedures as described in Figure 4.3.2.2.1.1 TS 23.502 [201].

7. If PCF detected the request relates to TSC based on information provided by TSN AF, the PCF may forward TSC related information to the SMF as part of PCC rules.

8. If SMF received TSC related information, the SMF select UPF for a PDU Session serving TSC, as specified in clause 6.3.3.3 in TS 23.501 [200].

When the TSN stream is for UE-UE communication, the SMF applies local switching in order to enable UPF locally forward uplink stream from one PDU session as downlink stream in another PDU session, as specified in clause 5.28.2 TS 23.501 [200].

9. The SMF may collect the usage of the TSN stream and reports the charging information per UE per PDU session from UPF.

9ch-a. The SMF sends Charging Data Request [Initial, Requested Quota] to the C-CHF for starting the charging session associated with PDU session, based on the trigger event specified in clause 5.2.1.2.

9ch-b. The C-CHF creates a CDR.

(Optional) 9ch-c. If the C-CHF detects the Charging Data Request [Initial, Requested Quota] is related to B2B charging, e.g., contains 5GS bridge information, the C-CHF sends the Charging Data Request [Initial, Requested Quota] to the B-CHF.

(Optional) 9ch-d. The B-CHF creates a CDR for the start of the TSN stream.

(Optional) 9ch-e. The B-CHF acknowledges by sending Charging Data Response [Initial, Granted Quota] to the C-CHF.

9ch-f. The C-CHF acknowledges by sending Charging Data Response [Initial, Granted Quota] to the SMF.

5.2.2.x.3	PDU session modification

The following Figure 5.2.2.x.3-1 describes the PDU session modification for time sensitive communication, based on Figure 4.3.3.2-1 TS 23.502 [202] description:

Figure 5.2.2.x.3-1: PDU Session Modification for Time Sensitive Communication

2ch-a. The SMF sends Charging Data Request [Update, Requested Quota] to the C-CHF based on the trigger event specified in clause 5.2.1.2.

2ch-b. The C-CHF updates the CDR for the PDU session.

(Optional) 2ch-c. If the C-CHF detects the Charging Data Request [Update, Requested Quota] is related to B2B charging, e.g., contains 5GS bridge information, the C-CHF sends the Charging Data Request [Update, Requested Quota] to the B-CHF.

(Optional) 2ch-d. The B-CHF updates the CDR for the PDU session with TSN stream.

(Optional) 2ch-e. The B-CHF acknowledges by sending Charging Data Response [Update, Granted Quota] to the C-CHF.

2ch-f. The C-CHF acknowledges by sending Charging Data Response [Update, Granted Quota] to the SMF.

5.2.2.x.4	PDU session release

The following Figure 5.2.2.x.4-1 describes a PDU session release for time sensitive communication, based on Figure 4.3.4.2-1 TS 23.502 [202] description:

Figure 5.2.2.x.4-1: PDU Session Release for Time Sensitive Communication

2ch-a. The SMF sends Charging Data Request [Termination] to the C-CHF for terminating the charging session associated with PDU session, with the trigger event specified in clause 5.2.1.2.

2ch-b. The C-CHF closes the CDR for the PDU session.

(Optional) 2ch-c. If the C-CHF detects the Charging Data Request [Termination] is related to B2B charging, e.g., contains 5GS Bridge information, the C-CHF sends the Charging Data Request [Termination] to the B-CHF.

(Optional) 2ch-d. The B-CHF closes the CDR for the PDU session with TSN stream.

(Optional) 2ch-e. The B-CHF acknowledges by sending Charging Data Response [Termination] to the C-CHF.

2ch-f. The C-CHF acknowledges by sending Charging Data Response [Termination] to the SMF.

End of change