# 3GPP TSG-SA5 Meeting #153	S5-240805

# Sevilla, Spain, 29 January - 02 February 2024

# Source:	Nokia, Nokia Shanghai Bell, Ericsson, INTEL, NEC, Ericsson France

# Title:		Input to DraftCR 28.105 AIML Inference Output

# Document for:	Approval

# Agenda Item:	6.4.3.1 - AIML_MGMT_WoP#1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	S5-237244 DraftCR : “Draft CR AIML_MGMT - TS 28.105; Enhancements for AI-ML management”.

# 3	Rationale

The observations of the inference function and information derived Outputs is registered in the inference report. This pCR adds the NRM AI/ML inference report that captures the Outputs and the model performance at inference.

# 4	Detailed proposal

Start of First change

### 7.3a.4	Information model definitions for ML inference phase

#### 7.3a.4.1	Class diagram

##### 7.3a.4.1.1	Relationships

Figure 7.3a.4.1.1-1: NRM fragment for ML update

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

skinparam nodesep 60

class AIMLInferenceFunction <<InformationObjectClass>>

class AIMLInferenceReport <<InformationObjectClass>>

class MLEntity <<InformationObjectClass>>

AIMLInferenceFunction  "1" *-- "*" AIMLInferenceReport: <<names>>

MLEntity "1..*" <--> "*" AIMLInferenceReport

@enduml

##### 7.3a.4.1.2	Inheritance

Figure 7.3a.4.1.2-1: Inheritance Hierarchy for ML update related NRMs

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class AIMLInferenceFunction << InformationObjectClass >>

class AIMLInferenceReport <<InformationObjectClass>>

ManagedFunction <|-- AIMLInferenceFunction

Top <|-- AIMLInferenceReport

@enduml

Figure 7.3a.4.1.2-2: Inheritance Hierarchy for AI/ML inference function

Next modified section

#### 7.3a.4.2	Class definitions

##### 7.3a.4.2.5 	AiMlInferenceCapability

###### 7.3a.4.2.5.1	Definition

This IOC represents the common properties of the AI/ML inference capability.

The AiMlInferenceCapability MOI may be associated with one or more MOIs that represent the functions/functionalities (Note) that use the AI/ML inference capability represented by the subject AiMlInferenceCapability MOI.

The MOI of AiMlInferenceCapability or the MOI of the IOC inheriting from the AiMlInferenceCapability IOC contains one or more MOI(s) of MLEntity.

NOTE: 	The IOCs representing the functions/functionalities (Note) that use the AI/ML inference capability include, but are not limited to MDAFunction, AnLFFunction, DMROFunction, DLBOFunction, and DESManagementFunction.

The generation of inference outputs is based on the configuration of inference, e.g., to start a stated time, or to be executed at all times. The observations of the inference function and information on derived Outputs is registered in the inference report.

###### 7.3a.4.2.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| activationStatus | M | T | T | F | T |
| managedActivationScope | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| MLEntityRef | M | T | F | T | T |

###### 7.3a.4.2.5.3	Attribute constraints

None.

###### 7.3a.4.2.5.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3.4.2.C 	AIMLInferenceReport

7.3.4.2.C.1	Definition

This <<IOC>> represents a report from a AI/ML Inference.

- AIMLInferenceFunction may generate one or more AIMLInferenceReport(s),

- Each AIMLInferenceReport provides information about inference outputs from one or more MLEntity AIMLInferenceReport(s).

7.3.4.2.C.2	Attributes

The AIMLInferenceReport includes inherited attributes from Top IOC (defined in TS28.622 [12] ) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputs | M | T | F | F | T |
| Attributes related to role |  |  |  |  |  |
| mLEntityRef | M | T | F | F | T |

7.3.4.2.C.3 Attribute constraints

None

7.3.4.2.C.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

Next change

## 7.4	Data type definitions

##### 7.4.L 	InferenceOutput <<dataType>>

#### 7.4.L.1	Definition

This dataType represents the properties of the content of an inference output.

The inference output contains a time stamp which indicates the time at which the inference output is generated.

#### 7.4.L.2	Attributes

The InferenceOutput includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputId | M | T | F | F | T |
| inferenceType | M | T | F | F | T |
| inferenceOutputTime | M | T | F | F | T |
| inferencePerformance | O | T | F | F | T |
| outputResult | M | T | F | F | T |

Note: The relation between the Output and Outputs of other instances like MDA is not addressed in the present document

Next change

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mLEntityId | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion | It indicates the version number of the ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTraining | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - “COLLECTING_DATA” - “PREPARING_TRAINING_DATA” - “TRAINING ” + DN of the MLEntity being trained The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.cancelRequest | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingRequest.suspendRequest | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.cancelProcess | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLTrainingProcess.suspendProcess | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the ML entity for inference. | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTrainRef | It identifies the DN of the MLEntity requested to be trained. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEnityGeneratedRef | It identifies the DN of the MLEntity generated by the ML training. allowedValues: DN | Type: DN multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| mLEntityRepositoryRef | It idenfifies the DN of the MLEntityRepository. | Type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLRepositoryId | It indicates the unique ID of the ML repository. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| modelPerformanceValidation | It indicates the performance score of the ML entity when performing on the validation data. allowedValues: N/A | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| dataRatioTrainingAndValidation | It indicates the ratio (in terms of quantity of data samples) of the training data and validation data used during the training and validation process. It is represented by the percentage of the validation data samples in the total training data set (including both training data samples and validation data samples). The value is an integer reflecting the rounded number of percent * 100. allowedValues: { 0 .. 100 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityIdList | It identifies a list of ML entities. allowedValues: N/A. | type: String multiplicity: * isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| MLTestingRequest.requestStatus | It describes the status of a particular ML testing request. allowedValues: NOT_STARTED, TESTING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityToTestRef | It identifies the DN of the MLEntity requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelPerformanceTesting | It indicates the performance score of the ML entity when performing on the testing data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTestingResult | It provides the address where the testing result (including the inference result for each testing data example) is provided. The detailed testing result format is vendor specific. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| testingRequestRef | It identifies the DN of the MLTestingRequest MOI. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| supportedPerformanceIndicators | It identifies a specific PerformanceIndicator of an ML entity. allowedValues: N/A. | type: SupportedPerfIndicator multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceIndicatorName | It indicates the identifier of the specific performance indicator. allowedValues: N/A | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isSupportedForTraining | It indicates whether the specific performance indicator is supported a performance metric of ML training for the ML entity Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| isSupportedForTesting | It indicates whether the specific performance indicator is supported a performance metric of ML testing for the ML entity. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: False isUnique: True defaultValue: FALSE isNullable: False |
| mLUpdateProcessRef | It is the DN of the mLUpdateProcess MOI that represents the process of updating an ML entity. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateRequestRef | It is the DN of the MLUpdateRequest MOI that represents an ML update request. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportRef | It is the DN of the MLUpdateReport MOI that represents an ML update report. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLUpdateReportingPeriod | It specifies the time duration upon which the MnS consumer expects the ML update is reported. | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availMLCapabilityReport | It represents the available ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UpdatedMLCapability | It represents the updated ML capabilities. allowedValues: N/A. | Type: AvailMLCapabilityReport multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| newCapabilityVersionId | It indicates the specific version of AI/ML capabilities to be applied for the update. It is typically the one indicated by the MLCapabilityVersionID in a newCapabilityVersion | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mlCapabilityVersionId | It indicates the version of ML capabilities that is available for the update. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| performanceGainThreshold | It defines the minimum performance gain as a percentage that shall be achieved with the capability update, i.e., the difference in the performances between the existing capabilities and the new capabilities should be at least performanceGainThreshold otherwise the new capabilities should not be applied. Allowed value: float between 0.0 and 100.0 | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| availableMLCapability | It indicates an available version of alternative ML capabilities that can be used for update. Note: MLCapability is FFS. | type: MLCapability multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectedPerformanceGains | It indicates the expected performance gain if/when the AI/ML capabilities of the respective network function are updated with/to the specific set of newly available AI/ML capabilities. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| achievedPerformanceGain | It indicates the performance gain that is achieved by updating the AI/ML entity or network function. | Type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| updateTimeDeadline | It indicates the maximum as stated in the MLUpdate request that should be taken to complete the update | Type: TimeWindow (see TS 28.622 [12]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityRef | It indicates the list of references to MLEntity instances that can be updated. | Type: DN multiplicity: 1 .. * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| MLUpdateRequest.requestStatus | It describes the status of a particular ML update request. allowedValues: NOT_STARTED, IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| memberMLEntityRefList | It identifies the list of member ML entities within a level of an ML entity coordination group. allowedValues: DN list | Type: DN (see TS 32.156 [13]) multiplicity: 2..* isOrdered: True isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTrainRef | It identifies the DN of the MlEntityCoordinationGroup requested to be trained. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEnityCoordinationGroupGeneratedRef | It identifies the DN of the MlEntityCoordinationGroup generated by the ML training. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLEntityCoordinationGroupToTestRef | It identifies the DN of the MlEntityCoordinationGroup requested to be tested. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 0..1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| retrainingEventsMonitorRef | It indicates the DN of the ThresholdMonitor MOI that indicates the performance measurements and its corresponding thresholds to be used by MnS producer to initiate the re-training of the MLEntity. | Type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sourceTrainedMLEntityRef | It identifies the DN of the source trained MLEntity whose copy has been loaded from the ML entity repository to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingRequest.requestStatus | It describes the status of a particular ML entity loading request. allowedValues: NOT_STARTED, LOADING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLEntityLoadingRequest.cancelRequest | It indicates whether the MnS consumer cancels the ML entity loading request. Setting this attribute to "TRUE" cancels the ML entity training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " LOADING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| mLEntityToLoadRef | It identifies the DN of a trained MLEntity requested to be loaded to the target inference function(s). | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| policyForLoading | It provides the policy for controlling ML entity loading triggered by the MnS producer. This policy contains two thresholds in the thresholdList attribute. The first threshold is related to the ML entity to be loaded, and the second threshold is related to the existing ML entity being used for inference. | Type: AiMlManagementPolicy multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdList | It provides the list of threshold. | Type: ThresholdInfo (see TS 28.622 [12]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingProcess.cancelProcess | It indicates whether the MnS consumer cancels the ML entity loading process. Setting this attribute to "TRUE" cancels the process. Cancellation is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.suspendProcess | It indicates whether the MnS consumer suspends the ML entity loading process. Setting this attribute to "TRUE" suspends the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingProcess.resumeProcess | It indicates whether the MnS consumer resumes the ML entity loading process. Setting this attribute to "TRUE" resumes the process. Suspension is possible when the "MLEntityLoadingProcess.progressStatus.status" is not the "SUSPENDED". Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| MLEntityLoadingRequestRef | It identifies the DN of the associated MLEntityLoadingRequest. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| MLEntityLoadingPolicyRef | It identifies the DN of the associated MLEntityLoadingPolicyRef. allowedValues: DN. | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| performanceMetricName | It identifies the name of the performance metric or measurement. AllowedValues: the name of the performance metrics specified for the ML training phase and AI/ML inference phase. | Type: String multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| thresholdDirection | It indicates the direction of a threshold indicating the direction for which a threshold crossing triggers a threshold. When the threshold direction is configured to "UP", the associated threshold is triggered only when the subject performance metric value is equal to or greater than the threshold value. The threshold is not triggered, when the performance metric value is smaller than the threshold value. Vice versa, When the threshold direction is configured to "DOWN", the associated threshold is triggered only when the subject performance metric value is equal or smaller than the threshold value. The threshold is not triggered, when the performance metric value s greater than the threshold value. allowedValues: - UP - DOWN | Type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdValue | It specifies the value against which the monitored performance metric is compared. allowedValues: float or integer | Type: Union multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| LoadedMLEntityRef | It identifies the DN of the MLEntity that has been loaded to the inference function. allowedValues: DN | Type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: True |
| activationStatus | It describes the activation status. allowedValues: ACTIVATED, DEACTIVATED. | Type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| managedActivationScope | It provides a list of sub scopes for which ML inference is activated as triggered by a policy on the MnS producer. For example, the sub scopes may be a list of cells or of geographical areas. The list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope if the policy evaluates to true. allowedValues: N/A | Type: ManagedActivationScope multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.dNList | It indicates the list of DN, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: DN multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.timeWindow | It indicates the list of time window; the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: TimeWindow (see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| ManagedActivationScope.geoPolygon | It indicates the list of GeoArea, the list is an ordered list indicating the inference is activated for the first sub scope and gradually extended to the next sub scope. allowedValues: N/A | Type: GeoArea(see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| inferenceOutputId | It identifies an inference output within an AIMLinferenceReport | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| aimlInferenceOutputs | It indicates the Outputs that have been derived by the MLInferenceFunction instance from a specific ML entity. Each MLentity, mLinferenceOutputs may be a set of values allowedValues: N/A. | type: InferenceOutput multiplicity:f 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferencePerformance | It indicates the performance score of the ML entity during Inference. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| inferenceOutputTime | It indicates the time at which the inference output is generated.. allowedValues: N/A | Type: DateTime (see TS 28.622 [12]) multiplicity: * isOrdered: True isUnique: True defaultValue: None isNullable: False |
| outputResult | It indicates the result of an inference | type: AttributeValuePair (see TS 32.156) multiplicity: * isOrdered: FALSE isUnique: TRUE defaultValue: Null isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML testing, the data set is the testing data set. |  |  |

End of Changes