# 3GPP TSG-SA5 Meeting #153	S5-240826

# Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to Draft CR 28.105 Modifications on ML entity validation performance reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-01-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current discription of the validation procedure in the ML entity validation performance reporting is not generic. There are different kinds of validation methods such as leave out validation, cross validation, etc. Therefore, there is a necessity to adjust the discriptions and add some more instructions to avoid the misunderstanding that there would be only leave out validation used during validation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adjust the discription of validation to make it more generic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The commonality of validation would be loss. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.1.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-240605 |  |  |  |  |  |  |  |  |

First modification

6.2	Void

6.2a	ML training phase

6.2a.1	ML model training

6.2a.1.2	Use cases

6.2a.1.2.7	ML entity validation performance reporting

During the ML training process, the generated ML entity needs to be validated. The purpose of ML validation is to evaluate the performance of the ML entity when performing on the validation data, and to identify the variance of the performance on the training data and the validation data. The training data and validation data are of the same pattern as they normally split from the same data set with a certain ratio in terms of quantity of the data samples.

In the ML training, the ML entity is generated based on the learning from the training data, and validated using the validation data. The performance of the ML entity has tight dependency on the data (i.e., training data) from which the ML entity is generated. Therefore, an ML entity performing well on the training data may not necessarily perform well on other data e.g., while conducting inference. If the performance of ML entity is not good enough according to the result of ML validation, the ML entity will be tuned (the model associated with it be re-trained) and validated again. The process of ML entity tuning and validation is repeated by the ML training function, until the performance of the ML entity meets the expectation on both training data and validation data.

The MnS producer subsequently selects one or more ML entities with the best level of performance on both training data and validation data as the result of the ML training, and reports accordingly to the consumer. The performance of each selected ML entity on both training data and validation data also needs to be reported.

The performance result of the validation may also be impacted by the ratio of the training data and the validation data. In general, there are different methods to validate the ML entity including hold-out validation, cross-validation, etc.  MnS consumer needs to be aware of the characteristics of validation including the methods of validation and the ratio of training data and the validation data, coupled with the performance score on each data set, in order to be confident about the performance of ML entity.

6.2a.1.3	Requirements for ML training

Table 6.2a.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to request ML training. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the inference type of the ML entity to be trained. | ML training requested by consumer (clause 6.2a.1.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result to the MLT MnS consumer. | ML training requested by consumer (clause 6.2a.1.2.1), ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-05 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML entity. (See Note) | ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-06 | The MLT MnS producer shall have a capability to provide the version number of the ML entity and the time when it is generated by ML re-training to the authorized MLT MnS consumer. | ML training requested by consumer (clause 6.2a.1.2.1), /ML training initiated by producer (clause 6.2a.1.2.2) |
| REQ- ML_TRAIN-FUN-07 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML training. | ML training requested by consumer (clause 6.2a.1.2.1), ML training initiated by producer (clause 6.2a.1.2.2), ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-08 | The MLT MnS producer should have a capability to provide the grouping of ML entities to an authorized MLT MnS consumer to enable coordinated inference. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The MLT MnS producer should have a capability to allow an authorized MLT MnS consumer to request joint training of a group of ML entities. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ- ML_TRAIN-FUN-10 | The MLT MnS producer should have a capability to jointly train a group of ML entities and provide the training results to an authorized consumer. | ML entity joint training (clause 6.2a.1.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to discover the characteristics of available ML entitiesincluding the contexts under which each of the models associated with the entities were trained. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to select an ML entity. | ML models and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have a capability to enable an authorized MLT MnS consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2a.2.1), ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request for information and be informed about the available alternative ML entities of differing complexity and performance. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML entity to the authorized MLT MnS consumer. | ML model and ML entity selection (clause 6.2a.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML training, e.g. to modify the characteristics of the request or to delete a request. | ML training requested by consumer (clause 6.2a.2.1),Managing ML Training Processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-02 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | ML training requested by consumer (clause 6.2a.2.1), Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer (e.g. the function/entity different from the function that generated a request for ML training) to request for a report on the outcomes of a specific training instance. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the MLT function to report to any authorized MLT MnS consumer about specific ML training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2a.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML-inference function. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the inference function. | Handling errors in data and ML decisions (clause 6.2a.2.5) |
| REQ-ML_VLD-01 | The MLT MnS producer should have a capability to validate the ML entities during the ML training process and report the performance of the ML entities on both the training data and validation data to the authorized consumer. | ML entity validation performance reporting (clause 6.2a.1.2.7) |
| REQ-ML_VLD-02 | The MLT MnS producer should have a capability to report the characteristics of validation including methods of validation and the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML training and validation process. | ML entity validation performance reporting (clause 6.2a.1.2.7) |
| REQ-TRAIN_EFF-01 | The 3GPP management system should have the capability to allow an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Training data effectiveness reporting (clause 6.2a.1.2.8) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML entity supports) of ML entity. |  |  |

End of modifications