3GPP TSG-SA5 Meeting #153 	S5-240228

Sevilla, Spain, 29 January - 02 February 2024

Source:                        ZTE Corporation

Title:                             pCR TR 28.914 Add Key Issues of Implicit Intent Report Subscription

Document for:             Approval

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	Study on intent driven management service for mobile network phase 3 v0.0.0

[2]	SP-231737 "Study on intent driven management services for mobile network phase 3"

3	Rationale

NtfSubscriptionControl IOC is used for MnS consumer to subscribe the intent report information explicitly in TS 28.312 [1], where the MnS Consumer sends a request to create an NtfSubscriptionControl instance. WT-3.3 of the FS_IDMS_MN_Ph3 SID [2] is to investigate implicit intent report subscription that can be triggered by intent creation request automatically.

4	Detailed proposal

Start of First change

4	Issue investigations and potential solutions for new areas

Editor's note: this clause will contain issues and potential solutions related to the new scenarios, requirements and generic capability for intent driven management.

4.x	Issues#4.X: Implicit IntentReport Subscription

4.x.1		Description

Intent-driven MnS Consumer may want to express an intent to obtain intent report instead of requesting to create an NtfSubscriptionControl instance. This requirement of Implicit IntentReport Subscription also meets the motivation of introducing intent-driven management service, increasing the autonomy and efficiency of network management.

By expressing the requirements of IntentReport subscription in an intent, the MnS Consumer can customize the content of an IntentReport. For example, the MnS Consumer can specify the targets that need to be contained in IntentReports and the events that will trigger automatic IntentReport. Furthermore, the MnS Consumer can define the specific conditions that the MnS Producer needs to send an IntentReport. For example, the MnS Consumer can ask the producer to send an IntentReport when a specific target crosses the threshold.

This study is to investigate potential requirements and solution(s) to extend the generic information model of intent IOC to support Implicit IntentReport Subscription.

4.x.2	Potential requirements

REQ-Intent_Report_01:  The intent-driven MnS Consumer shall have the capability to express requirements for IntentReport in an Intent.

REQ-Intent_Report_02:  The intent-driven MnS Consumer shall have the capability to define the specific conditions that the MnS Producer needs to send an IntentReport.

End of Second change