3GPP TSG-SA5 Meeting #153	S5-240320

Sevilla, Spain, 29 January - 02 February 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0494 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.255 Clarification on the C-CHF and B-CHF communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NETSLICE_CH_Ph2 |  |  |  |  |  | Date: |  |  | 2024-01-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the Network slice charging message flow, the UE CHF and Tenant CHF refer to the C-CHF and B-CHF architecture specified in clause 4.3. Propose to use the uniform CHF name in the charging specifications. The remaining editor’s note should be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the name of UE CHF to C-CHF, Tenant CHF to B-CHF. Refine the message flow description and remove the editor’s note. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for network slice charging message flow is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.21.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.21.2	5G data connectivity charging per network slice

The clause describes the charging procedure for Network slice converged charging using the C-CHF and B-CHF communication architecture specified in the clause 4.3.

Figure 5.2.2.21.2-1: Network slice converged charging

1.	Network slice charging for network slice subscriber per S-NSSAI is configured in C-CHF and B-CHF, which may include S-NSSAI, Subscriber identifier and Tenant ID, specified in the TS 28.202 [70], etc. The charging message flow of PDU session establishment is based on Figure 5.2.2.2.2-1 description with the differences of the interaction between C-CHF and B-CHF.

2.	According to clause 5.2.2.2.2 PDU Session charging from SMF about PDU session establishment for individual UE charging.

2ch-a.	The SMF creates a Charging Identifier for the PDU session, and sends Charging Data Request [Initial], triggered by "Start of PDU session" to C-CHF.

2ch-b.	The C-CHF opens CDR for this PDU session per UE.

2ch-c. C-CHF sends Charging Data Request [Initial] to B-CHF which may include Tenant ID and UE PDU session charging information based on S-NSSAI and DNN received from SMF for Network Slice Charging. Account, Rating and reservation control for the Tenant.

2ch-d.	 The B-CHF opens CDR for this PDU session per S-NSSAI.

2ch-e.	 B-CHF sends Charging Data Response [Initial] for Tenant.

2ch-f.	C-CHF sends Charging Data Response [Initial] for the individual UE.

3.	During the PDU session, the charging message flow of PDU session modification is based on Figure 5.2.2.2.3-1 description with the differences of the interaction between C-CHF and B-CHF.

3ch-a.	The SMF sends Charging Data Request [Update] to C-CHF.

3ch-b.	The C-CHF updates CDR for this PDU session per UE.

3ch-c. C-CHF sends Charging Data Request [Update] to B-CHF with S-NSSAI, Tenant ID (optional) and UE PDU session charging information for Network Slice Charging.

3ch-d.	 The B-CHF updates CDR for this PDU session per S-NSSAI.

3ch-e.	 B-CHF sends Charging Data Response [Update] for Tenant.

3ch-f.	C-CHF sends Charging Data Response [Update] for the individual UE.

4.	According to clause 5.2.2.2.2 PDU Session charging about PDU session release from SMF for individual UE charging. The charging message flow of PDU session release is based on Figure 5.2.2.2.4-1 description with the differences of the interaction between C-CHF and B-CHF.

4ch-a.	The SMF sends Charging Data Request [Termination], triggered by "End of PDU session" to C-CHF.

4ch-b.	The CHF close CDR for this PDU session per UE.

4ch-c. C-CHF sends Charging Data Request [Termination] to B-CHF with S-NSSAI, Tenant ID (optional) and UE PDU session charging information for Network Slice Charging.

4ch-d.	 The CHF close CDR for this PDU session per S-NSSAI.

4ch-e.	 B-CHF sends Charging Data Response [Termination] for Tenant.

4ch-f.	C-CHF sends Charging Data Response [Termination] for the individual UE.

End of change