3GPP TSG-SA5 Meeting #153	S5-240417

# Sevilla, Spain, 29 Jan- 2 Feb 2024

# Source:	Huawei

# Title:	pCR 28.830 Add conclusion of issue 2 potential enchancements for fault related analysis

# Document for:	Approval

Agenda Item:	6.7.3.3

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 "Fault supervision evolution"

3	Rationale

This tdoc addresses the conclusion of issue2 potential enhancements for fault related analysis

4	Detailed proposal

5.2	Issue 2: Potential enhancements for fault related analysis

5.2.1	Description

If a potential fault/failure is predicted and reported to the consumer, the consumer would like to know the consequence. More analysis information on 3GPP system may be provided for the consumer to perform more proper actions, e.g., performance degradation analysis and predictions, KPI anomaly analysis and predictions, etc.

For example, the impacts on 3GPP system may not be perceived significantly in densely populated urban areas if there are overlapping coverage when a few sites encounter faults. However, service outage may occur due to faults in a site if there are few overlapping coverage in suburban areas. If this kind of information can be provided, the different handlings may be performed by the consumer.

5.2.2	Potential solutions

5.2.2.1	Potential solution 1: Failure prediction enhancement

5.2.2.1.1	Introduction

Some potential enhancements to failure prediction are provided. The concrete potentialFailureType is defined as standardized value of the alarmType in TS 28.532 [5].

5.2.2.1.2	Description

Failure of service and network functions may occur during the network operation. It is necessary to predict potential failures and prevent more severe impacts. The MDA capability of failure prediction has been specified in TS 28.104 [11], the analytics output are as follows:

-	failurePredictionObject;

-	potentialFailureType;

-	eventTime;

-	issueID;

-	perceivedSeverity;

The potentialFailureType in the failure prediction analytics output need to be defined more concretely. The potentialFailureType may reference to the standardized AlarmType. The MDA service may coordinate with fault supervision for identification and analysis of alarm types. The attribute name is also modified to predictedFailureType.

Table 5.2.2.1.2-1: Analytics output for failure prediction

| Attribute Name | Description |
| --- | --- |
| predictedFailureType | Indication of type of issues that can cause the failures. NOTE 1: The values can be defined as a list of values of the alarmType described in TS 28.532 [5]. |

5.2.2.1	Potential solution 2: Fault impact analysis

5.2.2.2.1	Introduction

The MDA type of failure prediction may need to provide fault impact analysis information.

5.2.2.2.2	Description

The scope and degree of fault impacts are evaluated and provided in MDA type of failure prediction in TS 28.104 [11]. It may contain the following aspects:

-	Scope and service types which are impacted by the fault.

-	A list of managed objects which are impacted by the fault.

Table 5.2.2.2.2-1: MDA analytics output of fault impacts in failure prediction

| Attribute Name | Description |
| --- | --- |
| affectScope | Coverage areas which are affected by the fault, e.g. a list of cells, a list of tracking areas (TAs), etc. |
| affectService | Service types which are affected by the fault, e.g. the VoNR, URLLC service types. |
| affectNumOfPDUSessions | Number of PDU sessions which are affected by the fault. |
| affectManagedObjects | The object instances which are impacted by the fault, e.g. network slice, network slice subnet, network elements, network functions, a list of gNBs, etc. |

5.2.2.3	Potential solution 3: Fault cause analysis enhancement

5.2.2.3.1	Introduction

The MDA may provide capability of probableCause analysis.

5.2.2.3.2	Description

In TS 28.532 [5], the alarm notification can provide information of probableCause. The attribute probableCause uses the definitions in ITU-T Rec. X. 733 [4]. The MDA analytics report may add the probableCause, therefore the fault supervision management service producer may use this probableCause of MDA analytics output for the next coming alarm notification of the same alarm type. The values can refer to the Probable Causes list in Annex A (normative) in TS 28.111 [10]. The attribute of probableCause may be added to existing MDA Type MDAAssistedFaultManagement.FailurePrediction.

Table 5.2.2.3.2-1: MDA analytics output of probableCause in failure prediction

| Attribute Name | Description |
| --- | --- |
| probableCause | Provide the analysis report of the probableCause as listed in Annex A (normative) in TS 28.111 [10]. |

1st Change

5.2.3 Conclusion - Impact on normative work

It is proposed to update TS 28.104 [11] to add the attributes described above in Table 5.2.2.1.2-1, Table 5.2.2.2.2-1, and Table 5.2.2.3.2-1.

End of change