3GPP TSG-SA5 Meeting #153 	S5-240458

Sevilla, Spain, 29 January - 02 February 2024

# Source:	6.1

# Title:	Support for MCE ID

# Document for:	Information, Discussion

Agenda Item:

3GPP TSG-RAN WG3 Meeting #122	R3-238003

# Chicago, USA, November 13th – November 17th 2023

Title:	Support for MCE ID

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	RAN3

To:	SA5, RAN2

Cc:	SA3

Contact Person:	Filip Barac

filip.barac@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

As part of Rel-18 support for QoE, RAN3 discussed how to ensure the continuity of QoE measurements across all RRC states. When a gNB releases a UE configured with QMC to RRC_IDLE, the gNB needs to provide to the UE an identifier of the Measurement Collector Entity (MCE). Upon return from RRC_IDLE to RRC_CONNECTED state, the UE should provide an identifier of the MCE to the new gNB, so that the re-connecting gNB can forward the QoE reports to the right MCE.

For security reasons, RAN3 has agreed that the identifier of the MCE stored at the UE should not be the MCE IP address, but rather an identifier similar to the Trace Collector Entity (TCE) ID that the OAM provides to the gNB for MDT.

Upon transitioning from RRC_IDLE to RRC_CONNECTED, the UE is expected to provide to the network the MCE ID. Based on the MCE ID and a pre-configured mapping, the new gNB can determine the IP address of the MCE.

# 2	Actions

To SA5: RAN3 respectfully asks SA5 to provide specification support for an MCE Identifier, similar to the TCE ID for logged MDT case, and to provide this information to the gNB as part of a QoE configuration, both for signalling based QoE and management based QoE.

To RAN2: RAN3 respectfully requests RAN2 to support the following in their specifications:

- The gNB sending the MCE ID to the UE and ensuring that the UE maintains the MCE ID while in RRC_IDLE state.

- The UE sending the MCE ID to the gNB upon reconnecting from RRC_IDLE.

# 3	Date of next TSG RAN WG3 meeting

RAN3#123                         February 26th – March 03rd, 2024 – Athens, Greece