# 3GPP TSG SA WG5 Meeting #153                 	           S5-240542

# Sevilla, Spain, 29 Jan-02 Feb 2024

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0505 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the message flow for the dynamic satellite backhaul charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 7.4.11 |  |  |  |  |  | Date: |  |  | 2024-1-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Since the satellite backhaul category will be changed when the dynamic satellite backhaul is used, the message flow for the dynamic satellite backhaul should be added. Update the charging conditions for the satellite backhaul charging in clause 5.2.1.6 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Update the charging conditions for the satellite backhaul charging in clause 5.2.1.6 2. Add the message flow for the dynamic satellite backhaul charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of the satellite backhaul charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1.6, 5.2.2.20 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR … CR |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.1.6	QoS flow Based Charging (QBC)

QoS flow Based Charging allows the SMF to collect charging information related to data volumes per PDU session, categorized per QoS Flow. QBC doesn't support quota management.

The user can be identified by SUPI.

For a given PDU session, QBC shall be performed by the SMF within the same charging session used for Flow Based Charging. For the case where QBC is performed from SMF in VPLMN, Flow Based Charging is not applicable and there is no possibility to have quota management for the PDU Session. For the case where QBC is performed from SMF in HPLMN, FBC can be performed or not performed at the same time according to operator's policy.

The SMF categorizes the volume within PDU session by QoS Flow identified by QoS Flow Identifier (QFI).

The amount of data counted for the QoS Flow shall be the user plane payload at the UPF.

Table 5.2.1.6.1 summarizes the set of default trigger conditions and their category which shall be supported by the SMF in QBC. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Update, Termination] message sent from SMF towards the CHF.

Table 5.2.1.6.1: Default Chargeable events in SMF for QBC

| Chargeable event | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of PDU session | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Start of a QoS Flow | QoS Flow | Deferred | Not Applicable | Not Applicable | Charging Data Request [Update] |
| Change of Charging conditions |  |  |  |  |  |
| QoS change | QoS Flow | Deferred | Yes | Yes |  |
| GFBR guaranteed status change | QoS Flow | Deferred | Yes | Yes |  |
| User Location change | PDU session | Deferred | Yes | Yes |  |
| Serving Node change | PDU session | Deferred | Yes | Yes |  |
| Change of 3GPP PS Data off Status | PDU session | Deferred | Yes | Yes |  |
| Tariff time change | PDU session | Deferred | No | No |  |
| UE time zone change | PDU session | Immediate | Yes | Yes |  |
| PLMN change | PDU session | Immediate | Yes | Yes |  |
| RAT type change | PDU session | Immediate | Yes | Yes |  |
| Session-AMBR change | PDU session | Immediate | Yes | Yes |  |
| Addition of UPF | PDU session | Immediate | Yes | Yes |  |
| Removal of UPF | PDU session | Immediate | Yes | Yes |  |
| Handover cancel | PDU session | Immediate | Yes | Yes |  |
| Handover start | PDU session | Immediate | Yes | Yes |  |
| Handover complete | PDU session | Immediate | Yes | Yes |  |
| Redundant transmission change | QoS Flow | Immediate | Yes | Yes |  |
| Satellite backhaul category change | PDU session | Deferred | Yes | Yes |  |
| Satellite Backhaul QoS change | QoS Flow | Deferred | Yes | Yes |  |
| GEO satellite ID change | PDU session | Deferred | Yes | Yes |  |
| Limit per PDU session |  |  |  |  |  |
| Expiry of data time limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of data volume limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of data event limit per PDU session | PDU session | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | PDU session | Immediate | No | Yes |  |
| Limit per QoS Flow |  |  |  |  |  |
| Expiry of data time limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Expiry of data volume limit per QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Others |  |  |  |  |  |
| End of QoS Flow | QoS Flow | Deferred | Yes | Yes |  |
| Management intervention | PDU session | Immediate | No | No | Charging Data Request [Update] Charging Data Request [Termination] |
| V-SMF change | PDU session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] Charging Data Request [Termination] |
| End of PDU session | PDU session | Immediate | No | No | Charging Data Request [Termination] |
| Abort request is received from the CHF | PDU session | Immediate | No | No |  |
| NOTE 1: If GFBR guaranteed status change is enabled, SMF needs to ensure the request for the notification from the access network (i.e. 3GPP RAN) when the GFBR can no longer (or can again) be guaranteed for a QoS Flow during the lifetime of the QoS Flow. NOTE 2: The columns CHF allowed to change category, and CHF allowed enable and disable are only applicable for the PDU session establishment, for other cases they are not applicable. |  |  |  |  |  |

Second change

5.2.2.20	Satellite backhaul charging

5.2.2.20.1	General

The support of satellite backhaul is specified in TS 23.501 [200] and TS 23.502 [201], message flows for different scenarios:

-	Satellite backhaul charging.

-	Edge Computing charging with satellite backhaul.

-	SCC-to-SCC communications charging via satellite backhaul.

5.2.2.20.2	Message flows for different scenarios

5.2.2.20.2.1	Satellite backhaul charging

The charging message flow of PDU session establishment for supporting Satellite backhaul, is based on Figure 5.2.2.2.2-1 description with the differences identified in clause 4.3.2.2.1.1 TS 23.502 [202].

9ch-a,	If SMF received the satellite backhaul category in step1 in clause 4.3.2.2.1.1 TS 23.502 [202], SMF sends the Charging Data Request [Initial] including satellite backhaul category and satellite backhaul usage report to CHF for the satellite backhaul charging.

16ch-a,	The SMF sends Charging Data Request[Update] including the satellite backhaul usage report and the corresponding current satellite backhaul category to the CHF.

The charging message flow of PDU session Release for supporting Satellite backhaul, is based on Figure 5.2.2.2.4-1 description:

2ch-a, 	The SMF sends Charging Data Request [Termination] to CHF including the satellite backhaul traffic usage report and the corresponding current satellite backhaul category.

5.2.2.20.2.2	Dynamic Satellite backhaul charging

If dynamic satellite backhaul is used, e.g. the satellite backhaul category changes from the MEO to LEO, the capabilities (latency) of the satellite backhaul change over time due to use of varying inter-satellite links as part of backhaul. Then, the AMF notifies the SMF of the corresponding dynamic satellite backhaul category to serve the PDU Session. SMF reports the current satellite backhaul category to the CHF using Charging Data Request[Update]. The figure 5.2.2.20.2.2-1 describes the high level charging procedure for SMF (CTF) Converged charging for dynamic satellite backhaul charging.

Figure 5.2.2.20.2.2-1: Message flow for dynamic satellite backhaul charging

1.	SMF received the PDU session establishment request including satellite backhaul category (e.g. MEO) as specified in clause 4.3.2.2 TS 23.502 [3].

1ch-a. The SMF sends Charging Data Request [Initial] including satellite backhaul category (e.g. MEO) and satellite backhaul traffic usage report to CHF for the satellite backhaul charging.

1ch-b. The CHF creates a CDR.

1ch-c. The CHF acknowledges by sending Charging Data Response [Initial] including satellite backhaul category (e.g. MEO) to the SMF.

2.	SMF sends the satellite backhaul usage report trigger to UPF during N4 Session Establishment/Modification procedure.

3.	The SMF obtains the new satellite backhaul category (e.g. LEO) or satellite backhaul category changes from AMF, the SMF initiates SM policy Association Modification procedure.

3ch-a. The SMF sends Charging Data Request [Update] including current satellite backhaul category(e.g. LEO),  Satellite Backhaul QoS and satellite backhaul traffic usage report to the CHF.

3ch-b. The CHF updates a CDR.

3ch-c. The CHF acknowledges by sending Charging Data Response [Update] including current satellite backhaul category(e.g. LEO), Satellite Backhaul QoS and current satellite backhaul traffic usage report to the SMF.

4.	Start PDU session release procedure.

5.	UPF/PSA UPF reports the usage reporting of the last satellite backhaul traffic for the current satellite backhaul category to SMF.

5ch-a. The SMF sends Charging Data Request [Termination] including current satellite backhaul category(e.g. LEO), Satellite Backhaul QoS and current satellite backhaul traffic usage report to CHF.

5ch-b. The CHF closes a CDR.

5ch-c. The CHF acknowledges by sending Charging Data Response [Termination] to the SMF.

6.	Continue PDU session release procedure.

5.2.2.20.2.3	Edge Computing charging with satellite backhaul

The message flow is based on the charging principles for 5GS usage for Edge Computing specified in clause 5.1.2 of the TS 32.257 [14] with the enhancement of the satellite backhaul information for the PDU session charging information.

5.2.2.20.2.4	SCC-to-SCC communications charging via satellite backhaul

The message flow is similar as the message flows for PDU session establishment, modification and release in clause 5.2.2.2 in TS 32.255 [10] with the enhancement of the satellite backhaul information for the PDU session charging information.

End of change