3GPP TSG-SA5 Meeting #153	S5-240863

# Sevilla, Spain, 29th January- 2nd February 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Rel-18 pCR TS28.319 on concept of NETCONF Solution for Access control for management service

# Document for:	Approval

Agenda Item:	6.6.1.3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

None

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TS 28.319

Begin of modifications

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[aa]	IETF RFC 8341: " Network Configuration Access Control Model"

Next modification

## A.2	NETCONF/YANG solution set

.

The NETCONF/YANG solution is based on IETF RFC 8341 [aa].

The YANG module "ietf-netconf-acm" specifies the datastore for configuring the access control rules into the NETCONF server. Table A.2-1 specifies the mapping of the stage 2 class names to the YANG node names.

Table A.2-1: Mapping stage 2 class names to YANG node names

| Stage 2 class name | YANG node name |
| --- | --- |
| Identity | user-name |
| Role | group |
| AccessRule | rule |

The NETCONF server enforces the access control rules.

User authentication is not handled by NACM, but by other processes depending on how the user connects, for example over TLS.

End of changes