3GPP TSG-SA5 Meeting #153	S5-240919

# Sevilla, Spain, 29 Jan- 2 Feb 2024

# Source:	Huawei

# Title:	pCR 28.830 Add description of relation with performance management

# Document for:	Approval

Agenda Item:	6.7.3.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 "Fault supervision evolution"

3	Rationale

This tdoc discusses the relation with performance management in FSEV study [1, 2].

WoP#1 of the FS_FSEV study item is as follows: “1. The relationship between fault supervision evolution and other aspect, e.g. performance management”. There were some discussions about threshold crossing notifications in previous meetings. This issue has not been addressed in the TR.

The "notifyThresholdCrossing" notification builds the connection between fault supervision management service and performance assurance management service. The "notifyThresholdCrossing" notification is generated when the monitored performance data exceeds the threshold range.

The threshold may be configured by the MnS consumer of performance assurance management service. It may also be some policies or AI/ML models which the MnS producer may apply to determine when the "notifyThresholdCrossing" notification should be generated. The MDA assisted fault analysis capability may be utilized to report more accurate threshold settings.

Proposal 1: It is proposed to include the relation of fault supervision management service with performance assurance management service in the TR.

4	Detailed proposal

1st Change

7	Relation between the fault supervision management service and the performance assurance management service

The "notifyThresholdCrossing" notification builds the connection between fault supervision management service and performance assurance management service. In TS 28.532 [5],the "notifyThresholdCrossing" notification is utilized to describe the performane threshold assurance service  when the monitored performance data exceeds the threshold range; meanwhile one attribute " thresholdinfo" in the definition of type NotifyNewAlarm in fault supervision management service, provides information for threshold crossing alarms.

The threshold may be configured by the MnS consumer of performance assurance management service. Mutiple dimension data based approaches might be used to calculate threshold values,  and the performance assurance management service producer may apply to determine when the "notifyThresholdCrossing" notification should be generated.

End of change