3GPP TSG-SA5 Meeting #153 	S5-241032

Sevilla, Spain, 29 January - 02 February 2024                      revision of S5-240497

# Source:	Ericsson, Deutsche Telekom, Huawei

# Title:	Discussion paper on management support for 5G Core

# Document for:	Approval

Agenda Item:	6.1

# 1	Decision/action requested

The group is asked to endorse the text proposal in section 4.

# 2	References

[1]	3GPP TS 23.501 System architecture for the 5G System (5GS)

[2]	3GPP TS 29.510. 5G System; Network function repository services; Stage 3.

[3]	3GPP TS 28.541  Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3

[4]	3GPP TS 23.229 IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3

[5]	3GPP TS 24.237 IP Multimedia (IM) Core Network (CN) subsystem IP Multimedia Subsystem (IMS) service continuity; Stage 3

[x]	3GPP TS 28.705 Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

[y]	3GPP TS 23.401 General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

[z]	3GPP TS 29.213 Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping

[a]	3GPP TS 33.535 Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)

[b]	3GPP TS 28.708 Telecommunication management; Evolved Packet Core (EPC) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

# 3	Rationale

This contribution compares the level of management support for a 5GC as described in TS 23.501, see reference [1] and the named network functions and entities as specified in TS 29.510, see reference [2]. The network functions and entities supported by the management system can be found in the 5G NRM in TS 28.541, see reference [3] clause 5.

Editor’s Note:  other TS specifying other 5G network functions and entities (for example Edge and RAN) are FFS.

For a complete overview 5GC there are the following groups of network function and entities:

- 5GC (without IMS) network functions and entities

- IMS (5GC IMS and IMS) network functions and entities

- 5GC (combined EPC-5GC network functions) interworking with EPC network functions and entities

- network functions and entities included in TS 29.510 but outside 5GC

In a 5G system the Network Repository Function, NRF is the single point of registration for all network functions and entities in a 5G system.  The NRF therefore needs to know the names (stage 3) of all the network functions and entities that can register with a 5GC, these names can be found in TS 29.510 clause 6.6.6.3.

A 5GC supports IMS but not all the IMS network functions and entities have to be registered in NRF. Therefore, the network functions and entities listed in TS 29.501 does not give the full for IMS view. To make the list for IMS complete the network functions and entities of following specifications have been used, IMS in TS 24.229, see reference [4] and SRVCC (Single Radio Voice Call Continuity) from 5GS to 3G in TS 24.237, see reference [5].

In the management domain the stage 3 name of a network function or entity is mapped to a class that is contained by and inherits from of the abstract class ManagedFunction, for example the network function NRF is mapped to managed function with class name NRFFunction.

Table 3.1 compiles the information on 5GC network functions and entities specified in TS 29.510 [2] clause 6.1.6.3.3 in the 1st column, while the 2nd column shows the inventory of the corresponding managed functions which are taken from TS 28.541 [3] clause 5. The 2nd column is split into one column using the Managed function class name when defined in management domain and one column with description of network function or entity.

Table 3.1 Overview of 5GC network functions and entities, and managed functions

| Network function [2] | Network function management model inventory |  |
| --- | --- | --- |
|  | Managed function name [3] | Description |
| Network Function: NRF | NRFFunction | Network Repository Function |
| Network Function: UDM | UDMFunction | Unified Data Management |
| Network Function: AMF | AMFFunction | Access and Mobility Management Function |
| Network Function: SMF | SMSFFunction | Short Message Service Function |
| Network Function: AUSF | AUSFFunction | Authentication Server Function |
| Network Function: NEF | NEFFunction | Network Exposure Function |
| Network Function: PCF | PCFFunction | Policy Control Function |
| Network Function: SMSF | SMSFFunction | SMS Function |
| Network Function: NSSF | NSSFFunction | Network Slice Selection Function |
| Network Function: UDR | UDRFunction | Unified Data Repository |
| Network Function: LMF | LMFFunction | Location Management Function |
| Network Function: GMLC | GMLCFunction | Gateway Mobile Location Centre |
| Network Function: 5G-EIR | NGEIRFunction | 5G-Equipment Identity Register |
| Network Entity: SEPP | SEPPFunction | Security Edge Protection Proxy |
| Network Function: UPF | UPFFunction | User Plane Function |
| Network Function and Entity: N3IWF | N3IWFFunction | Non-3GPP InterWorking Function |
| Network Function: AF | AFFunction | Application Function |
| Network Function: UDSF | UDSFFunction | Unstructured Data Storage Function |
| Network Function: BSF | BSFFunction | Binding Support Function |
| Network Function: CHF | CHFFunction | Charging Function |
| Network Function: NWDAF | NWDAFFunction | Network Data Analytics Function |
| Network Function: CBCF |  | Cell Broadcast Centre Function |
| Network Function: UCMF |  | UE radio Capability Management Function |
| Network Function: SOR-AF |  | Steering Of Roaming Application Function |
| Network Function: SP-AF |  | Secured Packet Application Function |
| Network Entity: SCP | SCPFunction | Service Communication Proxy |
| Network Function: NSSAAF | NSSAAFFunction | Network Slice-specific and SNPN Authentication and Authorization Function |
| Network Function: AAnF | AANFFunction | AKMA Anchor Function, [a] |
| Network Function: 5G DDNMF | DDNMFFunction | 5G Direct Discovery Name Management Function |
| Network Function: NSACF | NSACFFunction | Network Slice Admission Control Function |
| Network Function: MFAF | MFAFFunction | Messaging Framework Adaptor Function |
| Network Function: EASDF | EASDFFunction | Edge Application Server Discovery Function |
| Network Function: DCCF | DCCFFunction | Data Collection Coordination Function |
| Network Function: MB-SMF |  | Multicast/Broadcast Session Management Function |
| Network Function: TSCTSF | TSCTSFFunction | Time Sensitive Communication and Time Synchronization Function |
| Network Function: ADRF |  | Analytics Data Repository Function |
| Network Function: GBA BSF |  | Generic Bootstrapping Architecture Bootstrapping Server Function |
| Network Function: CEF |  | Charging Enablement Function |
| Network Function: MB-UPF |  | Multicast/Broadcast User Plane Function |
| Network Function: NSWOF |  | Non-Seamless WLAN Offload Function |
| Network Function: PKMF |  | ProSe Key Management Function |
| Network Function: MNPF |  | Mobile Number Portability Function |
| Network Function: SMS-GMSC |  | Gateway MSC For Short Message Service |
| Network Function: SMS-IWMSC |  | Short Message Service Interworking MSC |
| Network Function: MBSF |  | Multicast/Broadcast Service Function |
| Network Function: MBSTF |  | Multicast/Broadcast Service Transport Function |
| Network Function: PANF |  | Prose Anchor Function |

Table 3.2 compiles the information on IMS network functions and entities specified in TS 29.510 [2], TS 24.229 [4], and  TS 24.237 [5].

Table 3.2 Overview of IMS network functions and entities, and managed functions

| Network function [2] | Network function management model inventory |  |
| --- | --- | --- |
|  | Managed function name [x] | Description |
| Network Function: P-CSCF | CSCFFunction | Proxy Call Session Control Function |
| Network Function: I-CSCF | CSCFFunction | Interrogating Call Session Control Function |
| Network Function: S-CSCF | CSCFFunction | ServingCall Session Control Function |
| Network Function: HSS | HSSFunction | Home Subscriber Server |
| Network Function: IMS-AS |  | IP Multimedia Subsystem (IMS) Application Server |
| Network Function: DCSF |  | Data Channel Service Function |
| Network Function: MRF |  | Media Resource Function |
| Network Function: MRFP | MRFPFunction | Multimedia Resource Function Processor |
| Network Function: MF |  | Media Function |
| Network Function: LRF |  | Location Retrieval Function |
| Network Function: MGCF | MGCFFunction | Media Gateway Control Function |
| Network Function: BGCF | BGCFFunction | Breakout Gateway Control Function |
| Network Function: AS | ASFunction | Application Server |
| Network Function: MRFC | MRFCFunction | Multimedia Resource Function Controller |
| Network Function: IBCF |  | Interconnection Border Control Function |
| Network Function: MRB |  | Media Resource Broker |
| Network Function: ISC gateway function |  | IP Multimedia Subsystem Service Control gateway function |
| Network Function: ATCF |  | Access Transfer Control Function (SRVCC) |
| Network Function: SCC AS |  | Service Centralization and Continuity AS (IMS-SRVCC) |
| Network Function: EATF |  | Emergency Access Transfer Function (SRVCC) |
| Network Function: MSC server |  | Mobile-services Switching Centre server (SRVCC) |

Table 3.3 compiles the information on 5GC interworking with EPC network functions and entities specified in TS 29.510 [2].

Table 3.3 Overview of 5GC interworking with EPC network functions and entities, and managed functions

| Network function [2] | Network function management model inventory |  |
| --- | --- | --- |
|  | Managed function name | Description |
| Combined Network Function: SMF+PGW-C |  | Session Management Function and PDN GateWay Control plane function |
| Combined Network Function: UPF+PGW-U |  | User Plane Function and PDN GateWay User plane function |
| Combined Network Function: SCEF+NEF |  | Service Capability Exposure Function and Network Exposure Function |

Table 3.4 compiles the information on network functions and entities specified in TS 29.510 [2] but outside 5GC.

Table 3.4 Overview of network functions and entities included in TS 29.510 but outside 5GC, and managed functions

| Network function [2] | Network function management model inventory |  |
| --- | --- | --- |
|  | Managed function name | Description |
| Network Function: MME |  | Mobility Management Entity |
| Network Function: DRA |  | Diameter Routing Agent |

NOTE 1: The MME is a network function that is needed in an EPC (Evolved Packet Core) and it is specified in TS 23.401, see reference [y] and TS 29.510 see reference [2].

NOTE 2: The DRA is a network function that is needed for PCRF discovery. The PCRF discovery procedures are needed where more than one PCRF is present in an operator’s network realm. Within such a deployment, an additional functional element, called DRA, is needed. PCRF discovery procedures include all the procedures that involve a DRA functional element. See reference [z] clause 7.1 and TS 29.510 see reference [2].

Observation 1: The management system does not provide a complete view of a 5GC network and definitions of some network functions are missing (see Table 3.1).

Observation 2: The management system does not provide a complete view of an IMS network and definitions of the IMS network functions are missing (see Table 3.2). The managed functions listed in the table are not supported by the SBMA specifications.

Observation 3: The management system does not provide a view of 5GC interworking with EPC and the definitions of the combined network functions are missing (see Table 3.3).

Observation 4: The management system does not provide a view of network functions and entities included in TS 29.510 but outside 5GC (see Table 3.4).

Analysis of observation 1: For the management system to provide management of the full scope of a 5GC system, the definitions of the not modelled network functions and entities in Table 3.1 should be added to NRM.

Analysis of observation 2: For the management system to provide management of the full scope of an IMS, the definitions of all network functions and entities in Table 3.2 should be added to NRM.

Analysis of observation 3: For the management system to provide management of 5GC interworking with EPC, the definitions of the individual network functions and entities in Table 3.3 are specified in 28.708, 28.705 and 28.541 except the SCEF is not captured in any OAM specification and should be added to NRM for EPC.

Analysis of observation 4: For the management system to provide management of the full scope of a 5GC system, the network function and entities in Table 3.4 are not required. The MME is specified as MMEFunction in EPC, see TS 28.708 reference [b].  The DRA is a Diameter routing agent and considered outside the 3GPP management scope.

Conclusion 1: The group concludes that the OAM specifications are missing definitions of managed functions and entities to manage the full scope of a 5GC system.

Conclusion 2: The group concludes that the OAM specifications are missing definitions of managed functions and entities to manage an IMS system.

Conclusion 3: The group concludes that the OAM specifications includes the combined network function as each network function is specified in respectively 5GC NRM and EPC NRM (except the SCEF) and does not require any updates.

Conclusion 4:  The group concludes that the OAM specifications includes the network function which is specified in EPC NRM and does not require any updates.

# 4	Detailed proposal

The group is asked to endorse the following recommendations:

Recommendation 1: The group recommends to study adding management definitions for the missing network functions and entities in Table 3.1 to the OAM specifications (TS 28.541).

Recommendation 2: For the implementation of conclusion 2 in section 3 the group agrees that this would require to be included in a work item.