3GPP TSG-SA5 Meeting #154 	S5-241151

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0030 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR 28.405 missing QoE parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2024-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the incoming RAN3 LS (R3-240599): For QoE/RVQoE measurements for sessions carried via the MBS, the gNB serving the UE when the UE returns from RRC_IDLE to RRC_CONNECTED needs to be informed about Available RVQoE metrics. This step is missing in TS28.405. RAN3 kindly ask SA5 to include in TS 28.405 (and, if applicable, TS 28.541) the RAN3 agreement that the UE can be provided with the availableRANqoEMetrics, for both signalling- and management-based QoE measurements. Moreover, RAN3 kindly ask SA5 to consider whether the ran-VisibleParameters should be included in TS 28.405, clause 4.5.1 pertaining to management-based activation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the missing availableRANqoEMetrics and ran-VisibleParameters in in Activation of measurement collection job and reporting of collected information in NR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligment with RAN3 specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.1, 4.6.1.1, 4.6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

### 4.5.1	Activation of measurement collection job and reporting of collected information in NR

Management Based Activation enables collection of application layer measurements from the UEs in the specified area for specified end user service type.

The parameters for the network request session are sent from the MnS Consumer via the management system to the gNBs that host the cells that are included in the QoE Measurement Collection job request in the Create MOI QMC operation [15]. A QMC Job is activated by creating a QMCJob object instance in the MnS producer.

Figure 4.5.1-1 and the text below describe the activation of QoE measurement collection.

Figure 4.5.1-1: QMC activation example for Management Based Activation and Reporting in NR

1)	A QoE Measurement Collection Job begins when the MnS Consumer sends createMOI request for QMCJob to the MnS Producer serving the impacted gNB(s), and includes the parameters: serviceType, areaScope, qoECollectionEntityAddress, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics and qMCConfigFile. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

NOTE: The RAN visible QoE measurements are supported for the DASH streaming and VR services.

2)	Application level measurement configuration. The Application in the UE shall send AT command +CAPPLEVMCNR [5] containing the parameter <n> set to 1 to the Access Stratum. This enables the the Access Stratum to present an unsolicited result code to the Application at a later time. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

3)	UE registration procedure. The UE shall register QMC capability with the gNB by setting qoe-Streaming-MeasReport to “supported” in UE-NR-Capability [11]. Step 2 and step 3 are asynchronous in time with respect to Step 1; they may occur before or after step 1.

4)	The gNB starts a network request session, with the QoE Reference [3] given in createMOI for QMCJob [3]. For the duration of the network request session, the gNB(s) checks for connections where the UE has qoe-Streaming-MeasReport set to “supported” [11] in step 2.

5)	If a UE has the wanted capability, the gNB stores the associated qoECollectionEntityAddress and starts a UE request session by sending RRCReconfiguration to the UE including serviceType, measConfigAppLayerId (corresponding to the value of qoEReference from step 4), measConfigAppLayerContainer, ran-VisibleParameters and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE: The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6)	The access stratum in the UE sends an unsolicited response to the Application including app-meas_service_type, meas_config_app_layer_id and app-meas_config_file. These IEs map directly to the IEs in step 5. The unsolicited response is for the AT command +CAPPLEVMCNR [5] which is sent from Application in step 2.

7)	The QMC Job remains inactive until the Application for the specified serviceType initiates service.

8)	When the Application begins service, it sends the AT command +CAPPLEVMRNR [5], including meas_config_app_layer_id and qoe_measurement_status to indicate a recording session has started, to the access stratum.

9)	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId,  appLayerSessionStatus, and qoECollectionEntityIdentity to the gNB to indicate a recording session has started.

10)	While active, the Application collects measurement reports. When the QMC is completed or at the end of period for periodic report, the recorded information is formatted in a QMC report, see [6] for DASH, [7] for MTSI or [13] for VR.

11)	When a formatted measurement report is available, the Application sends the AT command +CAPPLEVMRNR [5] including meas_config_app_layer_id, qoe_measurement_status and app-meas_report to the Access Stratum. If the QMC Job has ended, qoe_measurement_status is set to indicate the Job has ended.

12)	When the AT command +CAPPLEVMRNR is received from the Application, the Access Stratum sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

13)	The gNB translates the qoECollectionEntityIdentity into qoECollectionEntityAddress if it is needed, and sends the QMCRecord to the MCE associated to the qoECollectionEntityAddress. The report contains the QoE Reference from step 4 and measReportAppLayerContainer from step 12.

*** START OF NEXT CHANGE ***

#### 4.6.1.1	Activation of QoE measurement task after completion of UE registration procedure

Figure 4.6.1.1-1 and the text below describe the activation of QoE measurement collection after completion of UE registration procedure.

Figure 4.6.1.1-1: QMC activation and reporting example in NR after UE is registered

1.	The MnS Consumer sends createMOI request for QMCJob to UDM that controls the impacted gNB(s), and includes the parameters: serviceType, sliceScope, qoECollectionEntityAddress, qoETarget, qoEReference, mDTAlignmentInformation, availableRANqoEMetrics and qMCConfigFile.

2.	The UDM inserts subscriber related data and forwards it to the AMF.

3.	The AMF forwards the configuration parameters serviceType, sliceSupportListQMC, measCollEntityIPAddress, qoEReference, nGRANTraceId, availablerANqoEMetrics and containerForAppLayerMeasConfig in message UE Context Modification Request to the impacted gNB.

4.	The gNB checks if the UE capability matches the criteria for serviceType in the QoE measurement configuration information.

5.	If the UE has the wanted UE capability, the gNB starts a UE request session and stores the associated qoECollectionEntityAddress, sends the message RRCReconfiguration to the UE including serviceType, measConfigAppLayerId, transmissionOfSessionStartStop, ran-VisibleParameters, measConfigAppLayerContainer, and availableRANqoEMetrics.

If QoE measurement configuration pertains to MBS communication service, the gNB translates the qoECollectionEntityAddress into qoECollectionEntityIdentity and includes qoECollectionEntityIdentity in the RRCReconfiguration.

NOTE:	The IE measConfigAppLayerId indicates the identity of the application layer measurement configuration, see [11].

6.	The access stratum in the UE sends an unsolicited response to the application level including app-meas_service_type, meas_config_app_layer_id, transmission_of_session_start-end, ran_visible_periodicity, number_of_buffer_level_entries, report_playout_delay_for_media_startup, and app-meas_config_file. The unsolicited response is for the AT command +CAPPLEVMCNR which is sent from UE Application Level to UE Access Stratum during Registration procedure.

7.	When the application for the specified serviceType starts, the QMC is initiated. To specify the session which is started, the application generates a recordingSessionId.

8.	The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id and qoe_measurement_status that indicates that a session is started to the access stratum.

9.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId,  appLayerSessionStatus and qoECollectionEntityIdentity to the gNB.

10.	When the QMC is completed or at the end of period for periodic report, the recorded information is collected in a QMC report, see [6], [7] or [13].

11.	The application layer sends the AT command +CAPPLEVMRNR [7] including meas_config_app_layer_id, playout_delay_for_media_startup, number_of_buffer_level_entries, qoe_measurement_status indicating that session has ended and app-meas_report including recordingSessionId to the access stratum.

12.	The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, ran-VisibleMeasurements and measReportAppLayerContainer including recordingSessionId and qoECollectionEntityIdentity to the gNB.

13.	The gNB translates the qoECollectionEntityIdentity to the qoECollectionEntityAddress, if it is needed, and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference and the RAN transparent container including the recordingSessionId and sliceScope, which contains only the S-NSSAI used. Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

*** START OF NEXT CHANGE ***

#### 4.6.2.1	NG Based Handover for Signalling Based Activation

The figure 4.6.2.1-1 and the text below describe the handling at NG Based handover between gNBs for Signalling Based Activation case.

Figure 4.6.2.1-1: Handling of QMC activation example in case of NG Based handover in NR for Signalling Based Activation

1. Source gNB sends the message HANDOVER REQUIRED to AMF. The message includes the Source to Target Transparent Container which contains qoEReference, serviceType, measConfigAppLayerID, measCollEntityIPAddress, qoEMeasurementStatus, sliceSupportListQMC, nGRANTraceId, availableRANVisibleqoEMetrics, containerForAppLayerMeasConfig, and ran-VisibleParameters.

2. AMF sends the message HANDOVER REQUEST to the Target gNB. The message includes the Source to Target Transparent Container which contains the same parameters as in step 1.

3. The target gNB checks if the measurements will be continued or not.

4. The target gNB sends the message HANDOVER REQUEST ACKNOWLEDGE to the AMF.

5. The AMF sends the message HANDOVER COMMAND to the source gNB.

6. The source gNB sends the message RRCReconfiguration to the UE.

7. The UE sends the message RRCReconfigurationComplete to the gNB.

8. When the ongoing QMC is completed or end of periodic reporting is reached, the recorded information is collected in a QMC report, see [6], [7] or [13], at UE Application Level.

9. The application layer sends the AT command +CAPPLEVMRNR including meas_config_app_layer_id, qoe_measurement_status playout_delay_for_media_startup, number_of_buffer_level_entries, report_initial_playout_delay, and the app-meas_report to the access stratum.

10. The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, appLayerSessionStatus, measReportAppLayerContainer, and qoECollectionEntityIdentity to the gNB.

11. The gNB translates the qoECollectionEntityIdentity into qoECollectionEntityAddress and sends the QMC report to the MCE associated to the qoEReference. The report contains the qoEReference and the RAN transparent container including the recordingSessionId and sliceScope (which contains only S-NSSAI that is used). Note that the qoEReference is mapped to the measConfigAppLayerId at gNB on the previous step and is included in QMC report.

*** END OF CHANGE ***