3GPP TSG-SA5 Meeting #154 	S5-241220

# Changsha, China, 15 - 19 April 2024

# Source:	Huawei, Ericsson, Intel

# Title:	Discussion Paper on RAN3 AI/ML for NG-RAN Energy Saving Energy Cost index

# Document for:	Endorsement

Agenda Item:	6.4.25

# 1	Decision/action requested

Discuss and endorse the proposal

# 2	References

[1]	S5-235036 (R3-233515): LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[2]	S5-235777: Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[3]	S5-240460 (R3-235743): Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[4]	S5-241076: Reply LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[5]	S5-25abcd (R3-240998): Reply LS to SA5 on EC index v1

# 3	Discussion

SA5 received a LS from RAN3 [1] on their new use case “AI/ML for NG-RAN Energy Saving” highlighting the need for an ‘Energy Cost index’ being exchanged between gNBs over Xn. SA5 replied in [2] with questions for clarification. In [3], RAN3 answers to SA5 questions. In [4], SA5 asked additional questions to RAN3. RAN3 replied in [5].

Based on RAN3 answers, SA5 understanding of the overall use case is as described in below section 3.1. A proposal on how to address this use case is made in section 3.2.

## 3.1	Use case and OAM requirements

- In a given area (i.e. group of gNBs), the Operator may decide to use AI/ML for NG-RAN energy saving based on energy cost index;

NOTE 1: In Rel-18, ng-eNBs are not in the scope of this use case (see item a) in [3])

NOTE 2: In Rel-18, the Energy Cost index concept is limited to non-split NG-RAN nodes (see RAN3 answer to Q4 in [3]).

- Operator defines the unified “Energy Consumption – to – Energy Cost” mapping rule (see RAN3 answer to Q4 in [5]) to be provisioned via OAM to all gNBs pertaining to the area/group, allowing later on any gNB of the area/group to map its actual energy consumption value (between the minimum and maximum energy consumption values) into a corresponding dimensionless Energy Cost index value between the minimum (0) and maximum (10,000) Energy Cost index values

- OAM Requirement No. 1: the 3GPP management system shall enable the Operator to provision the unified “Energy Consumption – to – Energy Cost” mapping rule for all the gNBs pertaining to the area/group.

NOTE 3: the “Energy Consumption – to – Energy Cost” mapping rule is not standardized. It is specified by the Operator

NOTE 4: the “Energy Consumption – to – Energy Cost” mapping rule is not transferred over the Xn interface.

NOTE 5: The “Energy Consumption – to – Energy Cost” mapping rule applies to a certain area/group, i.e. not necessarily the whole Operator’s network, in which the Energy Cost index reporting between gNBs may be triggered (see RAN3 answer to Q4 item b) in [3]).

NOTE 6: In Rel-18, any given gNB in the Operator’s network belongs to at most one area/group for Energy Cost reporting.

NOTE 7: How gNBs measure their actual energy consumption is not standardized.

- any gNB (called ‘requesting gNB’ here) within the area/group may trigger the Energy Cost index reporting over the Xn interface towards any of its neighbour gNBs (called ‘reporting gNBs’ here) within the same area/group

- upon reception of this request, the reporting gNB:

  - starts measuring its own energy consumption and averages it over a time interval specified by the Operator and provisioned via OAM

  - applies the “Energy Consumption – to – Energy Cost” mapping rule to the averaged energy consumption value so as to obtain its own corresponding Energy Cost index value

  - reports its Energy Cost index value to the requesting gNB over the Xn interface

- OAM Requirement No. 2: the 3GPP management system shall enable the Operator to configure the time interval within which the gNBs should average their actual energy consumption values.

- the AI/ML model within the requesting gNB may use the Energy Cost index value received from the reporting gNB e.g. as feedback for a previous energy saving action performed at the ‘reporting gNB’ or to infer further energy saving actions / recommendations.

## 3.2	Proposed way forward for SA5

It is proposed that the following be made in Rel-18 Cat-B CRs:

### 3.2.1	Capture new OAM requirements in TS 28.540

It is proposed that:

- background information on AI/ML energy saving based on Energy Cost index (from above section 3.1) be captured in a new sub-clause of TS 28.540 clause 4 (Concepts and background);

- new OAM requirements (from above section 3.1) be captured in a new sub-clause of TS 28.540 clause 5 (Requirements).

### 3.2.2	Capture new NRM definitions in TS 28.541

It is proposed that:

- New NRM Stage 2 definitions be introduced in clause 4 (Information model definitions for NR NRM) of TS 28.541

- Corresponding Stage 3 definitions be introduced in Forge.

Wrt. Stage 2, it is proposed that a new IOC NGRANEnergySavingGroup be defined, with the following attributes

- eciMappingRule, as a readable / writable string (see OAM Requirement No. 1). Allowed values: N/A;

- eciTtimeInterval, as a readable / writable integer. Unit: second (see OAM Requirement No. 2). Allowed values: N/A.

The new IOC will be name-contained by SubNetwork and will have references (attributes) towards GNBCUCPFunction and ExternalGNBCUCPFunction.

# 4	Detailed proposal

It is requested to:

- endorse the use case description and the OAM requirements from section 3.1 and the proposal made in section 3.2

- endorse that TEI-18 Cat-B CRs may be used to implement the solution proposed in section 3.2, the justification being that the OAM must support, in Rel-18, the AI/ML for NG-RAN Energy Saving use case specified by RAN3.