3GPP TSG-SA5 Meeting #154	S5-241280

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0109 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.105 Correct inference capability configuration management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Several use case and requirements in clause 6.5.4 have no solution agreed in Rel 18. This contribution proposes to remove the Use Cases and requirements related to it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove : clause 6.5.4.2.1 Managing NG-RAN AI/ML-based distributed Network Energy Saving clause 6.5.4.2.2 Managing NG-RAN AI/ML-based distributed Mobility Optimization clause 6.5.4.2.3 Managing NG-RAN AI/ML-based distributed Load Balancing, REQ- AI/ML_INF-01 REQ- AI/ML_INF-02 REQ- AI/ML_INF-03 REQ-AIML_ INF_ACT-2 Correct REQ-AIML_ INF_ACT-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | leads to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.4.2 and 6.5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of modification

### 6.5.4	AI/ML inference capability configuration management

#### 6.5.4.1	Description

The AI/ML inference function and the associated ML entity may need to be managed and configured to conduct inference in the 5G system to align with the consumer´s expectation, e.g., to enable the AI/ML inference function to perform inference.

The MnS producer for AI/ML inference management needs to provide a capability for configuration of the AI/ML inference function.

#### 6.5.4.2	Use cases

6.5.4.2.X Managing AI/ML Inference Function

The MnS producer for AI/ML inference function needs to have a capability allowing an authorized MnS consumer to activate and deactivate an AI/ML inference function.

#### 6.5.4.3	Requirements for AI/ML inference management

Table 6.5.4.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_ INF_ACT-1 | The MnS producer for AI/ML inference management should have a capability allowing an authorized MnS consumer to activate and deactivate an ML inference function. | 6.5.4.2.X Managing AI/ML Inference Function |

End of modification