U3GPP TSG-SA5 Meeting #154 	S5-241388

Changsha, China, 15 – 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0236 | rev | - | Current version: | 17.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of deallocateNsi and deallocateNssi api |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-03-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The REST solution set for deallocateNsi and deallocateNssi does not adhere to Open API 3.0 specification. The specification does not allow a GET or DELETE operation to have a requestBody payload, instead they only operate on a REST endpoint. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 9.1.1.3: remove networkSliceDN from mapping table 9.1.1.3-1and associated text under 1. - Clause 9.2.1.3: remove networkSliceSubnetDN from mapping table 9.2.1.3-1 and associated text under 1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Non complaince to Open API 3.0 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.3, 9.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 9.1.1.3	Operation deallocateNsi

This operation deallocate a service profile in an NSI. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

Table 9.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| serviceProfileId | URI | n/a | Resource | M |

Table 9.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case ServiceProfile) to be deleted.

-

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

-	On success, "204 No content" shall be returned.

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

Second change

#### 9.2.1.3	Operation deallocateNssi

This operation deallocate a slice profile in an NSSI. The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

Table 9.2.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| sliceProfileId | URI | n/a | Resource | M |

Table 9.2.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case SliceProfile) to be deleted.

-

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

-	On success, "204 No content" shall be returned.

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

End of changes