3GPP TSG-SA5 Meeting #154 	S5-241460

Changsha, China, 15 - 19 April 2024

# Source:		Nokia

# Title:	Rel-19 pCR TR 28.880 Add use case and potential requirements for Energy aware QoS management

# Document for:	Approval

Agenda Item:	6.19.20

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TR 28.880: " Study on energy efficiency and energy saving aspects of 5G networks and services"

# 3	Rationale

This contribution proposes to add a new use case to enable energy aware QoS management. In order to perform QoS management that takes into account energy consumption aspects, there is a need to identify the relation between the QoS KPIs/measurements and the energy consumption.

This is key issue related to the following objective that is described in FS_Energy_OAM_Ph3.

- WT-1.3 Study new use cases, requirements and solutions for energy efficiency and energy saving, applying to NG-RAN and/or 5GC and/or network slicing, (including but not limited to intent based, analytics based and AI/ML assisted energy saving).

# 4	Detailed proposal

The following changes are proposed for TR 28.880[1].

1st Change

## 5.B	Use case #<X>: Enable energy aware QoS management

### 5.B.1	Description

In order to save energy in relation to QoS fulfillment the consumer need to have transparency on the relation between the QoS KPIs/measurements and the energy consumption. Such relation gives an insight to consumer on the potential for saving energy along with the direct impact to the QoS KPIs/measurements. In particular the customer needs to have insight on:

•	what QoS KPIs/measurements impact the energy consumption and to what extent

•	 where and when the energy saving has the highest potential, i.e. in which context and for which QoS KPI/measurements value the energy consumption is minimal

### 5.B.2	Potential requirements

| Requirement label | Description |
| --- | --- |
| PREQ-EE_QoS-1 | The MnS producer should have a capability allowing the authorized MnS consumer request the information on the relation between the QoS KPIs/measurements and the energy consumption for given context. |
| PREQ- EE_QoS-2 | The MnS producer should have a capability provide the relation between the QoS KPI/measurements and the energy consumption for given context. |
| PREQ- EE_QoS-3 | The MnS conusmer should have a capability to configure the target QoS in relation to energy consumption. |

### 5.B.3	Potential solutions

#### 5.B.3.i	Potential solution #<i>: <Potential Solution i Title>

##### 5.B.3.i.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

##### 5.B.3.i.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

### 5.B.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

End of change