3GPP TSG-SA5 Meeting #154 	S5-241667

Changsha, China, 15 - 19 April 2024

# Source:	CATT

# Title:	pCR TR 28.908 Add use case on ML entity distributed training

# Document for:	Approval

Agenda Item:	6.19.1

1	Decision/action requested

Approval

2	References

[1]	3GPP TR 28.908 V18.0.0: " Study on Artificial Intelligence/Machine Learning (AI/ML)

management "

[2]		SP-231780: "New SID: Study on AI/ML management - phase 2 "

3	Rationale

It is proposed to add a new use case on ML entity distributed training.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

5.1.x	ML entity distributed training

5.1.x.1	Description

Distributed training is a model training paradigm that involves spreading training workload across multiple training functions, to accelerate the training process and/or reduce the required computational resources. Distributed training can be used for traditional machine learning models, but is better suited for large models and compute demanding tasks as deep learning. There are two types of distributed training: model-distributed training and data-distributed training.

In 5GS, the ML training function considered as a worker node may be located in the management system or in the NF (e.g. gNB or NWDAF), each node has different computing resources and storage capacity. In order to obtain load balance between nodes and maximize the efficiency of resource utilization, the training workload is necessary to split up and shared among multiple training functions according to the actual situation of nodes. Thus, aspects of distributed learning that are appropriate in multi-vendor environments need to be supported in the network management systems.

NOTE:	Federated learning is applied to data-distributed training, which focus on address critical issues such as data privacy, data security and data access rights, etc., and training on its own subset of the data.

5.1.x.2	Use cases

5.1.x.2.1	ML model distributed training

ML model parallel is widely-used in distributed training techniques. In 5GS, the MnS consumer may expect to speed up the training process or train the ML entity efficiently under certain conditions (e.g. the size of the model may be too large for a single training function), the ML entity needs to be segmented into different parts that can run concurrently in different training function (e.g. management system, gNB or NWDAF), and each one runs on the same data. The MLT MnS producer needs to synchronize the shared parameters (e.g. once for each forward or backward-propagation step). As only part of a model operates on any individual MLT MnS producer, a set of MLT MnS producer can collectively training a larger model.

5.1.x.3	Potential requirements

REQ-ML_DIST-TRNG-01	The MLT MnS producer should have a capability allowing the authorized consumer to request ML distributed training.

5.1.x.4	Possible solutions

TBD

5.1.x.5	Evaluation

TBD

End of change