3GPP TSG-SA5 Meeting #154	S5-241738

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0238 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 CR TS 28.531 Correct stage3 definition for NSProvMnS and NSSProvMnS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huuawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The RESTful HTTP-based solution set of provisioning is not aligned with the latest Procedure of Network Slice (Subnet) Instance Allocation; - The YAML schema definition for NSProvMnS and NSSProvMnS is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the RESTful HTTP-based solution set of provisioning to align with the latest Procedure of Network Slice (Subnet) Instance Allocation - Add YAML schema definition for NSProvMnS and NSSProvMnS is missing |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | InCorrect RESTful HTTP-based solution set of provisioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.1.1.1, 9.1.1.2, 9.1.1.3. 9.1.2.0, 9.1.2.1.1, 9.1.2.1.1.2, 9.1.2.1.1.3.1, 9.1.2.1.2, 9.1.2.1.2.3, 9.2.1.1, 9.2.1.2, 9.2.1.3, 9.2.2.0, 9.2.2.1.1, 9.2.2.1.1.2, 9.2.2.1.1.3.1, 9.2.2.1.2, 9.2.2.1.2.3, X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1095 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.525: "Telecommunication management; Life Cycle Management (LCM) for mobile networks that include virtualized network functions; Requirements".

[3]	ETSI GS NFV-IFA 013 (V4.5.1) (2023-09): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Os-Ma-nfvo reference point - Interface and Information Model Specification".

[4]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[5]	3GPP TS 22.261 "Service requirements for next generation new services and markets".

[6]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[7]	Void.

[8]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[9]	GSMA NG.116 Generic Network Slice Template v9.0 (2023-04-27) - NG.116-v9.0.pdf (gsma.com).

[10]	3GPP TS 23.501: "Technical Specification Group Services and System Aspects;System Architecture for the 5G System;Stage 2".

[11]	3GPP TS 38.300: "Technical Specification Group Radio Access Network;NR; NR and NG-RAN Overall Description;Stage 2".

[12]	ETSI GS NFV-IFA 014 (V4.2.1) (2021-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Network Service Templates Specification".

[13]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[14]	ETSI GS NFV-IFA 011 (V4.3.1) (2022-06): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; VNF Descriptor and Packaging Specification".

[15]	ETSI GS NFV-IFA 008 (V4.3.1) (2022-05): "Network Function Virtualisation (NFV); Release 4; Management and Orchestration; Ve-Vnfm reference point - Interface and Information Model Specification".

[X]	Management and Orchestration APIs Stage3 repository, "https://forge.3gpp.org/rep/sa5/MnS/-/tree/Tag_Rel18_SA104/".

[Y]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)".

2nd Change

9	RESTful HTTP-based solution set of provisioning

9.1	Network slice provisioning management service

9.1.1	Mapping of operations

9.1.1.1	Introduction

Table 9.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| allocateNsi | POST | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfiles | M |
| deallocateNsi | DELETE | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfiles/{ServiceProfileId} | M |

9.1.1.2	Operation allocateNsi

This operation is to allocate a network slice instance provided by the service provider, the network slice instance may be new or existing.

Table 9.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | serviceProfile | ServiceProfile-Type | M |

Table 9.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| serviceProfileId | location header | n/a | ServiceProfileId-Type (uri-Type) | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponseDefault | O |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case ServiceProfile) to be created.

-	The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

-	On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case ServiceProfile).

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

9.1.1.3	Operation deallocateNsi

This operation deallocates a NSI (delete the ServiceProfile resource). The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

Table 9.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| serviceProfileId | path | /ServiceProfiles/{ServiceProfileId} | ServiceProfileId-Type (uri-Type) | M |

Table 9.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponseDefault | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the resource (in this case ServiceProfile) to be deleted.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

-	On success, "204 No content" shall be returned.

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

9.1.2	Resources

9.1.2.0	Resource structure

Figure 9.1.2.0-1 shows the resource structure of the network slice provisioning MnS.

Figure 9.1.2.0-1 resource structure of the network slice provisioning MnS

Table 9.1.2.0-1 provides an overview of the resources and applicable HTTP methods.

Table 9.1.2.0-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| ServiceProfiles | …/ServiceProfies | POST | Create a new ServiceProfile resource |
| ServiceProfile | …/ServiceProfies/{ServiceProfileId} | DELETE | Delete a ServiceProfile resource |

9.1.2.1	Resource definitions

9.1.2.1.1	Resource “…/ServiceProfiles

9.1.2.1.1.1	Description

This resource represents collects of network slice related requirement (i.e. ServiceProfiles).

9.1.2.1.1.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfiles

The resource URI variables are defined in table 9.1.2.1.1.2-1.

Table 9.1.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [Y] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [Y] |

9.1.2.1.1.3	HTTP methods

9.1.2.1.1.3.1	POST

The POST method creates a serviceProfile, the provider may create a NSI or using existing NSI to satisfy the serviceProfile.

This method shall support the URI query parameters specified in the following table.

Table 9.1.2.1.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.1.2.1.1.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| serviceProfile-Type | The resource representation of the set of information about ServiceProfile to be posted. | M |

Table 9.1.2.1.1.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| serviceProfileId-Type | 201 Posted | Unique ServiceProfileId assigned by the MnS producer | M |
| ErrorResponseDefault | 4xx/5xx | Returned in case of an error | M |

9.1.2.1.2	Resource “…/ServiceProfiles/{ServiceProfileId}

9.1.2.1.2.1	Description

This resource represents a network slice related requirement (i.e. ServiceProfile).

9.1.2.1.2.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfiles/{ServiceProfileId}

The resource URI variables are defined in table 9.1.2.1.2.2-1.

Table 9.1.2.1.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [Y] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [Y] |

9.1.2.1.2.3	HTTP methods

9.1.2.1.2.3.1	DELETE

The DELETE method deletes a ServiceProfile.

This method shall support the URI query parameters specified in the following table.

Table 9.1.2.1.1.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a |  |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 9.1.2.1.2.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.1.2.1.2.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponseDefault | 4xx/5xx | Returned in case of an error | M |

9.2	Network slice subnet provisioning management service

9.2.1	Mapping of operations

9.2.1.1	Introduction

Table 9.2.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | S |
| --- | --- | --- | --- |
| allocateNssi | POST | {MnSRoot}/NSSProvMnS/{MnSVersion}/ SliceProfiles | M |
| deallocateNssi | DELETE | {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceProfiles/{SliceProfieId} | M |

9.2.1.2	Operation allocateNssi

This operation is to allocate a network slice subnet instance provided by the service provider, the network slice subnet instance may be new or existing.

Table 9.2.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | sliceProfile | SliceProfile-Typ | M |

Table 9.2.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| sliceProfileId | Location header | n/a | ServiceProfileId-Type (uri-Type) | M |
| status | response status codes | n/a | n/a | M |
|  | Response body | error | ErrorResponseDefault | O |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case SliceProfile) to be created.

-	The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

-	On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case SliceProfile).

- 	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

9.2.1.3	Operation deallocateNssi

This operation deallocates a NSSI (delete SliceProfile resource). The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

Table 9.2.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| sliceProfileId | path | /SliceProfies/{SliceProfileId} | SliceProfileId-Type (uri-Type) | M |

Table 9.2.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | S |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponseDefault | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

-	The target URI is equal to the concatenation of URI of the  resource (in this case SliceProfile) to be deleted.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

-	On success, "204 No content" shall be returned.

-	On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

9.2.2	Resources

9.2.2.0	Resource structure

Figure 9.2.2.0-1 shows the resource structure of the network slice subnet provisioning MnS.

Figure 9.2.2.0-1 resource structure of the network slice subnet provisioning MnS

Table 9.2.2.0-1 provides an overview of the resources and applicable HTTP methods.

Table 9.2.2.0-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| SliceProfiles | …/SliceProfies | POST | Create a new SliceProfile resource |
| SliceProfile | …/SliceProfiles/{SliceProfileId} | DELETE | Delete a SliceProfile resource |

9.2.2.1	Resource definitions

9.2.2.1.1	Resource “.../SliceProfiles

9.2.2.1.1.1	Description

This resource represents collects of network slice subnet related requirements (i.e. SliceProfile).

9.2.2.1.1.2	URI

Resource URI: {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceProfile

The resource URI variables are defined in table 9.2.2.1.1.2-1.

Table 9.2.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [Y] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [Y] |

9.2.2.1.1.3	HTTP methods

9.2.2.1.1.3.1	POST

The POST method creates a SliceProfile, the provider may create a new NSSI or using existing NSSI to support the SliceProfile.

This method shall support the URI query parameters specified in the following table.

Table 9.2.2.1.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table 9.2.2.1.1.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| sliceProfile-Type | The resource representation of the set of information about SliceProfile to be posted. | M |

Table 9.2.2.1.1.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| sliceProfileId-Type | 201 Posted | Unique SliceProfileId assigned by the MnS producer | M |
| ErrorResponseDefault | 4xx/5xx | Returned in case of an error | M |

9.2.2.1.2	Resource “…/SliceProfiles/{SliceProfileId}

9.2.2.1.2.1	Description

This resource represents a network slice subnet related requirement (i.e. SliceProfile).

9.2.2.1.2.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/SliceProfiles/{SliceProfileId}

The resource URI variables are defined in table 9.2.2.1.2.2-1.

Table 9.2.2.1.2.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [Y] |
| MnSVersion | See clause 4.4.2 of TS 32.158 [Y] |

9.2.2.1.2.3	HTTP methods

9.2.2.1.2.3.1	DELETE

The DELETE method deletes a SliceProfile.

This method shall support the URI query parameters specified in the following table.

Table 9.2.2.1.2.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a |  |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table 9.2.2.1.2.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 9.2.2.1.2.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| ErrorResponseDefault | 4xx/5xx | Returned in case of an error | M |

4th Change

X	OpenAPI specification

X.1	OpenAPI document for network slice provisioning MnS

The OpenAPI/YAML definitions for provisioning MnS are specified in 3GPP Forge [X].

Directory: OpenAPI

File: TS28531_NSProvMnS.yaml

X.2	OpenAPI document for network slice provisioning MnS

The OpenAPI/YAML definitions for provisioning MnS are specified in 3GPP Forge [X].

Directory: OpenAPI

File: TS28531_NSSProvMnS.yaml

5th Change

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1095 at commit 2171224e2f839bc0b1750bda69984f0fad267018

*** START OF CHANGE 2 ***

*** OpenAPI/TS28531_NSProvMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Network Slice Provisioning MnS

version: 18.5.0

description: >-

OAS 3.0.1 definition of the Network Slice Provisioning MnS

Â© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.531; Provisioning

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.531/

servers:

- url: '{MnSRoot}/NSProvMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/ServiceProfiles:

post:

summary: Create a ServiceProfile

description: To create a ServiceProfile resource to represent network slice related requirements.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceProfile-Type'

responses:

'201':

description: Success case ("201 Created"). The representation of the newly created ServiceProfile resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceProfileId-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

/ServiceProfiles/{ServiceProfileId}:

delete:

summary: Delete a ServiceProfile

description: To delete a ServiceProfile resource to represent network slice related requirements.

parameters:

- name: ServiceProfileId

in: path

description: Identifies an individual ServiceProfille.

required: true

schema:

$ref: '#/components/schemas/ServiceProfileId-Type'

responses:

'204':

description: >-

Success case ("204 No Content").

The ServiceProfile resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

components:

schemas:

uri-Type:

description: Resource URI

type: string

ServiceProfileId-Type:

$ref: '#/components/schemas/uri-Type'

ServiceProfile-Type:

$ref: 'TS28541_SliceNrm.yaml#/components/schemas/ServiceProfile'

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** OpenAPI/TS28531_NSSProvMnS.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: Network Slice Subnet Provisioning MnS

version: 18.5.0

description: >-

OAS 3.0.1 definition of the Network Slice Suubnet Provisioning MnS

Â© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.531; Provisioning

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.531/

servers:

- url: '{MnSRoot}/NSSProvMnS/{MnSVersion}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

/SliceProfiles:

post:

summary: Create a SliceProfile

description: To create a SliceeProfile resource to represent network slice subnet related requirements.

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ServiceProfileId-Type'

responses:

'201':

description: Success case ("201 Created"). The representation of the newly created SliceProfile resource shall be returned.

content:

application/json:

schema:

$ref: '#/components/schemas/SliceProfileId-Type'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

/SliceProfiles/{SliceProfileId}:

delete:

summary: Delete a SliceProfile

description: To delete a SliceProfile resource to represent network slice suubnet related requirements.

parameters:

- name: SliceProfileId

in: path

description: Identifies an individual SliceProfille.

required: true

schema:

$ref: '#/components/schemas/SliceProfileId-Type'

responses:

'204':

description: >-

Success case ("204 No Content").

The ServiceProfile resource has been deleted. The response message body is absent.

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponseDefault'

components:

schemas:

uri-Type:

description: Resource URI

type: string

SliceProfileId-Type:

$ref: '#/components/schemas/uri-Type'

ServiceProfileId-Type:

$ref: 'TS28541_SliceNrm.yaml#/components/schemas/SliceProfile'

<CODE ENDS>

*** END OF CHANGE 3 ***

End of Changes