3GPP TSG-SA5 Meeting #154 	S5-241748

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.279 | CR | 0006 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.279 Correction MB-SMF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18, 5MBS_CH |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The requirements regarding units of measurements and usage of charging identifier are unclear, all reporting and triggers are defined for MBS session. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding that quota management can be for time, while downlink data volume can be reported per MBS session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent requirements compared to supported information elements may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1, 5.1.2, 5.1.3, and 5.1.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.1.1	General

The charging functions specified for the 5G MBS session charging:

-	MBS session in MB-SMF, refer to TS 23.247 [9].

Second change

### 5.1.2	Requirements

The following are high-level charging requirements applicable if5G MBS session charging is supported:

-	The MB-SMF shall support converged charging.

-	The MB-SMF shall support MBS session charging.

-	The MB-SMF shall be capable of identifying duration (elapsed time) per MBS session.

-	The MB-SMF may support quota management for duration.

-	The MB-SMF may be capable of identifying transmitted volume per MBS session.

-	The quota management shall be per rating group per MBS session.

-	Every MBS session shall be assigned a unique identity number for billing purposes per PLMN.

Third change

### 5.1.3	Charging information

The MB-SMF may collect following charging information.

-	usage of the access and core network resources: the amount of multicast or broadcast data transmitted;

-	usage duration: counted as the time interval from MBS session creation to MBS session deletion;

-	service area: the charging information describes the area over which the MBS session data is distributed. MBS service area provided by AF (clause 6.2 of TS 23.247 [9]), or a list of gNBs and UPFs that have established tunnels with MB-UPFs (clause 6.9 of TS 23.247 [9]).

For the multicast communication, the MB-SMF may report above charging information to CHF for the following cases.

-	MBS session creation and deletion;

-	Establishment and release of shared delivery towards gNBs;

-	Establishment and release of individual delivery towards UPFs.

For the broadcast communication, MB-SMF may report above charging information to CHF for the following cases.

-	MBS session creation, establishment and deletion.

Fourth change

### 5.1.4	Charging Identifier

Charging identifier is created to allow correlation of charging information.

For the MB-SMF the charging identifier is assigned per MBS session, at each MBS session create. The Charging Identifier shall be unique within the MB-SMF (that means that the charging identifier is unique within the MB-SMF set if MB-SMF set is used) which assigned it and is then used in all subsequent messages for that MBS session. The Charging Identifier shall be used throughout the MBS session’s lifetime once assigned.

End of changes