| 3GPP TR 28.874 V0.1.0 (2024-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on management aspects of NTN – Phase 2 (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Concepts and background	8

4.1	Overview	8

4.2	Management relationship between 3GPP management system and regenerative satellite components	8

5	Use cases, potential requirements and solutions	9

5.1	Management of connections and associations between satellite and ground systems (gNB/eNB/CN/management system)	9

5.1.1 	Use case #1: Connections between RAN node on-board satellite and CN (regenerative mode)	9

5.1.1.1	Description	9

5.1.1.2	Potential requirements	10

5.1.1.3	Potential solutions	10

5.1.1.3.1	Potential solution #<i>: <Potential Solution i Title>	10

5.1.1.4	Evaluation of potential solutions	10

5.1.2 	Use case #2: Associations between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground (transparent mode)	10

5.1.2.1	Description	10

5.1.2.2	Potential requirements	11

5.1.2.3	Potential solutions	11

5.1.2.3.1	Potential solution #<i>: <Potential Solution i Title>	11

5.1.2.4	Evaluation of potential solutions	11

5.1.3	 Use case #3: Support of non-terrestrial network architecture with 5G system functions on board the NTN	11

5.1.3.1	Description	11

5.1.3.2	Potential requirements	12

5.1.3.3	Potential solutions	12

5.1.3.3.1	Potential solution #<i>: <Potential Solution i Title>	12

5.1.3.4	Evaluation of potential solutions	12

5.2	Management for mobility coordination	12

5.2.1	Use case #1: NTN neighbour cell management	12

5.2.1.1	Description	12

5.2.1.2	Potential requirements	13

5.2.1.3	Potential solutions	13

5.2.1.3.1	Potential solution #<i>: <Potential Solution i Title>	13

5.2.1.4	Evaluation of potential solutions	13

5.3	Management support of Store and Forward Satellite operation	13

5.3.1	Use case #1: Store and Forward	13

5.3.1.1	Description	13

5.3.1.2	Potential requirements	13

5.3.1.3	Potential solutions	14

5.3.1.3.1	Potential solution #<i>: <Potential Solution i Title>	14

5.3.1.4	Evaluation of potential solutions	14

5.4	UE-Satellite-UE communication	14

5.4.1	Use case #1: UE-Satellite-UE Communication via UPFs on-board the satellites	14

5.4.1.1	Description	14

5.4.1.2	Potential requirements	15

5.4.1.3	Potential solutions	15

5.4.1.3.1	Potential solution #<i>: <Potential Solution i Title>	15

5.4.1.4	Evaluation of potential solutions	15

6 	  Conclusions and recommendations	15

Annex <X> (informative): Change history	16

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document  studies the management aspects of NTN in R19. It mainly investigates use cases, potential requirements and potential solutions of NTN management.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.423: "Technical Specification Group Radio Access Network; NG-RAN; Xn application protocol (XnAP) "

[3]	3GPP TS 38.300: "Technical Specification Group Radio Access Network; NR; NR and NG-RAN Overall Description; Stage 2"

[4]	3GPP TR 38.821: "Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) "

[5]	3GPP TR 22.865: “Study on satellite access Phase 3”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Concepts and background

4.1	Overview

The following management architecture assumptions of NTN are applied to the study:

-	The 5GC architecture for satellite access for NR as defined in TS 23.501 [x] is used as a baseline.

-	The EPC architecture for satellite access for IoT as defined in TS 23.401 [x] and the architecture enhancements to facilitate communications with packet data networks and applications as defined in TS 23.682 [x] are used as a baseline.

-	The reference management scenario for NTN as defined in TS 28.530 [x] is used as a baseline.

-	The management architecture shall support Store and Forward (S&F) satellite operation.

-	The management architecture shall support UE-Satellite-UE communication, including at least UPF, is present onboard of the satellites.

4.2	Management relationship between 3GPP management system and regenerative satellite components

The management relationship between 3GPP management system and regenerative-based satellite with gNB/eNB and/or core network functions on-board depicts in figure 4.2-1. The 3GPP Management system manages one or multiple moving regenerative satellite component(s), including gNB/eNB on-board satellite, core network function on-board satellite, Xn interface over inter-satellite link, and feeder link switchover.

Figure 4.2-1: Management relationship between 3GPP management system and regenerative satellite components

5	Use cases, potential requirements and solutions

5.1	Management of connections and associations between satellite and ground systems (gNB/eNB/CN/management system)

5.1.1 	Use case #1: Connections between RAN node on-board satellite and CN (regenerative mode)

5.1.1.1	Description

When non-geo synchronized objects like LEO and MEO satellites are used for the NTN system, the satellites will not always be at the same position relative the earth’s surface, and the coverage area on the earth surface for one satellite varies over time.

One consequence of non-geosynchronous satellites is that the associations between the entities on ground segment and entities in space segment are changing frequently, typically with a period of one to several minutes.

Figure 5.1.1.1-1	Non-geosynchronous satellites in NTN with regenerative gNB processed satellite payload

Fig. 5.1.1.1-1 illustrates this association change in an NTN system with regenerative gNB satellite payload. In this case, the ground segment Core Network (CN) will serve the same spotbeams all the time, while the space segment gNB on different satellites (satellite 1, 2 and 3) will serve the spotbeam in different time period as the satellites are approaching and leaving the coverage of the spotbeam over time. From management point of view, it will e.g. impact the association between GNBCUCPFunction and AMFFunction.

Another issue is the topology between space segment Managed Element (MnS producer) and the ground based Management System (MnS consumer): With long distances in between, disturbances (e.g. bad weather conditions), and partial reachability issues (when satellites fly over oceans with no gateway coverage), the latency, availability and reliability of the interface between them (feeder link + Inter-satellite link) are impacted.

Summary:

For the deployment scenario of RAN nodes on-board satellites, this would result in the following scenario: a LEO or MEO satellite with an onboard RAN node leaves the coverage area of a CN and then returns to the coverage area of that CN after cycling around the Earth.

From the operator’s perspective, it’s necessary to investigate how to efficiently manage the connections between RAN nodes and CN to avoid errors in CN due to stale connections, e.g. AMF/MME sending paging requests or AMF configuration updates to an unavailable RAN node. For example, 3GPP management system configures AMF/MME and/or gNB/eNB to add necessary information to support their awareness of when connectivity between a RAN node and a CN NF is available or unavailable.

5.1.1.2	Potential requirements

REQ-NTN-REGCON-1：3GPP MnS producer should have the capability to configure the connections between RAN nodes on-board satellite and 5GC on an unreliable management interface.

5.1.1.3	Potential solutions

5.1.1.3.1	Potential solution #<i>: <Potential Solution i Title>

5.1.1.4	Evaluation of potential solutions

5.1.2 	Use case #2: Associations between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground (transparent mode)

5.1.2.1	Description

When non-geo synchronized objects like LEO and MEO satellites are used for the NTN system, the satellites will not always be at the same position relative the earth’s surface, and the coverage area on the earth surface for one satellite varies over time.

One consequence of non-geosynchronous satellites is that the associations between the entities on ground segment and entities in space segment are changing frequently, typically with a period of one to several minutes.

Figure 5.1.2.1-1	Non-geosynchronous satellites in NTN system with transparent satellite payload

Fig. 5.1.2.1-1 illustrates this association change in an NTN system with transparent satellite payload. In this case, the ground segment gNB will serve the same spotbeams all the time, while different satellites (satellite 1, 2 and 3) in the space segment will serve the same spotbeam in different time periods as the satellites are approaching and leaving the coverage to the spotbeam over time. From management point of view, it will e.g. impact the association between NRSectorCarrier in the gNB and SectorEquipmentFunction in the satellite.

Another issue is the topology between space segment Managed Element (MnS producer) and the ground based Management System (MnS consumer): With long distances in between, disturbances (e.g. bad weather conditions), and partial reachability issues (when satellites fly over oceans with no gateway coverage), the latency, availability and reliability of the interface between them (feeder link + Inter-satellite link) are impacted.

Summary:

For the deployment scenario of SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground, this would result in the following scenario: a LEO or MEO satellite with an onboard SectorEquipmentFunction leaves the coverage area of a RAN node (gNB/eNB) on ground and then returns to the coverage area of that RAN node (gNB/eNB) after cycling around the Earth.

From the operator’s perspective, it’s necessary to investigate how to efficiently manage the connections between SectorEquipmentFunction on-board satellite and the RAN nodes (gNB/eNB) on ground due to stale connections. For example, 3GPP management system configures association between NRSectorCarrier in the gNB and SectorEquipmentFunction in the satellite, adding necessary information to support their awareness of when connectivity between the satellite and the RAN nodes (gNB/eNB) on ground is available or unavailable.

5.1.2.2	Potential requirements

REQ-NTN-TRANSCON-1：3GPP MnS producer should have the capability to configure the associations between SectorEquipmentFunction on-board satellite and NRSectorCarrier in the RAN nodes (gNB/eNB) on ground on an unreliable management interface.

5.1.2.3	Potential solutions

5.1.2.3.1	Potential solution #<i>: <Potential Solution i Title>

5.1.2.4	Evaluation of potential solutions

5.1.3	 Use case #3: Support of non-terrestrial network architecture with 5G system functions on board the NTN

5.1.3.1	Description

Support of non-terrestrial network architecture with 5G system functions on board the NTN (i.e. regenerative payloads) provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. TR 38.821 [4] gives an overview about the typical scenario based on regenerative payload is depicted below:

Figure 5.1.3.1-1: Non-terrestrial network typical scenario based on regenerative payload

Satellite regenerative payload is effectively equivalent to having all or part of base station functions (e.g. gNB, CU, or DU) and/or CN functions on board the satellite. Support of regenerative payload brings some benefits on radio resource handling in Uu, and radio resource coordination between the gNBs/eNBs via the ISL. But it also introduces system complexity due to moving RAN node, different LEO/MEO deployment and feeder link switchover, etc. Thus, the 3GPP management systems should provide means for an MnS consumer to manage and control different NTN configurations in the following scenarios:

- Feeder link switchover, due to e.g. maintenance, traffic offloading, or (for LEO) due to the satellite and eNB/gNB moving out of visibility with respect to the current NTN GW.

- Configuration enhancement on the interface management between the RAN and the 5GC, e.g., N2/S1 management.

5.1.3.2	Potential requirements

REQ-NTN-FUN-0X: The 3GPP management system shall have the capability to manage feeder link switchover.

5.1.3.3	Potential solutions

5.1.3.3.1	Potential solution #<i>: <Potential Solution i Title>

5.1.3.4	Evaluation of potential solutions

5.2	Management for mobility coordination

5.2.1	Use case #1: NTN neighbour cell management

5.2.1.1	Description

As defined in TS 38.423 [2] clause 9.2.2.13, the maximum number of neighbour cells associated to a served cell is 1024. In TN scenario the limitation on the number of neighbour cells is acceptable, but in NTN scenario the number of neighbour cells may overwhelm for two reasons:

- NTN cell footprint is larger than a TN cell

The typical beam footprint size of an NTN cell ranges from 100 km to 3500 km according to TR 38.821 [4], while for a TN cell the size is generally less than 10 km.

- NTN neighbour cell changes as NTN cell moving.

Figure 5.2.1-2: NTN neighbour cell changes in LEO earth-moving scenario

There are three types of service links described in TS 38.300 [3], one of the types is Earth-moving, which means the coverage area of earth-moving beam(s) slides over the Earth surface. In this scenario, NTN neighbour cell changes as NTN cell moving. Figure 5.2.1-2 shows the NTN neighbour cell changes in LEO earth-moving scenario. The NTN cell coverage is geographical area #1 at 10:00. At that moment, the neighbour cells are NTN cells and TN cells whose coverage are overlapping with geographical area #1. When the satellite moves to geographical area #2 at 10:15, the TN cells that overlap with an NTN cell change, which means the neighbour cells of an NTN cell are not the ones in geographical area #1 at 10:00. In this case, if the cell recognized as neighbour cell in geographical area #1 still remains as neighbour cell at geographical area #2 and the information of the cell is sent to UE for handover, the performance of mobility could be worse since this cell is invalid at geographical area #2.

Overall, simply enlarging the limit of maximum number of neighbour cells leads two drawbacks in NTN:

1. As satellite moving, the number of neighbour cells passed by could be extremely large resulting in high storage demand in RAN.

2. As satellite moving, plenty of neighbour cells lose validity.

Therefore, to provide better mobility coordination and service continuity, how to configure NTN neighbour cells becomes a problem to be solved.

5.2.1.2	Potential requirements

REQ-NTN-FUN-0X: The 3GPP management system shall have the capability to configure NTN neighbour cells considering the satellite movement.

5.2.1.3	Potential solutions

5.2.1.3.1	Potential solution #<i>: <Potential Solution i Title>

5.2.1.4	Evaluation of potential solutions

5.3	Management support of Store and Forward Satellite operation

5.3.1	Use case #1: Store and Forward

5.3.1.1	Description

The use case and requirements of store and forward functionality in defined in [5]. The Store and Forward Satellite (S&F) operation in a 5G system with satellite access is intended to provide some level of communication service for UEs under satellite coverage with intermittent/temporary satellite connectivity (e.g. when the satellite is not connected via a feeder link or via ISL to the ground network) for delay-tolerant communication service. The concept of “S&F” service is widely used in the fields of delay-tolerant networking and disruption-tolerant networking.

The management of the S&F functionality need to be defined. The limitations on the size/amount of data that can be sent from the UE to the AF (Application Function, e.g IoT devices) and vice versa need to be configured. Forwarding priority for the stored data to the ground station or to the UE and data retention period for the exchanged data should be configured. The acknowledgment can be provided for the received messages. The acknowledge policy may dictate that the acknowledgment should not be provided. Whether to acknowledgement of the received data by the satellite could be issued possibly with the additional information about the store and forward including (not limited to)estimated time to deliver the messages need to be configured.

The S&F functionality requires to store the messages, in case of MO (mobile originated message), until the satellite coverage is available and the UE is connected to the network. This is when the stored messages are sent to the UE. Same goes for MT (mobile terminated messages) messages where UE messages are stored until the connection is established with the ground network (gateway) and messages can be delivered to the appropriate AF. Since the stored messages have to be read by the network entity at run time the format and the composition, needed to enforce the S&F delivery policies, of the stored message need to be defined.

5.3.1.2	Potential requirements

REQ-SNF-REQ-01: The 3GPP management system should enable an authorized MnS consumer to configure the limitations to the size/amount of data that can be sent from the UE to the ground station.

REQ-SNF-REQ-02: The 3GPP management system should enable an authorized MnS consumer to the size/amount of data that can be sent from the ground station to the UE.

NOTE: The above requirement is not for a specific UE but for all the connected UE to the satellite.

REQ-SNF-REQ-03: The 3GPP management system should enable an authorized MnS consumer to configure the forwarding priority for the stored data to the ground station.

REQ-SNF-REQ-04: The 3GPP management system should enable an authorized MnS consumer to configure the forwarding priority for the stored data to the UE.

NOTE: The above requirement is not for a specific UE but for all the connected UE to the satellite.

REQ-SNF-REQ-05: The 3GPP management system should enable an authorized MnS consumer to configure the data retention period.

REQ-SNF-REQ-06: The 3GPP management system should enable an authorized MnS consumer to configure the acknowledgement policy for both MO and MT messages.

REQ-SNF-REQ-07: The 3GPP management system should enable an authorized MnS consumer to configure the estimated time to deliver the messages to the UE.

NOTE: The above requirement is not for a specific UE but for all the connected UE to the satellite.

REQ-SNF-REQ-08: The 3GPP management system should enable an authorized MnS consumer to configure the estimated time to deliver the messages to the ground station.

REQ-SNF-REQ-09: The 3GPP management system should enable an authorized MnS consumer to configure the elements of network on the ground to support the S&F functionality of a satellite.

5.3.1.3	Potential solutions

5.3.1.3.1	Potential solution #<i>: <Potential Solution i Title>

5.3.1.4	Evaluation of potential solutions

5.4	UE-Satellite-UE communication

5.4.1	Use case #1: UE-Satellite-UE Communication via UPFs on-board the satellites

5.4.1.1	Description

SA2 R19 work has specified the requirement for architectural enhancements, including core network functions on-board satellite, to support new communication mode or service, such as UE-Satellite-UE. This introduces new management requirements, for example, when UPF on-board satellite and SMF on the ground, the N4 associations change frequently. So, the UPF and/or SMF need to know when to establish and release the N4 associations with which SMF and/or UPF. Another example, to support UE-satellite-UE communication, 3GPP management system configures indication information (i.e., indicates whether UPF is on-board satellite or not) enabling the SMF to know which UPF is on-board satellite. Thus, the UPF on-board satellite is selected to keep user plane traffic remain in the satellites.

An example diagram of the gNB and UPF deployed on satellite to enable UE-Satellite-UE is shown in Figure 5.4.1.1-1.

Figure 5.4.1.1-1: UE-Satellite-UE Communication via UPFs on-board the satellites

5.4.1.2	Potential requirements

REQ-FS_NTN_OAM_Ph2-1：3GPP management service producer should have the capability to configure the associations between UPF on-board satellite and SMF on the ground.

REQ-FS_NTN_OAM_Ph2-2：3GPP management service producer should have the capability to configure the UPF or SMF enabling the SMF to know which UPF is on-board satellite.

5.4.1.3	Potential solutions

5.4.1.3.1	Potential solution #<i>: <Potential Solution i Title>

5.4.1.4	Evaluation of potential solutions

6 			Conclusions and recommendations

Editor's Note: This clause captures the conclusions and the recommendations of the study.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-04 | SA5#154 | N/A | - | - | - | Initial skeleton | 0.0.0 |
| 2024-04 | SA5#154 | S5‑242105 |  |  |  | pCR TR 28.874 Add Structure | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242106 |  |  |  | pCR TR 28.874 Add scope | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242107 |  |  |  | pCR TR 28.874 Add concepts and background of assumptions | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242108 |  |  |  | pCR TR 28.874 Add concepts and background | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242109 |  |  |  | Rel-19 pCR 28.874 Store and forward | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242111 |  |  |  | Rel-19 pCR TR 28.874 Add use case on 5G system functions on board the NTN | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242112 |  |  |  | Rel-19 pCR TR 28.874 New NTN study issue and use cases | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242113 |  |  |  | pCR TR 28.874 Add use case to support management of Core Network Function(s) on-board satellite(s) | 0.1.0 |
| 2024-04 | SA5#154 | S5‑242114 |  |  |  | pCR TR 28.874 New Use Case on NTN neighbour cell management | 0.1.0 |