3GPP TSG-SA5 Meeting #154 	S5-242111

Changsha, China, 15 - 19 April 2024

# Source:	CATT

# Title:	pCR TR 28.874 Add use case on 5G system functions on board the NTN

# Document for:	Approval

Agenda Item:	6.19.15

1	Decision/action requested

Approval

2	References

[1]	3GPP TR 28.874: " Study on management aspects of NTN – Phase 2"

[2]		SP-231733: "New SID: Study on Management Aspects of NTN Phase 2"

3	Rationale

It is proposed to add a new use case on support of non-terrestrial network architecture with 5G system functions on board the NTN.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TR 38.821: "Technical Specification Group Radio Access Network; Solutions for NR to support non-terrestrial networks (NTN) "

Next change

5	Use cases

5.X	<Topic>

5.X.1	Support of non-terrestrial network architecture with 5G system functions on board the NTN

5.X.1.1	Description

Support of non-terrestrial network architecture with 5G system functions on board the NTN (i.e. regenerative payloads) provides new architecture option(s) besides the transparent payload as specified in Rel-17 and Rel-18, which makes the deployment of non-terrestrial network more flexible. TR 38.821 [x] gives an overview about the typical scenario based on regenerative payload is depicted below:

Figure 5.x.1.1-1: Non-terrestrial network typical scenario based on regenerative payload

Satellite regenerative payload is effectively equivalent to having all or part of base station functions (e.g. gNB, CU, or DU) and/or CN functions on board the satellite. Support of regenerative payload brings some benefits on radio resource handling in Uu, and radio resource coordination between the gNBs/eNBs via the ISL. But it also introduces system complexity due to moving RAN node, different LEO/MEO deployment and feeder link switchover, etc. Thus, the 3GPP management systems should provide means for an MnS consumer to manage and control different NTN configurations in the following scenarios:

- Feeder link switchover, due to e.g. maintenance, traffic offloading, or (for LEO) due to the satellite and eNB/gNB moving out of visibility with respect to the current NTN GW.

- Configuration enhancement on the interface management between the RAN and the 5GC, e.g., N2/S1 management.

5.X.1.2	Potential requirements

REQ-NTN-FUN-0X: The 3GPP management system shall have the capability to manage feeder link switchover.

5.X.1.3	Potential solutions

5.X.1.3.i		Potential solution #<i>: <Potential Solution i Title>

5.X.1.4	Evaluation of potential solutions

End of change