3GPP TSG-SA5 Meeting #154 	S5-241159

Changsha, China, 15 April - 19 April 2024

Source:                        ZTE Corporation

Title:                             pCR TR 28.914 add use case, requirements and solution for negotiation on a vague intent

Document for:             Approval

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	Study on intent driven management service for mobile network phase 3 v0.0.0

[2]	SP-231737 "Study on intent driven management services for mobile network phase 3"

3	Rationale

When the MnS Consumer expresses a feasible but vague intent for fulfilment, the producer may have numerous solutions for implementation. The MnS producer may not have the capability to determine which solution is better, which means it should negotiate with the MnS Consumer to make a decision. It proposes to add a key issue for introducing the MnS capability to negotiate for a vague intent.

This contribution is related to WT-3.1 of the FS_IDMS_MN_Ph3 SID [2], which is to investigate intent negotiation functionalities.

4	Detailed proposal

Start of First change

5	Use cases

5.X Use case#<X>: Negotiation on a vague intent

5.X.1 Description

In TS 28.312 R18, the intent fulfillment feasibility check is already supported. When intent intent-driven MnS producer receives the intent instance creation or modification request, it automatically conducts a feasibility check to determine whether the intent instance is feasible. If the feasibility check result is feasible, the MnS producer performs the service or network management tasks to satisfy the intent instance.

However, the MnS Producer cannot decide the content of intent it receives and the MnS Consumer may express a vague intent without a comprehensive consideration (specify which conditions should be avoided). When the MnS Consumer expresses a feasible but vague intent for fulfillment, the producer may have numerous solutions for implementation.  One of these solutions might be better in one aspect and another solution might be better in another aspect. Although this intent is feasible, the producer may not have the knowledge and capability to determine which solution is appropriate, then the intent instance cannot be performed. For example, the MnS Consumer may express intent for energy saving but only specify the target for energy consumption reduction without more constraints, e.g., reduce 20% energy consumption. There will be numerous solutions to fulfill the energy consumption target with various impacts, e.g., one solution may reduce the coverage and one may reduce the user number. The consumer may have concerns about the coverage that is not specified in the expressed intent. The MnS producer may not have the knowledge and capability to determine which solution is appropriate and negotiation with the MnS Consumer is necessary.

Therefore, it is important to introduce the MnS capability to enable the MnS producer to provide feedback (e.g., information on potential solutions, possible outcomes, and expected impacts) to allow the MnS Consumer to determine the appropriate solution.

5.X.2 Potential requirements

REQ-Intent_Negotiation _01:  The intent-driven MnS producer should have the capability to enable the MnS producer to provide feedback (e.g., information on potential solutions, possible outcome and expected impacts) to allow the MnS Consumer to determine the appropriate solution.

REQ-Intent_Negotiation_02: The intent-driven MnS should have the capability to allow MnS Consumer negotiate with the MnS Producer to decide appropriate solution for implementation.

5.X.3 Potential solutions

5.X.3.i Potential solution #<i>: <Potential Solution i Negotiation on a vague intent>

This solution proposes to enhance the existing Intent IOC and IntentReport IOC defined in TS 28.312 [X] to enable the MnS producer to provide feedback (e.g., possible outcome, information on potential solutions, and expected impact) to allow the MnS Consumer to determine the appropriate solution. Following are the proposed enhancements:

Enhancement on Intent IOC: Introduce attribute “NegotiationType” in the Intent IOC to determine the possible feedback the MnS Consumer can obtain from the MnS Producer. The allowed values for “NegotiationType” are as follows:

- RecommendedOutcomes, it represents that the MnS Consumer requests the MnS Producer to provide possible outcomes (e.g., values for attributes in the intent) based on the received intent.

- RecommendedSolutions, it represents that the MnS Consumer requests the MnS Producer to provide possible solutions based on the received intent.

Enhancement on IntentReport IOC: Introduce IntentNegotiationReoprt <<dataType>> as an attribute of IntentReport IOC to represent the feedback provided by MnS producer to allow the MnS Consumer determine the appropriate solution. IntentNegotiationReoprt <<dataType>> includes following attributes:

- PossibleOutcomeList, supported when the NegotiationType is “RecommendedOutcomes”. It includes a list of possible outcomes.

- PossibleSolutionList, supported when the NegotiationType is “RecommendedSolutions”. It includes a list of possible solutions.

- ExpectedImpacts, it includes the potential impact of corresponding outcomes or solutions above.

End of First change