3GPP TSG-SA5 Meeting #154	S5-241351

Changsha, China, 15th Apr 2024 – 19th Apr 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0536 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add use case and measurements related to NWDAF roaming analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PMKPI |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 23.288 (SA2), four service operations are specified under “NWDAF Roaming Analytics” that enables the consumer to request or to subscribe or to unsubscribe for roaming analytics generated by the RE-NWDAF. These service operations include the following: - Nnwdaf_RoamingAnalytics_Subscribe service operation - Nnwdaf_RoamingAnalytics_Unsubscribe service operation - Nnwdaf_RoamingAnalytics_Notify service operation - Nnwdaf_RoamingAnalytics_Request service operation The number of roaming analytics subscriptions, requests, notifications, and responses are some of the basic statistic information to monitor the performance of RE-NWDAF. These basic measurements can be used to derive the other performance of RE-NWDAF such as roaming analytics service successful and/or failed rate, etc. With these measurements, operators can analyze and evaluate the performance of the RE-NWDAF. Operators can use these measurements to take management actions for configuration, resource allocation, load balance and placement purposes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. A new measurement “Number of roaming analytics service subscriptions received by RE-NWDAF” is introduced. 2. A new measruement “Number of roaming analytics service requests received by RE-NWDAF” is introduced. 3. A new measurement “Number of roaming analytics service notifications generated by RE-NWDAF” is introduced. 4. A new measurement “Number of failed roaming analytics service subscriptions” is introduced. 5. A new measurement “Number of successful roaming analytics service subscriptions” is introduced. 6. A new measurement “Time consumed by the RE-NWDAF to provide roaming analytics service information” 7. A use case on “Monitoring of RE-NWDAF roaming analytics” is introduced. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator will not be able to monitor the performance of roaming analytics service operations and to take RE-NWDAF management actions for configuration, resource allocation, load balancing and placement purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18, A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 5.18	Performance measurements for NWDAF

## 5.18.X	Measurements related to RE-NWDAF roaming analytics

## 5.18.X.1	Number of roaming analytics service subscriptions received by RE-NWDAF

a)	This measurement provides the number of roaming analytics service subscriptions received by the RE-NWDAF.

b)	CC

c)	On receiving the service subscription by the RE-NWDAF for analytics information about roaming analytics, each subscription is added to the corresponding counter. The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN.

d)	It is an integer value

e)	DANS.RoamingSerSubReceived

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

## 5.18.X.2	Number of roaming analytics service requests received by RE-NWDAF

a)	This measurement provides the number of roaming analytics service requests received by the RE-NWDAF.

b)	CC

c)	On receiving the service request by the RE-NWDAF for analytics information about roaming analytics, each request is added to the corresponding counter. The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN.

d)	It is an integer value

e)	DANS.RoamingSerReqReceived

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

## 5.18.X.3	Number of roaming analytics service notifications generated by RE-NWDAF

a)	This measurement provides the number of roaming analytics service notifications generated by the RE-NWDAF.

b)	CC

c)	On sending the notification by the RE-NWDAF corresponding to the roaming analytics service subscription, each sent notification is added to the corresponding counter. The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN.

d)	It is an integer value

e)	DANS.RoamingSerNotificationGenerated

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

## 5.18.X.4	Number of failed roaming analytics service subscriptions

a)	This measurement provides the number of roaming analytics service subscriptions received and rejected by the RE-NWDAF.

b)	CC

c)	On receiving the subscription by the RE-NWDAF for the roaming analytics service, each rejected subscription is added to the corresponding counter. The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN. Furthermore, the measurement can be split into sub-counter per rejection cause (e.g., due to user consent for UE(s) revoked, due to RE-NWDAF overloading, out-of-coverage of roaming UE(s) from the serving RE-NWDAF, unavailable data related to roaming UE(s), roaming analytics not ready before the deadline, etc).

d)	It is an integer value

e)	DANS.RoamingSerSubscriptionRejected

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

## 5.18.X.5	Number of successful roaming analytics service subscriptions

a)	This measurement provides the number of roaming analytics service subscriptions received and accepted by the RE-NWDAF.

b)	CC

c)	On receiving the subscription by the RE-NWDAF for the roaming analytics service, each accepted subscription is added to the corresponding counter. The measurement can be split into sub-counters per Analytics ID, per S-NSSAI and per PLMN.

d)	It is an integer value

e)	DANS.RoamingSerSubscriptionAccepted

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

## 5.18.X.6	Time consumed by the RE-NWDAF to provide roaming analytics service information

a)	This measurement provides the time consumed by the RE-NWDAF to generate and provide the roaming analytics service information.

b)	DER(n=1)

c)	This measurement is obtained by the following method:

- the time when the RE-NWDAF sends the notification related to the roaming analytics service subscription to the RE-NWDAF roaming analytics consumer, minus the time when the RE-NWDAF receives the corresponding roaming analytics service subscription from the RE-NWDAF roaming analytics consumer. In the case where there are multiple notifications for single subscription, this measurement is obtained by calculating the time when the RE-NWDAF sends the first notification related to the roaming analytics service subscription to the RE-NWDAF roaming analytics consumer, minus the time when the RE-NWDAF receives the corresponding roaming analytics service subscription from the RE-NWDAF roaming analytics consumer. The measurement can be per Analytics ID.

d)	A real value in milliseconds

e)	DANS.RoamingAnalyticsSerTimeCons

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

Start of Next change

Annex A (informative):
Use cases for performance measurements

A.Y		Monitoring of RE-NWDAF roaming analytics

As described in TS 23.288 [59], analytics may be exchanged between PLMNs, based on operator’s policy, using NWDAFs that have roaming exchange capability (RE-NWDAF) as exchange point. The home roaming exchange NWDAF (H-RE-NWDAF) may provide analytics to the visitor roaming exchange NWDAF (V-RE-NWDAF) and vice-versa. The H-RE-NWDAF or V-RE-NWDAF provides the Nnwdaf_RoamingAnalytics service operations for that purpose. A RE-NWDAF is the only consumer of these services, i.e., both H-NWDAF and V-NWDAF need to have roaming exchange capability when used as entry point to exchange analytics in roaming scenario.

V-RE-NWDAF may request or subscribe to HPLMN analytics from the H-RE-NWDAF and then the analytics can be leveraged by the 5GC NF in the VPLMN, for example:

-	In home routed roaming scenarios, HPLMN analytics (i.e. slice load level analytics, NF load analytics, etc.) can be leveraged by the AMF in the VPLMN for network slice selection and SMF selection for PDU Session management.

-	UE-related analytics (e.g., service experience analytics, etc.) can include statistics or predictions for outbound roaming UEs.

H-RE-NWDAF may request or subscribe to VPLMN analytics from the V-RE-NWDAF and then the analytics can be leveraged by the 5GC NF in the HPLMN, for example:

-	In home routed roaming scenarios, analytics information with statistics or predictions for outbound roaming UEs can be leveraged by the H-PCF for QoS control of the PDU Session.

-	Analytics (i.e. service experience analytics, slice load level analytics, etc.) can be leveraged by the H-PCF for decision of NSSP in URSP rules provisioned to the UE roaming in the VPLMN.

The number of roaming analytics subscriptions, requests, notifications and responses are some of the basic statistic information to monitor the performance of RE-NWDAF. These basic measurements can be used to derive the other performance of RE-NWDAF such as roaming analytics service successful and/or failed rate, etc. With these measurements, operators can analyze and evaluate the performance of the RE-NWDAF. Operators can use these measurements to take management actions for configuration, resource allocation, load balance and placement purposes.

End of Changes