3GPP TSG-SA5 Meeting #154 	S5-241775

Changsha, China, 15 - 19 April 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0186 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add network KPI for supporting service availability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 28.541, availability is one of the attributes for ServiceProfile <<dataType>> which specifies the communication service availability requirement. In TS 22.261, communication service availability is defined as percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service. However, currently there is no method to measure service availability at network level. Besides, in TS 28.554 clause 4, there is an editor's note states that: “For future update of the document it will also include: - Availability.” Therefore, it is useful to measure availability by defining KPIs for the 5G network and network slicing. This contribution proposes some network KPIs to measure service availability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the definition of service availability and some availability KPIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Service availability can not be measured at network level. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.X	Network KPI for supporting service availability

6.X.1	Definition

Communication service availability is defined (see TS 22.261 [13] clause 3.1) as percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service. In the context of network layer, the following KPI(s) could be used for measuring service availability at network level.

6.X.1.1	Registration success rate satisfaction duration

a)	RSRDuration.

b)	This KPI describes the accumulated time within a measurement period for which the registration success rate is equal to or higher than the specified value requested by 5G network customer. It is obtained by accumulating the time when the value of registration success rate for a network slice (see clause 6.2.3) is equal to or higher than the specified value. It is a time interval (millisecond).

d)	NetworkSlice

6.X.1.2	PDU session establishment success rate satisfaction duration

a)	PDUSessionEstSRDuration.

b)	This KPI describes the accumulated time within a measurement period for which the PDU session establishment success rate is equal to or higher than the specified value requested by 5G network customer. It is obtained by accumulating the time when the value of PDU session establishment success rate for a network slice (see clause 6.2.5) is equal to or higher than the specified value. It is a time interval (millisecond).

d)	NetworkSlice

6.X.1.3	GTP Data Packet Loss satisfaction duration

6.X.1.3.1	Incoming GTP Data Packet Loss satisfaction duration in UPF over N3

a) 	InDataPktPacketLossN3UPFDuration.

b)	This KPI describes the accumulated time within a measurement period for which the incoming pakect loss rate in UPF over N3 is equal to or lower than the specified value requested by 5G network customer. It is obtained by accumulating the time when the value of incoming pakect loss rate in UPF over N3 (see TS 28.552 [6] clause 5.4.1.7) is equal to or lower than the specified value. It is a time interval (millisecond).

d)	SubNetwork

6.X.1.3.2	Outgoing GTP Data Packet Loss satisfaction duration in UPF over N3

a)	OutDataPktPacketLossN3UPFDuration.

b)	This KPI describes the accumulated time within a measurement period for which the outgoing pakect loss rate in UPF over N3 is equal to or lower than the specified value requested by 5G network customer. It is obtained by accumulating the time when the value of outgoing pakect loss rate in UPF over N3 (see TS 28.552 [6] clause 5.4.1.8) is equal to or lower than the specified value. It is a time interval (millisecond).

d)	SubNetwork

End of Change