3GPP TSG-SA5 Meeting #154	S5-242152

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | - | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.105 Correct Training Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson India Private Limited |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Use case Managing ML training processes and requirements for ML training needs to be corrected in order to fit the specified solutions for training in for Rel 17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This contribution proposes to correct use case “Managing ML training processes” and requirements related to it. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | leads to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.4 and 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of modification

#### 6.2.2.4	Managing ML training processes

To achieve the desired outcomes of any machine learning related use-case, the ML model, needs to be trained with the appropriate data.

The 3GPP management system should have the required training capabilities as well to have the means to manage the training request of the ML modelsby starting to suspending or cancelling the process

Next modification

### 6.2.3	Requirements for ML training

Table 6.2.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing the consumer to request ML training. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the consumer to specify the inference type of the ML model entity to be trained. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result to the consumer. | ML training requested by consumer (clause 6.2.2.1), /ML training initiated by producer (clause 6.2.2.2) |
| REQ-ML_SELECT-01 | 3GPP management system shall have the capability to enable an authorized consumer to discover the characteristics of available models including the contexts under which each of the models was trained. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have the capability to enable an authorized consumer to select an ML model. | ML models and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have the capability to enable an authorized consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2.2.1), ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have the capability to enable an authorized consumer to request for information and be informed about the available alternative models of differing complexity and performance. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have the capability to enable an authorized consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML model/entity to the consumer. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the training of specific ML models or ML entities, e.g. to modify the characteristics of the request. | ML training requested by consumer (clause 6.2.2.1),Managing ML Training Processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have the capability to enable an authorized consumer to define the reporting characteristics related to a specific training request. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have the capability to enable the MLT function to report to any authorized consumer about specific ML Training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer to request decision confidence score which is the numerical value that represents the dependability/of the associated performance metric. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer to provide to an authorized consumer an decision confidence score which is the numerical value that represents the dependability/quality of the associated performance metric. | Handling errors in data and ML decisions (clause 6.2.2.5) |

End of modification