# 3GPP TSG-SA5 Meeting #155	S5-242335

# 27 - 31 May 2024, Jeju, South Korea

# Source:	Nokia, Samsung

# Title:	pCR 28.867 CCL conflicts management

# Document for:	Approval

# Agenda Item:	6.19.4

# 1	Decision/action requested

Discuss and agree on the text

# 2	References

[1] 	3GPP TR 28.867-010 “Closed control loop management”.

# 3	Rationale

Multiple CCLs acting along each other in the same environment are expected to affect one another i.e., there may be conflicts which need to be detected and the according to resolutions executed. This pCR introduces the different kinds of conflicts and requirements on the detection of any such conflicts.

# 4	Detailed proposal

Start of First change

# 5. 			Use Cases

5.X1 Use case X1: CCL conflicts management

5.X1.1	Description

Multiple CCLs could co-exist and concurrently act within the same environment. The CCLs can affect one another, in the worst cases leading to conflicts.  The different kinds of conflicts are summarized by Table 5.X1.1-1.

5.X1.1-1: Types of potential conflicts among CCL instances

| Conflict Type | Description | CCL-A | CCL-B | Comments |
| --- | --- | --- | --- | --- |
| Goal Conflict | When same at least 1 target of a goal is present in both CCL asking for different outcomes on that target on same controlled entity (ME1). | Control Scope: ME1 Goal targets: - Throughput > 20gbps - latency < 10ms | Control Scope: ME1 Goal target: - Throughput > 10gbps | Conflict among the goals - due to different required target outcomes |
| Action Conflict | When both C1 and C2 is trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Example 1 |  | Conflict due to configuration actions at execution step because both CCL want different contradicting value for a particular characteristic of gNB-g1. Effect: even when executed at different times, the value may ping-pong continuously. |
|  |  | Goals target: - Throughput > 10gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out virtual resource | Goals target: - EC is < 10KWA Actions: - Target Entity: gNB-g1 - Target Change: scale-in virtual resource |  |
|  |  | Example 2 |  |  |
|  |  | Goal target: - HO failure is < 2% Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small positive value{to guarantee HOs with low chances of HO failure} | Goal target: - Load < 80% Actions: - Target Entity: gNB-g1 - Target Change: set CIO to a small negative value [to advance HOs and move load to other cells] |  |
| Indirect target conflict | When C1 [optimize handover] and C2 [minimize interference] have different goals but the actions of C1 effect the goals of C2 | Goal target: - HO failure is < 2% Actions: - Target Entity: gNB-g1 - Target Change: reduce CIO {to reduce chances of HO failure} | Goal target: - SINR > 10dB Actions: - Target Entity: gNB-g1 - Target Change: lower antenna tilt | By reducing antenna tilt to minimize interference C2 effect the HO goal target of C1 |
| Action Execution Time Conflict | When both C1 and C2 is trying to configure the same characteristics of same target entity (gNB-g1) in contradiction. | Goals: - Throughput > 10gbps Actions: - Target Entity: gNB-g1 - Target Change: scale-out - Target Time: 04:00 | Goals: - EC is < 10KWA Actions: - Target Entity: gNB-g1 - Target Change: scale-in - Target Time: 04:00 | Conflict due to the time of executing the configuration actions at the execution step |
| Scope conflict | C1 and C2 have different goals and actions but their scopes are overlapping – e.g. C1’s control scope is part of C2’s measurement scope | Measurement scope: cells g1 Control Scope: g1 Goal targets: - EC/bit is < 1WA Actions: - Target Entity: gNB-g2 - Target Change: switch off g2 | Measurement scope: cells g1, g2, g3, g4 Control Scope: g2 Goals: - Load < 80% Actions: - Target Entity: gNB-g2 - Target Change: change CIO | By switching off g2, C1 affects the scope which C2 reads for its load distribution measurements |

The CCL may detect or observe events that identify the possibility of any one of the above conflicts. The conflict can be avoided using some information or the policies (e.g., priority) provided by the consumer. If the conflict actually occurs, the CCL MnS producer should support services to inform MnS consumers the confirmed detected conflicts. This may also include informing MnS consumer about the candidate conflict.

4.X1.3	Potential Requirements

REQ-CCL-CONFLICT-1: The CCL MnS Producer should support a capability to detect a potential or actual conflict.

REQ-CCL-CONFLICT-2: The CCL MnS Producer should support a capability to inform an authorized MnS consumer about a potential conflict that has been detected.

REQ-CCL-CONFLICT-3: The CCL MnS Producer should support a capability to detect a potential or actual conflict.

REQ-CCL-CONFLICT-4: The CCL MnS Producer should support a capability to inform an authorized MnS consumer about a conflict that has been detected.

REQ-CCL-CONFLICT-4: The CCL MnS Producer should enable authorized MnS consumers to provide information that can be used to avoid the conflict.

REQ-CCL-CONFLICT-5: The CCL MnS Producer should enable authorized MnS consumers to provide information that can be used to resolve the conflict.

End of modifications