3GPP TSG-SA5 Meeting #155 	S5-242416

Jeju, South Korea, 27 – 31 May 2024

# Source:	Ericsson

# Title:	pCR TR 28.873 globally unique collectionId

# Document for:	Approval

Agenda Item:	6.19.11

# 1	Decision/action requested

For agreement and approval

# 2	References

[1]	TR 28.873 " Study on data management, subscriptions and reporting"

[2]	SP-231732 "Study on data management regarding subscriptions and reporting"

# 3	Rationale

As discussed and agreed in pCR S5-242096, the identity used in a measurement job shall be globally unique.

# 4	Detailed proposals

*** START OF NEXT CHANGE ***

# 6 Potential solutions

## 6.X	Potential solution #X: a globally unique collectionId

### 6.X.1 Description

As discussed and agreed in subclause 5.1 that the identity used in a measurement job shall be globally unique between consumer and productor.

This solution introduces a new globally unique identity, named as collectionId for use case 1 defined in section 5.1.

This collectionId can be added into the TraceJob and QMCJob as second identity for backward compatibility reason. For any pre-Rel19 implementation, the existing Trace Reference and QoE Reference is used. For after Rel19 implementation, collectionId will replace the Trace Reference and QoE Reference to identify a PM / Trace / MDT / QoE.

The collectionId format is MCC + MNC + consumerId + taskId. The consumerId is one byte string, and it is either tceId or qceId. The uniqueness is per PLMN. The taskId is n bytes string, and it is generated by MnS consumer. The uniqueness is per MnS consumer.

consumerId + taskId provides the uniqueness per PLMN. With MCC and MNC, it makes the collectionId globally unique.

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mcc | M | T | T | T | N/A |
| mnc | M | T | T | T | N/A |
| taskId | M | T | T | T | N/A |
| consumerId | M | T | T | T | N/A |

*** END OF CHANGE ***