3GPP TSG-SA5 Meeting #155 	S5-242590

# Jeju, South Korea, 27 - 31 May 2024

# Source:	Huawei

# Title:	Discussion Paper on RAN3 NG-RAN Energy Saving based on Energy Cost

# Document for:	Endorsement

Agenda Item:	6.4.25

# 1	Decision/action requested

Discuss and endorse the proposal

# 2	References

[1]	S5-235036 (R3-233515): LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[2]	S5-235777: Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[3]	S5-240460 (R3-235743): Reply LS on Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[4]	S5-241076: Reply LS on AI/ML for NG-RAN Energy Saving Energy Cost index

[5]	S5-242355 (R3-240998): Reply LS to SA5 on EC index v1

# 3	Discussion

SA5 received a LS from RAN3 [1] on their new use case “AI/ML for NG-RAN Energy Saving” highlighting the need for an ‘Energy Cost index’ being exchanged between gNBs over Xn. SA5 replied in [2] with questions for clarification. In [3], RAN3 answers to SA5 questions. In [4], SA5 asked additional questions to RAN3. RAN3 replied in [5].

Based on RAN3 answers, SA5 understanding of the overall use case is as described in below section 3.1. A proposal on how to address this use case is made in section 3.2.

## 3.1	Use case and OAM requirements

- In a given area (i.e. group of gNBs), the Operator may decide to use AI/ML for NG-RAN energy saving based on energy cost index;

NOTE 1: In Rel-18, ng-eNBs are not in the scope of this use case (see item a) in [3])

NOTE 2: In Rel-18, the Energy Cost index concept is limited to non-split NG-RAN nodes (see RAN3 answer to Q4 in [3]).

- Operator defines the unified “Energy Consumption – to – Energy Cost” mapping rule (see RAN3 answer to Q4 in [5]) to be provisioned via OAM to all gNBs pertaining to the area/group, allowing later on any gNB of the area/group to map its actual energy consumption value (between the minimum and maximum energy consumption values) into a corresponding dimensionless Energy Cost index value between the minimum (0) and maximum (10,000) Energy Cost index values

- OAM Requirement No. 1: the 3GPP management system shall enable the Operator to provision the unified “Energy Consumption – to – Energy Cost” mapping rule for all the gNBs pertaining to the area/group.

NOTE 3: the “Energy Consumption – to – Energy Cost” mapping rule is not standardized. It is specified by the Operator

NOTE 4: the “Energy Consumption – to – Energy Cost” mapping rule is not transferred over the Xn interface.

NOTE 5: The “Energy Consumption – to – Energy Cost” mapping rule applies to a certain area/group, i.e. not necessarily the whole Operator’s network, in which the Energy Cost reporting between gNBs may be triggered (see RAN3 answer to Q4 item b) in [3]).

NOTE 6: In Rel-18, any given gNB in the Operator’s network belongs to at most one area/group for Energy Cost reporting.

NOTE 7: How gNBs measure their actual energy consumption is not standardized.

- any gNB (called ‘requesting gNB’ here) within the area/group may trigger the Energy Cost index reporting over the Xn interface towards any of its neighbour gNBs (called ‘reporting gNBs’ here) within the same area/group

- upon reception of this request, the reporting gNB:

  - starts measuring its own energy consumption and averages it over a time interval specified by the Operator and provisioned via OAM

  - applies the “Energy Consumption – to – Energy Cost” mapping rule to the averaged energy consumption value so as to obtain its own corresponding Energy Cost index value

  - reports its Energy Cost index value to the requesting gNB over the Xn interface

- OAM Requirement No. 2: the 3GPP management system shall enable the Operator to configure the time interval within which the gNBs should average their actual energy consumption values.

- the AI/ML model within the requesting gNB may use the Energy Cost index value received from the reporting gNB e.g. as feedback for a previous energy saving action performed at the ‘reporting gNB’ or to infer further energy saving actions / recommendations.

## 3.2	Proposed way forward for SA5

It is proposed that the following be made in Rel-18 Cat-B CRs:

### 3.2.1	Capture new OAM requirements in TS 28.540

It is proposed that new OAM requirements (from above section 3.1) are captured in TS 28.540 clause 5.1 (Requirements for management of NG-RAN).

### 3.2.2	Capture new NRM definitions in TS 28.541

It is proposed that:

- New NRM Stage 2 definitions be introduced in clause 4 (Information model definitions for NR NRM) of TS 28.541

- Corresponding Stage 3 definitions be introduced in Forge.

Wrt. Stage 2, it is proposed that a new IOC NGRANEnergySavingGroup be defined, with the following attribute:

- eciMappingRule, as a readable / writable tuple (see OAM Requirement No. 1). This attribute is composed of:

  - Input to the mapping rule. In Rel-18, it’s composed of the minimum and the maximum energy consumption values of the gNBs within the group. To allow for potential introduction of additional metrics in future releases (other than minimum and the maximum energy consumption values), this input to the mapping rule is defined as multi-valued;

- eciTimeInterval, as a readable / writable integer. Unit: second (see OAM Requirement No. 2). Allowed values: N/A.

The new IOC will be name-contained by SubNetwork and will have references (attributes) towards GNBCUCPFunction and ExternalGNBCUCPFunction.

# 4	Detailed proposal

It is requested to:

- endorse the use case description and the OAM requirements from section 3.1 and the proposal made in section 3.2;

- endorse that TEI-18 Cat-B CRs may be used to implement the solution proposed in section 3.2, the justification being that the OAM must support, in Rel-18, the NG-RAN Energy Saving based on Energy Cost Index use case specified by RAN3;

- endorse that further requirements from RAN3 regarding OAM supporting for NG-RAN Energy Saving based on Energy Cost Index use case will be considered in SA5 Rel-19 SI FS_Energy_OAM_Ph3.