3GPP TSG-SA5 Meeting #155	S5-242644

# Jeju, Korea (Republic Of), 27th May 2024 - 31st May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0566 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.552 Add Small Data Transmission related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom Corporation Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph4 |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Small Data Transmission (SDT) is a procedure allowing data and/or signalling transmission while remaining in RRC_INACTIVE state (i.e. without transitioning to RRC_CONNECTED state). Number of initiated and completed SDT procedures can help for reflecting the implementation effect of SDT mechanism, which is helpful for operators to evaluate the network load and congestion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add SDT procedure related measurements： - Number of SDT procedure initiated attempts over RACH - Number of SDT procedure completed successfully over RACH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot monitor the the small data transmission procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1.1.X(new), A.Y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

CHO	Conditional Handover

CLI	Cross Link Interference

DAPS	Dual Active Protocol Stack

GP	Guard Period

HO	Handover

kbit	kilobit (1000 bits)

LHO	Legacy Handover

MA PDU	Multi-Access PDU

MN	Master Node.

MPQUIC	Multi-Path QUIC

MPTCP	Multi-Path TCP Protocol

NG-RAN	Next Generation Radio Access Network

RNA	RAN-based Notification Area

PI	Performance Indicator

SA PDU	Single-Access PDU

SDT                     Small Data Transmission

SN	Secondary Node.

SRS	Sounding Reference Signal

TEID	Tunnel Endpoint IDentifier

2nd change

5.1.1	Performance measurements valid for all gNB deployment scenarios

5.1.1.X	Small Data Transmission

##### 5.1.1.X.1	Number of SDT procedure initiated attempts over RACH

a)	This measurement provides the number of SDT procedure initiated attempts over RACH.

b)	CC.

c)	On reception of RRCResumeRequest/RRCResumeRequest1 message as well as UL SDT data and/or UL SDT signalling (see clause 18.2/18.3 of TS 38.300 [49]), or on reception of RRCResumeRequest/RRCResumeRequest1 message with an indicator mt-SDT-v1810 in ResumeCause (see clause 6.3.2 of TS 38.331 [20]) by the Receiving gNB from the UE.

d)	A single integer value.

e)	RRC.SdtAttRach.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

##### 5.1.1.X.2	Number of SDT procedure completed successfully over RACH

a)	This measurement provides the number of SDT procedure completed successfully over RACH.

b)	CC.

c)	On transmission of a RRCRelease message including resumeIndication-r18 indication in SuspendConfig (see clause 6.2.2 of TS 38.331 [20]) by the Receiving gNB to the UE to terminate the SDT procedure.

d)	A single integer value.

e)	RRC.SdtSuccRach.

f)	NRCellCU.

g)	Valid for packet switching.

h)	5GS.

3rd change

# A.Y	Monitoring of Small Data Transmission

Small data applications are expected to be a major growth area with the potential for billions of connected devices. SDT is a feature enhancement for high efficiency and low power small data transmission that allows UEs with infrequent (periodic and/or non-periodic) data transmissions to remain in the RRC_INACTIVE state without transitioning to RRC_CONNECTED state, thus avoiding frequent control signalling exchanges and reducing the power consumption of the terminal.

The measurement of the initiated SDT procedures and the expected completion can reflect the implementation effect of the SDT mechanism and the quality of small data transmission service, which helps operators to evaluate the network load and congestion.

End of change