3GPP TSG-SA5 Meeting #155	S5-242686

# Jeju, Korea (Republic Of), 27th May 2024 - 31st May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.623 | CR | 0362 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.623 Remove notifyFileDeletion as notification type (YANG, stage 3) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson España S.A. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2024-05-17 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When the file retrieval NRM fragment is used together with a data collection job (PerfMetricJob or TraceJob), the job object shall support an attribute with a link to the created Files instances. This attribute is named “_linkToFiles”. The specification notes that the “_linkToFiles” attribute allows a MnS consumer to create subscriptions for multiple notification types. One of the notification type listed is “notifyFileDeletion”. However, “notifyFileDeletion” does not exist -- see complete list “notificationTypes” in clause 4.4.1. Actually, the only file-related notifications are: “notifyFileReady” and “notifyFilePreparationError”; both are imported from TS 28.532. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to remove “notifyFileDeletion”, since this notification type is not defined/supported in the specifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reference to a non-existing notification type may cause confusion, and if implemented, interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS 28.622 CR 0388 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1131 at commit 20fa4d81764ecdc02f21304068b5353b617924a2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-common-files.yang ***

<CODE BEGINS>

module _3gpp-common-files {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-common-files";

prefix "files3gpp";

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of File retrieval NRM fragment

including the IOCs File and Files.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2024-05-11 { reference CR-0362 ; }

revision 2023-09-18 { reference CR-0271 ; }

revision 2022-09-28 { reference CR-0191; }

grouping FileGrp {

description "Represents the File IOC.";

leaf fileLocation {

type inet:uri ;

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Location of the file incl. the file transfer protocol,

and the file name for the case the file content cannot be retrieved

by reading the 'fileContent' attribute.

The allowed file transfer protocols are:

- sftp

- ftpes

- https

Examples:

'sftp://companyA.com/datastore/fileName.xml',

'https://companyA.com/ManagedElement=1/Files=1/File=1'

";

}

leaf fileCompression {

type string ;

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Name of the algorithm used for compressing the file.

An empty or absent 'fileCompression' parameter indicates the file is

not compressed. The MnS producer selects the compression algorithm.

It is encouraged to use popular algorithms such as GZIP.";

}

leaf fileSize {

type uint64 ;

mandatory true;

units bytes;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Size of the file";

}

leaf fileDataType {

type enumeration {

enum PERFORMANCE {

description "The value 'PERFORMANCE' refers to measurements and KPIs";

}

enum TRACE;

enum ANALYTICS;

enum PROPRIETARY;

}

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Type of the management data stored in the file.";

}

leaf fileFormat {

type string ;

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Identifier of the XML or ASN.1 schema (incl. its version)

used to produce the file content.";

}

leaf fileReadyTime {

type yang:date-and-time ;

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Date and time, when the file was closed (the last time)

and made available on the MnS producer.

The file content will not be changed anymore.";

}

leaf fileExpirationTime {

type yang:date-and-time ;

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Date and time after which the file may be deleted.";

}

leaf fileContent {

type string ; // String is very restrictive

mandatory true;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "File content";

}

leaf-list jobRef {

type types3gpp:DistinguishedName ;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Object instance of the 'PerfMetricJob' or 'TraceJob'

that produced the file.";

}

leaf jobId {

type string ;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Identifier of a PerfMetricJob job or a TraceJob.";

}

}

grouping FilesGrp {

description "Represents the Files IOC.";

leaf numberOfFiles {

type uint64 ;

yext3gpp:notNotifyable ;

description "Number of files in a file collection.";

}

leaf-list jobRef {

type types3gpp:DistinguishedName ;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Object instance of the 'PerfMetricJob' or 'TraceJob'

that produced the file.";

}

leaf jobId {

type string ;

yext3gpp:notNotifyable ;

yext3gpp:inVariant ;

description "Identifier of a PerfMetricJob job or a TraceJob.";

}

}

grouping FilesSubtree {

description "Contains classes of the File retrieval NRM fragment.

Should be used in classes (or classes inheriting from)

- SubNnetwork

- ManagedElement

- PerfMetricJob

- TraceJob

If a YANG module wants to augment these classes/list/groupings they must

augment all user classes!";

list Files {

description "This IOC represents a collection of files. It can be

name-contained by 'SubNetwork', 'ManagedElement', 'PerfMetricJob' or

'TraceJob'. The 'Files' object name-contains 'File' objects, that

represent the files of the collection. File collections allow to

structure related files under a common root.

Instances of 'Files' are created by MnS producers. They shall be

created at latest when the first file of the collection becomes

available for retrieval by MnS consumers.

The attributes of 'Files' represent properties of the file collection

and not properties of individual files.

When the file retrieval NRM fragment is used together with a data

collection job ('PerfMetricJob' or 'TraceJob') the following

provisions shall apply:

- The 'Files' object shall be created at the same time as the object

representing the data collection job.

- The attributes 'jobRef' and 'jobId' shall be supported and present

in a 'Files' instance. They shall identify the job that the files in

the file collection relate to.

- A 'Files' instance shall contain files related to one and only one

job.

- The files produced by one job shall be contained in one and only

one 'Files' instance.

- The job object shall support an attribute with a link to the

created 'Files' instance ('_linkToFiles').

- The attribute '_linkToFiles' shall be returned in the job creation

response, if the stage 3 protocol supports returning attributes in an

object creation response.

- The MnS producer decides where to name-contain the 'Files' instance

related to a job.

The attribute '_linkToFiles' allows a MnS consumer to create simple

and targeted subscriptions for 'notifyFileReady'and

'notifyFilePreparationError', or

'notifyMOICreation', 'notifyMOIChanges' and 'notifyFilePreparationError'

related to 'File' instances created or deleted

under the 'Files' instance of a specific job. The subscription needs

to scope simply objects one level below the 'Files' object.

In addition, the attribute '_linkToFiles' allows for simple

deployments not relying on notifications for reporting the

availability of new files, where the MnS consumer polls regularly

for new files under 'Files'.";

key id;

config false;

uses top3gpp:Top_Grp ;

container attributes {

uses FilesGrp ;

}

list File {

description "Represents a file. It is name-contained by 'Files'.

When a file becomes available on a MnS producer for retrieval by a

MnS consumer, the MnS producer shall create a 'File' instance

representing that file.

The time of creation shall be captured by the MnS producer in the

'fileReadyTime' attribute. The MnS producer shall keep the file at

least until the time specified by 'fileExpirationTime'. After that

time the MnS producer may delete the 'File' instance.

The 'fileExpirationTime' is determined by the MnS producer based on

considerations such as available storage space or file retention

policies.

The attributes 'fileSize', 'fileCompression', 'fileDataType' and

'fileFormat' describe the file properties.

The 'fileLocation' attribute indicates the address where the file can

be retrieved. The address includes the file transfer protocol (schema).

Allowed file transfer protocols are 'sftp', 'ftpes' and 'https'.

The value of 'fileLocation' can be identical to or different from the

address of the 'File' instance. The attribute 'fileContent' is

provided for retrieving the actual file content. When identifying in

the Read request a 'File' instance and specifying only the

'fileContent' attribute be returned, then only the file content shall

be returned in the response. Note, as usual, multiple attributes can

be specified to be returned, so that the file content together with

some or all file meta data attributes can be returned in response to

a single request.

In case the 'fileLocation' specifies a location different than the

'File' object location, then the attribute 'fileContent' cannot be

used for retrieving the file content. For example, the 'File' object

location may be given by

'https://companyA.com/ManagedElement=1/Files=1/File=1'

and the value of the 'fileLocation' attribute by

'sftp://companyA.com/datastore/fileName.xml'

In this case the file needs to be retrieved using 'sftp' from

'sftp://companyA.com/datastore/fileName.xml'. Attempts to read the

'fileContent' attribute shall return an error.

When the file retrieval NRM fragment is used together with a data

collection job ('PerfMetricJob' or 'TraceJob') the following

provisions shall apply:

- The attributes 'jobRef' and 'jobId' shall be supported and

present. They shall identify the job that the file is related to.

The attributes 'jobRef' and 'jobId' allow to set notification filters

in the subscription in such a way that only 'notifyMOICreation',

notifyMOIChanges and

'notifyMOIDeletion' notifications are sent to subscribed MnS consumers

if the created or deleted 'File' instance represents data related to

jobs the subscribed MnS consumer created or is interested in.

Upon creation of a 'File' instance, a notification of type

'notifyMOICreation' or 'notifyMOIChanges' shall be emitted to

subscribed MnS consumers as

usual. For the case that the file contains performance metric data

('fileDataType' is 'PERFORMANCE') the MnS producer shall emit either

a notification of type 'notifyMOICreation', 'notifyMOICreation' or

of type

'notifyFileReady'. The MnS consumer selects the notification type he

wishes to receive with the subscription created on the MnS producer.

The 'objectClass' and 'objectInstance' parameters in the notification

header of 'notifyFileReady' shall identify the new 'File' instance,

instead of the related 'PerfMetricJob', 'TraceJob', 'ManagedElement'

or 'ManagementNode'as described in 3GPP TS 28.532,

clause 11.6.1.1.1 for the case that 'notifyFileReady' is used as

part of the file data reporting MnS.

The notification 'notifyFilePreparationError' shall be supported as

well by the 'File' object. It shall be sent when an error occurs

during the preparation of the file. No 'notifyFileReady' or

'notifMOICreation shall be sent in that case. The 'objectClass'

and 'objectInstance' parameters of the notification header shall

identify the new 'File' instance representing the corrupted file,

instead of the related 'PerfMetricJob', 'TraceJob', 'ManagedElement'

or 'ManagementNode'as described in 3GPP TS 28.532, clause 11.6.1.1.1

for the case that 'notifyFilePreparationError' is used as part of

the file data reporting MnS. When the file is not created at all or

deleted, the 'objectClass' and 'objectInstance' parameters of the

notification header are populated as described in 3GPP TS 28.532,

clause 11.6.1.1.1. Note that to receive 'notifyFilePreparationError'

in that case the notification subscription needs to include these

objects in its scope.";

key id;

uses top3gpp:Top_Grp ;

container attributes {

uses FileGrp ;

}

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***