3GPP TSG-SA5 Meeting #155 	S5-242704

Jeju, South Korea, 27 - 31 May 2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.279 | CR | 0008 | rev | 2 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.279 Correcting message contents for MB-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Huawei, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_CH |  |  |  |  |  | Date: |  |  | 2024-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Missing information elements like supported feature, charging identifier, requested units including time and downlink volume. TR 28.286 concluded on Solution #6.10: Only Applicable Common IEs should be reflected in common part description compared to TS 32.290. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding missing information elements to support the requirements. Remove not applicable IEs from Table 6.1.1.2-1,Table 6.1.1.3-1, Table 6.2.2-1 and Table 6.2.2-2. Expand the sub-fields in 6.2.2 tables. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent definition of MBS session charging information may lead to interoperability issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2, 6.1.1.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-241750. Revision of S5-241840. |  |  |  |  |  |  |  |  |

First change

6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the MB-SMF as used for 5G MBS session converged charging.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [4] |
| Tenant Identifier | OC | Described in TS 32.290 [4], and can hold the identifier of the AF application identifier. |
| NF Consumer Identification | M | Described in TS 32.290 [4] |
| NF Functionality | M | Described in TS 32.290 [4] |
| NF Name | OC | Described in TS 32.290 [4] |
| NF Address | OC | Described in TS 32.290 [4] |
| NF PLMN ID | OC | Described in TS 32.290 [4] |
| Charging Identifier | OM | Described in TS 32.290 [4] |
| Invocation Timestamp | M | Described in TS 32.290 [4] |
| Invocation Sequence Number | M | Described in TS 32.290 [4] |
| Retransmission Indicator | OC | Described in TS 32.290 [4] |
| Notify URI | OC | Described in TS 32.290 [4] |
| Supported Features | OC | Described in TS 32.290 [4] |
| Service Specification Information | OC | Described in TS 32.290 [4] |
| Triggers | OC | Described in TS 32.290 [4] and holds the MBS session specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OC | Described in TS 32.290 [4] |
| Rating Group | M | Described in TS 32.290 [4] |
| Requested Unit | OC | Described in TS 32.290 [4] |
| Time | OC | Described in TS 32.290 [4] |
| Downlink Volume | OC | Described in TS 32.290 [4] |
| Used Unit Container | OC | Described in TS 32.290 [4] |
| Quota management Indicator | OC | Described in TS 32.290 [4] |
| Triggers | OC | This field is described in TS 32.290 [4] and holds triggers for usage reporting. |
| Trigger Timestamp | OC | Described in TS 32.290 [4] |
| Time | OC | Described in TS 32.290 [4] |
| Downlink Volume | OC | Described in TS 32.290 [4] |
| Local Sequence Number | OM | Described in TS 32.290 [4] |
| MBS Container Information | OC | This field holds the MBS session container specific information described in clause 6.2.1.3. |
| MB-UPF ID | OC | This field holds the MB-UPF identifier used to identify the MB-UPF. This field shall only be included when used units are reported per MB-UPF. |
| MBS Session Charging Information | OM | This field holds the MBS session specific information described in clause 6.2.1.2. |

Second change

6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF to MB-SMF as used for 5G MBS session converged charging.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [4] |
| Invocation Timestamp | M | Described in TS 32.290 [4] |
| Invocation Result | OC | Described in TS 32.290 [4] |
| Invocation Result Code | OC | Described in TS 32.290 [4] |
| Failed Parameter | OC | Described in TS 32.290 [4] |
| Failure Handling | OC | Described in TS 32.290 [4] |
| Invocation Sequence Number | M | Described in TS 32.290 [4] |
| Supported Features | OC | Described in TS 32.290 [4] |
| Triggers | OC | This field is described in TS 32.290 [4] and holds the MBS session specific triggers described in clause 5.2.1. |
| Multiple Unit Information | OC | Described in TS 32.290 [4] |
| Result Code | OC | Described in TS 32.290 [4] |
| Rating Group | OM | Described in TS 32.290 [4] |
| Granted Unit | OC | Described in TS 32.290 [4] |
| Time | OC | Described in TS 32.290 [4] |
| Downlink Volume | OC | Described in TS 32.290 [4] |
| Validity Time | OC | Described in TS 32.290 [4] |
| Final Unit Indication | OC | Described in TS 32.290 [4] |
| Time Quota Threshold | OC | Described in TS 32.290 [4] |
| Volume Quota Threshold | OC | Described in TS 32.290 [4] |
| Quota Holding Time | OC | Described in TS 32.290 [4] |

Third change

6.2.2	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by MB-SMF for 5G data connectivity converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.2-1 defines the basic structure of the supported fields in the Charging Data Request message for 5G MBS session converged charging in MB-SMF.

Table 6.2.2-1: Supported fields in Charging Data Request message

| Information Element | Functionality of MB-SMF | MBS session charging |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | -UT- |
| Tenant Identifier |  | IUT- |
| NF Consumer Identification |  | IUT- |
| NF Functionality |  | IUT- |
| NF Name |  | IUT- |
| NF Address |  | IUT- |
| NF PLMN ID |  | IUT- |
| Charging Identifier |  | IUT- |
| Invocation Timestamp |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Retransmission Indicator |  | IUT- |
| Notify URI |  | IUT- |
| Supported Features |  | IUT- |
| Service Specification Information |  | IUT- |
| Triggers |  | IUT- |
| Multiple Unit Usage |  | IUT- |
| Rating Group |  | IUT- |
| Requested Unit |  | IU-- |
| Time |  | IU-- |
| Downlink Volume |  | IU-- |
| Used Unit Container |  | -UT- |
| Quota management Indicator |  | -UT- |
| Triggers |  | -UT- |
| Trigger Timestamp |  | -UT- |
| Time |  | -UT- |
| Downlink Volume |  | -UT- |
| Local Sequence Number |  | -UT- |
| MBS Container Information |  | -UT- |
| MB-UPF ID |  | IUT- |
| MBS Session Charging Information |  | IUT- |
| MBS Service Type |  | IUT- |
| MBS Service Area |  | IUT- |
| MBS Session Start Time |  | I--- |
| MBS Session Stop Time |  | --T- |
| MBS Session Activity Status |  | -UT- |
| Serving Network Function ID |  | IUT- |

Table 6.2.2-2 defines the basic structure of the supported fields in the Charging Data Response message for 5G MBS session converged charging in MB-SMF.

Table 6.2.2-2: Supported fields in Charging Data Response message

| Information Element | Functionality of MB-SMF | MBS session charging |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | I--- |
| Invocation Timestamp |  | IUT- |
| Invocation Result |  | IUT- |
| Invocation Result Code |  | IUT- |
| Failed Parameter |  | IUT- |
| Failure Handling |  | IUT- |
| Invocation Sequence Number |  | IUT- |
| Supported Features |  | IU-- |
| Triggers |  | IU-- |
| Multiple Unit Information |  | IU-- |
| Result Code |  | IU-- |
| Rating Group |  | IU-- |
| Granted Unit |  | IU-- |
| Time |  | IU-- |
| Downlink Volume |  | IU-- |
| Validity Time |  | IU-- |
| Final Unit Indication |  | IU-- |
| Time Quota Threshold |  | IU-- |
| Volume Quota Threshold |  | IU-- |
| Quota Holding Time |  | IU-- |

End of changes