3GPP TSG-SA5 Meeting #155 	S5-242837

Jeju, South Korea, 27 - 31 May 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Registration of MnS producer into CAPIF

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] SP-231728: "New SID: Study on Enhanced OAM for management exposure to external consumers".

[2] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[3] 3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[4]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3"

# 3	Rationale

Among the study objectives in [1], WT-1 is focused on communication services and the relationship with network provisioning solutions such as network slicing, from concept and modelling viewpoints. The actual sub-objectives defined within WT-1 are detailed below:

“

- WT-1.1 Identify generic requirements for exposure of SA5 management services to external consumers, irrespective of the vertical industry.

- WT-1.2 Identify 1) potential mechanisms to discover management service producers and their respective capabilities while leveraging as much as possible the existing solutions and extending where necessary. Compare existing solutions (e.g., CAPIF, EGMF, MnS Registry and/or MSAC) that could be leveraged. 2) procedures for exposure of management services based on CAPIF, EGMF, MnS Registry, MSAC or a combination thereof. Some of the identified gaps for use of CAPIF in SA5 are  listed in S5-236381 clause 4.1.2.

- WT-1.3 Identify potential requirements that would be provided to the access control (MSAC) work item.

- WT-1.4 Study whether and how SA5 defined exposure mechanisms fit within the GSMA Open Gateway framework. This framework includes OPG, CAMARA and TM Forum

”.

In SA5#154, it was agreed to define the External MnS consumer as “An MnS consumer that has discovered an MnS via a discovery mechanism which is not defined in 3GPP SA5. CAPIF is an example of a framework providing such discovery mechanism”.  This definition is in TR 28.879 [2], clause 3.1.

CAPIF is a framework with a lot of built-in capabilities. The figure below provides a non-exhaustive summary of the different procedures relevant in CAPIF. For further details on the stage 2 and 3 of these procedures, see [3]-[4].

This contribution proposes to use CAPIF as exposure framework. For a MnS to be made available for consumption using CAPIF, the first step is to make sure that the MnS producer is allowed to use CAPIF. This requires registering MnS producer as a recognized CAPIF user.

The present pCR aims to focus on the MnS producer registration using CAPIF, which:

- leverages the procedure marked in green in the figure above.

- allows addressing WT-1.1 and WT-1.2 in [1].

# 4	Detailed proposal

It is proposed to make the following changes in the latest version of TR 28.879 [2].

Begin Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]            	3GPP TS 28.533: “Management and orchestration; Architecture Framework”.

[3]	3GPP TS 28.622: “Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)”

[4]           	3GPP TS 28.537: “Management and orchestration; Management capabilities”.

[5]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2"

[6]	SP-231669: "LS on collaboration and alignment of 3GPP defined application enablers with GSMA Open Gateway".

[7]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[8]	3GPP TS 23.255: "Application layer support for Vehicle-to-Everything (V2X) services; Functional architecture and information flows".

[9]	3GPP TS 23.286: "Application layer support for Uncrewed Aerial Systems (UAS) services; Functional architecture and information flows".

[10]	3GPP TS 23.545: "Application layer support for Factories of the Future (FF) ".

[11]	3GPP TS 23.542: "Application layer support for Personal IoT Networks".

[12] 	3GPP TS 23.554: "Application architecture for MSGin5G Service; Stage 2".

[13]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; stage 3".

[14]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP Northbound APIs".

[15]	 "The Ecosystem for Open Gateway NaaS API Development", white paper, June 2023 [link]

[16]	"GSMA Operator Platform Group – Requirements and Architecture", version 5.0, July 2023 [link]

[17]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[18]	3GPP TS 28.531: "Management and orchestration; Provisioning"

[19]	3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service"

[x]	3GPP TR 28.824: “Study on network slice management capability exposure”

Second Change

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

External MnS consumer: An MnS consumer that has discovered an MnS via a discovery mechanism which is not defined in 3GPP SA5. CAPIF [5] is an example of a framework providing such discovery mechanism.

MnS consumer: defined in TS 28.533[2].

MnS producer: defined in TS 28.533[2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC		5G Core

CAPIF 	Common API Framework

CCF 	CAPIF Core Function

CSP	Communication Service Provider

EAS	Edge Application Server

ECS	Edge Configuration Server

EDN	Edge Data Network

EEC	Edge Enabler Client

EES	Edge Enabler Server

FF	Factories of the Future

GSMA	GSM Association

MnS	Management Service (see TS28.533[2])

NaaS	Network as a Service

NEF	Network Exposure Function

NOP                    Network Operator

NSACF	Network Slice Access Control Function

NSCE 	Network Slice Capability Enablement

NWDAF	Network Data Analytics Function

OAM	Operation, Administration and Maintenance

OPAG	Operator Platform API Group

OPG	Operator Platform Group

SEAL		Service Enabler Abstraction Layer

UAS	Uncrewed Aerial Systems

V2X		Vehicle-to-Everything

VAE		Vertical App Enabler

WAS	Whole Agreement Services

WG	Working Group

AEF	API Exposing Function

APF	API Publishing Function

AMF				API Management Function

Third Change

## 5.1 Exposure of management services

### 5.1.A Use case A: MnS producer registration

#### 5.1.A.1	Description

For a MnS to be made available for consumption through an exposure framework, it is needed that this MnS producer registers into that exposure framework.

In the CAPIF, the scope of registration is limited to API provider domain functions. The registration is a procedure whereby these functions become recognized users of the CCF. These functions include:

- API Exposing Function (AEF), which supports two interfaces: CAPIF-3 interface (clause 6.4.6 in [5]) to communicate with the CCF; and CAPIF-2/2e interface (clauses 6.4.4 and 6.4.5 in [5]), to make service APIs available for API invokers.

- API Publishing Function (APF), which supports CAPIF-4 interface (clause 6.4.7 in [5]) to communicate with the CCF.

- API Management Function (AMF), which supports CAPIF-5 interface (clause 6.4.8 in [5]) to communicate with the CCF.

The registration procedure is described in clause 8.28 from [5], with stage 3 solution set detailed in clause 8.9 from [13]. The workflow of the procedure can be summarized in the figure below.

- The AMF sends a registration request to the CCF. This request contains:

  - AMF security information, necessary for CCF to validate the registration request.

  - Generic information related to the API provider domain, e.g. details of the API provider applications.

  - A list of details for each API provider function intended for registration, including registration information (e.g., public key, generic client certificate) and the role that this function represents (e.g., AEF, APF or AMF).

- The CCF validates the security information and determines if the request sent by the AMF is authorized or not. If AMF is authorized, the CCF proceeds as follows:

  - Creates the API provider domain profile, consisting of API provider domain ID and profiles for all the API provider domain functions listed in the registration request.

  - Assign the identifiers for the API provider domain functions.

3.	The CCF sends the generated information in the registration response message to the AMF.

4.	The AMF function configures the received information to the individual API provider domain functions.

Figure 5.1.A-1: Sequence flow for API provider domain registration. Extracted from [5], clause 8.28.3

According to the above workflow, it can be noted that to register (and/or update the registration information of) the API provider domain functions into CCF, the AMF is always the requestor. This means that:

- An API provider domain shall always have an AMF.

- To register (and/or update the registration information) of an APF in a specific API provider domain, the AMF of this provider domain shall be authorized user of CCF.

- To register (and/or update the registration information) of an AEF in a specific API provider domain, the AMF of this provider domain shall be authorized user of CCF.

- The AMF security information for CCF to validate the registration request is provisioned by the CAPIF administrator.

For a MnS to be made available for consumption through CAPIF, it is needed to register a MnS producer as a recognized API provider domain in the CAPIF. The CAPIF administrator needs to ensure that the API provider domain associated to the MnS producer is provisioned with AMF functionality.

#### 5.1.A.2	Potential requirements

PREQ-FS_MExpo-01 The 3GPP management system shall provide the capability to register a MnS producer as a recognized API provider domain in the CAPIF.

PREQ-FS_MExpo-02 The 3GPP management system shall provide the capability to deregister a MnS producer as a recognized API provider domain in the CAPIF.

PREQ-FS_MExpo-03 The 3GPP management system shall provide the capability to update the registration details of MnS producer in the CAPIF.

#### 5.1.A.3	Potential solutions

##### 5.1.A.3.i	Potential solution i: MnS producers in API provider domain

###### 5.1.A.3.i.1	Introduction

In this solution, the MnS producers play the role of API provider domain functions. This solution corresponds to the option 2 recommended in TR 28.824 [x].

###### 5.1.A.3.i.2	Description

Figure 5.1.A-2: MnS producers in API provider domain

MnS producers are registered as API provider domain functions in the CAPIF. Once registered, the MnS producer is able to communicate with the CCF for the subsequent procedures needed to configure accessibility to MnS from external MnS consumers. These procedures include publication, discovery and access control policy, among others.

During the registration, the MnS producer is configured with the following functionalities:

- AEF (mandatory). This means that MnS producer will need to support:

  - CAPIF-2/2e interface, so that the API invokers acting as external MnS consumers can access service APIs, when required. The API operations that will be implemented in this interface (see [13], clause 11.2) can be configured later, depending on the intended usage of MnS producer.

NOTE: The service APIs can convey different MnS information for different API invokers. This issue will be discussed as separate use case.

  - CAPIF-3 interface, so that the MnS producer can communicate with CCF to exercise access and policy related control for service API invocations initiated by the API invoker. The API operations that will be implemented in this interface (see [13], clauses 10.4, 10.6, 10.8, 10.10, 10.11) can be configured later, depending on the intended usage of MnS producer.

- APF (mandatory). This means that the MnS producer will need to support CAPIF-4 interface, so that it can communicate with CCF to publish (and manage the published) MnS information. The API operations that will be implemented in this interface (see [13], clause 10.4) can be configured later, depending on the intended usage of MnS producer.

- AMF (optional), depending on whether there exists an AMF in the API provider domain prior to MnS producer registration. If there is an existing AMF, the MnS producer does not need to play the role of AMF; otherwise, it shall support AMF functionality and CAPIF-5 interface. This interface will implement the API operations for CAPIF_API_provider_management API (see [13], clause 10.12), and can optionally implement API operations for CAPIF_events API (see [13], clause 10.4).

The registration procedure is executed with the CAPIF_API_Provider_Management API (see clause 8.9 in [13]), initiated by the AMF over the CAPIF-5 interface. The AMF sends a HTTP POST message to the CCF with a request body containing the following dataType: “APIProviderEnrolmentDetails” (see clause 8.9.4.2.2 in [13]). The attributes of this dataType are captured in the table below.

| APIProviderEnrolmentDetails | APIProviderFuncDetails attributes | RegistrationInformation attributes | Data Type | S | Cardinality |
| --- | --- | --- | --- | --- | --- |
| apiProvDomId |  |  | string | O | 1 |
| reqSec |  |  | string | M | 1 |
| apiProvFuncs |  |  | APIProviderFuncDetails | O | 1..N |
|  | apiProvFunc |  | string | C | 0..1 |
|  | regInfo |  | RegistrationInformation | M | 1 |
|  |  | apiProvPubKey | string | M | 1 |
|  |  | apiProvCert | string | O | 0…1 |
|  | apiProvFuncRole |  | Enum (aef, apf, amf) | M | 1 |
|  | apiProvFuncInfo |  | string | O | 0…1 |
| apiProvDomInfo |  |  | string | O | 0…1 |
| suppFeat |  |  | SupportedFeatures | C | 0…1 |
| failReason |  |  | string | C | 0…1 |
| apiProvNames |  |  | string | O | 0…1 |

In the proposed solution, it is needed to populate the mandatory attributes of “APIProviderEnrolmentDetails” data type with the information of the MnS producer. For example:

- The credentials of the MnS producer can be mapped to “/regSec”.

- The different roles played for the MnS producer can be mapped to “/apiProvFuncs”. Note that the MnS producer will play a minimum of two roles (AEF, APF), with the option to also play AMF, if/when needed.

- The public keys of MnS producers can be mapped to “apiProvFuncs/regInfo/apiProvPubKey”.

#### 5.1.A.4	Evaluation of potential solutions

Editor's Note:	This clause provides the evaluation of potential solutions.

End Change