3GPP TSG-SA5 Meeting #155 	S5-242901

# Jeju, Korea (Republic Of), 27th May 2024 - 31st May 2024

# Source:	Ericsson

# Title:	Rel-19 pCR TR 28.871 Mgmt Model and NRM Decoupling

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871: Study on Service Based Management Architecture enhancement phase 3

[2]	3GPP TS 28.532: Generic management services

[3]	3GPP TS 29.501: 5G System; Principles and Guidelines for Services Definition

[4]			Semantic Versioning Specification: https://semver.org.

# 3	Rationale

In the SBMA architecture each management service (MnS) has a specific version.  This version is used in the MnS Registry when exposing management services towards consumers.  Per TS 28.622[2], MnsInfo IOC is used to describe the management service information and detailed capabilities about management service, which includes its version:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsLabel | M | T | F | F | T |
| mnsType | M | T | F | F | T |
| mnsVersion | M | T | F | F | T |
| mnsAddress | M | T | F | F | T |
| mnsScope | M | T | F | F | T |

For OpenAPI solution set there are currently multiple version numbers in the OpenAPI files defining the MnSes.  For example, the Rel-18 ProvMnS.yaml contains:

openapi: 3.0.1

info:

title: Provisioning MnS

version: 18.2.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.532; Generic management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.532/

servers:

- url: '{MnSRoot}/ProvMnS/{MnSVersion}/{URI-LDN-first-part}'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

URI-LDN-first-part:

description: See clause 4.4.2 of TS 32.158

default: ''

There are also inconsistencies in the default values – some of which are ‘XXX’ and others which are blank (‘ ‘) in the yaml files.

For YANG there are revision statements for the modules.  No specific MnS version values are provided.  For example, the Rel-18 _3gpp-common-trace.yang contains:

description "Trace handling

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.623

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.622

Generic Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)" ;

revision 2024-01-29 { reference "CR-0316"; }

revision 2023-11-06 { reference "CR-0290 CR-0294"; }

Regardless of which solution set applies a vendor will provide their own implementation of each MnS comprising their solution.

It is currently unclear how the version/revision numbers defined in the stage3 are intended to map to the MnS versions provided in a vendor’s solution.  I.e. the versions which will be exposed in the MnS version in the MnS Registry.

The meaning and usage of each version/revision needs to be clarified.

# 4	Detailed proposal

First change

# 5.x MnS Version Number Handling

### 5.x.1 Description

Each MnS Producer needs to advertise itself in the MnS Registry with a version number sufficient for the MnS Consumer to understand its basic operations (i.e. component A as specified in a given 3GPP release) to then allow its Consumers to connect and perform detailed discovery of specific capabilities of that MnS (i.e. component B and component C).

For consistency, whether a vendor deploys the MnS definition as-is (i.e. no modifications) or extends them to add vendor defined capabilities a consistent versioning scheme should be used to allow MnS Consumers clearly identify a particular MnS Producer version.

A versioning scheme needs to be defined to allow an MnS Producer to differentiate its baseline capabilities across different versions.  This versioning will account for the basic operations the MnS Producer can perform (i.e. specific component A as defined in a particular 3GPP release), but does not imply other capabilities (i.e. component B and component C) which are provided during discovery.  In other words, the MnS Consumer needs to know about the MnS version to know how to perform basic operations, but still relies on capabilities discovery to get detailed management information.

### 5.x.2 Potential requirements

REQ-MNS-versioning-1: The MnS definitions shall follow a versioning scheme allowing MnS Producers to identify which 3GPP release definition applies.

REQ-MNS-versioning-2: The MnS definitions shall follow a versioning scheme allowing MnS Producers to complement the 3GPP release version with additional information to identify vendor-defined extensions.

REQ-MNS-versioning-3: The MnS shall advertise in the MnS Registry using vendor defined versioning, regardless of which solution set the implementation uses for implementation.

REQ-MNS-versioning-4: The MnS definition may comprise multiple yaml files or yang modules, each with their own versions, however the MnS Producer version in the registry shall provide a single version number.

### 5.x.3 Potential solutions

#### 5.x.3.1 Potential solution#1 MnS Version scheme based on semver.

This proposal is aligned with how other 3GPP groups have defined versions for OpenAPI as defined in [3].

MnS version numbers shall consist of at least 3 fields, following a MAJOR.MINOR.PATCH pattern according to the Semantic Versioning Specification [4].  The 1st Field (MAJOR), the 2nd Field (MINOR), and the 3rd Field (PATCH) shall contain unsigned integer numbers, and they shall not contain leading zeroes.

The “version” in the YAML file will be based on those defined for the TS associated with the stage3 definitions.  Examples:

- "18.2.0” indicates 3GPP TS baseline of v18.2.0 was used to define the yaml or yang

The “MnSVersion” default value in the yaml is not used.  It is replaced with a vendor assigned value for the MnS (i.e. MNSInfo.mnsVersion) used to advertise the MnS version in the registry.

- “1.0.0” indicates a vendor defined version, E.g. v1.0.0 of Prov MnS.

### 5.x.4 Evaluation of potential solutions

TBD

End of change