3GPP TSG-SA5 Meeting # 155	S5-243055

Jeju, South Korea, 27 - 31 May  2024

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0034 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 32.256 Correction roaming architecture decision |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHRACHF |  |  |  |  |  | Date: |  |  | 2024-05-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are two charging architectures specified for the roaming case and one or both architectures in figure 4.2.2.2 and figure 4.2.2.3 of TS 32.256. AMF and V-CHF determines which architecture should be selected for a roaming UE based on operator agreement. To apply N107 or N41, the decision logic can be made by the AMF. It can avoid a scenario where AMF makes decision on N41 and V-CHF makes decision on N107. The AMF can base the decision on wherever the CHF supports INTER_CHF from the supported feature in the NRF. When AMF decides to apply N107, AMF may want V-CHF to setup N107 inter-CHF charging session to H-CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new attribute to enable AMF to request V-CHF or MNO CHF to setup N107 charging session to H-CHF or MVNO CHF, and an attribute to allow the CHF to report if N107 has been setup. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May lead to interoperability issues if both N41 and N107 were to be used at the same time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-242790. |  |  |  |  |  |  |  |  |

First change

#### 6.2.1.5	Definition of inter-CHF information

Specific charging information used for information when the V-CHF have a connection to the H-CHF.

The detailed structure of the inter-CHF Information can be found in table 6.2.1.5.1.

Table 6.2.1.5.1: Structure of Inter-CHF Information

| Information Element | Category | Description |
| --- | --- | --- |
| Remote CHF resource | OC | This field holds the reference to the Charging Data resource in the CHF not directly connected to the NF (i.e., H-CHF) e.g., the resource URI (NOTE 1) |
| Original NF Consumer Id | OC | This field holds information on the NF triggering the request i.e., AMF (NOTE 2) |
| NOTE 1: The Remote CHF resource is included in the response to the original NF if inter-CHF communication has been used. NOTE 2: The Original NF Consumer Id is included in the request from the original NF if inter-CHF is expected to be used. |  |  |

End of changes