# 3GPP TSG-SA5 Meeting #156	S5-243552

# 19 - 23 August 2024, Maastricht, Netherlands

# Source:	Nokia

# Title:	pCR TR 28.915 Solution on ML training data generation

# Document for:	Approval

Agenda Item:	6.19.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		3GPP TR 28.915: " Study on management aspects of Network Digital Twin"

[2]		3GPP TS 28.104: " Management and orchestration; Management Data Analytics (MDA)"

# 3	Rationale

The large quantity of ML training data set improves the probability to get a ML model with better performance (e.g., higher accuracy, etc.). General the ML training data is obtained through historical network management data. For instance, assuming that there is a ML model supporting MDA SLS analysis described in TS 28.104 [2] clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of [2]. The label of training data could be information elements specified analytics output, such as issue type, affected objects, etc.

The training data mentioned above is collected from historical network management data, which means only when an issue happened in actual mobile network, the training data set can be accumulated with a new record. The preparation of ML model training data in this way has two drawbacks:

- The quantity of issues happened in actual mobile network is limited.

- The variety of issues happened in actual mobile network is limited. There could be corner network issues cases that hardly happen in real life.

Therefore, NDT can simulate network issues (as training data label) and collect performance measurements (as training data raw feature) to enrich the ML training data set.

# 4	Detailed proposal

This document proposes the following changes in TR 28.915.

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[a]	3GPP TS 28.104: " Management and orchestration; Management Data Analytics (MDA)".

Next Change

## 5.6	Use case 6: Using NDT to generate ML training data

### 5.6.1 Description

For many use cases, ML training requires large amounts of data to guarantee good performance of the ML models. In general, the ML training data for network related use cases is obtained through historical network management data. For instance, assuming that there is a ML model supporting MDA SLS analysis described in TS 28.104 [2] clause 7.2.2, the raw feature of training data could be the enabling data, such as UL/DL throughput, uplink/downlink delay, etc., as specified in clause 8.4.2 of [2].

However, obtaining data from the network has two limitations:

- The quantity of issues happened in actual mobile network is limited.

- The variety of issues happened in actual mobile network is limited. There could be corner network issues cases that hardly happen in real life.

Sufficient ML training data plays a key role to a useful ML model. The more training data provided, the better performance of ML model. To overcome, these challenges, an NDT can be used as simulated data generation entity that simulates the network and its characteristics including problems to generate configuration and performance measurements data which can be used to enrich the ML training data set.

### 5.6.2	Potential requirements

REQ-NDT-FUN-01 The NDT MnS producer should have the capability to allow an authorized MnS consumer to request generation of simulated network data to be used for ML training.

### 5.6.3	Potential Solutions

consider that the NDT

- enables the MnS consumer to configure on to the NDT instance the network scenario to be modelled, e.g., the instance for which to obtain ML data.

- enables the MnS consumer to configure the parameters of the NDT instance,

- can provide output to the MnS consumer, including values on PMs and KPIs of all the objects that have been modelled by the NDT instance.

The NDT should enable the MnS consumer to

- indicate that the simulation should be for collecting  data, i.e. that the nDT should collect the data for the simulation. This should indicate collecting data on observed scenarios – e.g. user movement, weather changes, etc. as well as the outcomes including changed parameters, observed KPI and PMs.

  - Introduce a data type and an attribute on the NDT to represent the data to be collected by the NDT instance

- define the length of the simulation as a way of defining how much should be collected by the simulator

  - Introduce an attribute on the NDT to represent the length of the simulation to be executed a given NDT instance

### 5.7.5	Evaluation of solutions

TBD

End of change