3GPP TSG-SA5 Meeting #156	S5-243553

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.111 | CR | 0011 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.111 FM Corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2024-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Rapporteur cleanup |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correcting typos Changing supportQualifier from C to CM. According to 32.156 "C" should only be used in case of multiple constraints: "Used for items that has multiple constraints". Separated run-time and design-time conditions. Change alarmInformation to alarmRecord Correct some references |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstandings, poor quality, Incorrect references |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.3, 7.4.1, 8.3.2, 8.4.2, A.1.1, A.2.1, A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 7.3.1.3	Attribute constraints

| Name |  | Definition |
| --- | --- | --- |
| rootCauseIndicator correlatedNotifications | At least one of these attributes shall be supported if the MnS producer supports alarm correlation. |  |
| comments | This attribute shall be supported if the MnS producer supports alarm commenting |  |
| ackTime ackUserId ackState ackSystemId | These attributes shall be supported if the MnS producer supports the alarm acknowledgement feature. |  |
| clearUserId clearSystemId |  | These attributes shall be supported for alarm records that represent ADMC alarms. |
| serviceUser serviceProvider securityAlarmDetector | These attributes shall be supported for alarm records that represent security alarms. |  |

Next change

### 7.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| objectClass | Class of a managed object instance. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | Managed object instance identified by its DN. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| systemDN | Distinguished Name (DN) of an MnSAgent. | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| administrativeState | Administrative state of a managed object instance. The administrative state describes the permission to use or prohibition against using the object instance. The administrative state is set by the MnS consumer. allowedValues: LOCKED, UNLOCKED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: LOCKED isNullable: False |
| operationalState | Operational state of manged object instance. The operational state describes if an object instance is operable ("ENABLED") or inoperable ("DISABLED"). This state is set by the object instance or the MnS producer and is hence READ-ONLY. allowedValues: ENABLED, DISABLED. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: DISABLED isNullable: False |
| alarmRecords | List of alarm records | type: AlarmRecord multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| numOfAlarmRecords | Number of alarm records in the AlarmList. allowedValues: Non-negative numbers. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| lastModification | Time an alarm record was modified the last time. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| unreliableAlarmScope | Identifies, the part of the alarm scope that may not be reliable. If this parameter is equal to the instance carried in systemDN, then all AlarmRecord instances in the AlarmList may not be reliable. If this parameter is equal to some instance represented by MonitoredEntity, then only AlarmRecord related to this instance and its descendants may not be reliable. | type: DN multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| alarmId | Identifies an AlarmRecord in the AlarmList. The value is unique within the AlarmList MOI. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notificationId | The Id of the last notification sent as a consequence of updating the AlarmRecord. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmRaisedTime | Date and time the alarm was raised. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmChangedTime | It indicates the last date and time when the AlarmRecord is changed by the alarmed resource. Changes to AlarmRecord caused by invocations of the management service consumer would not change this date and time. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmClearedTime | Date and time the alarm was cleared. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| alarmType | It indicates the type of alarm. Communications Alarm: An alarm of this type is associated with the procedure and/or process required conveying information from one point to another (ITU-T Recommendation X.733 [8]). Quality of Service Alarm: An alarm of this type is associated with degradation in the quality of a service (ITU T Recommendation X.733 [8]). Processing Error Alarm: An alarm of this type is associated with a software or processing fault (ITU T Recommendation X.733 [8]). Equipment Alarm: An alarm of this type is associated with an equipment fault (ITU-T Recommendation X.733 [8]). Environmental Alarm: An alarm of this type is associated with a condition related to an enclosure in which the equipment resides (ITU-T Recommendation X.733 [8]). Security related alarm types Integrity Violation: An indication that information may have been illegally modified, inserted or deleted. Operational Violation: An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service. Physical Violation: An indication that a physical resource has been violated in a way that suggests a security attack. Security Service or Mechanism Violation: An indication that a security attack has been detected by a security service or mechanism. Time Domain Violation: An indication that an event has occurred at an unexpected or prohibited time. Allow values: COMMUNICATIONS_ALARM, QUALITY_OF_SERVICE_ALARM, PROCESSING_ERROR_ALARM, EQUIPMENT_ALARM, ENVIRONMENTAL_ALARM, INTEGRITY_VIOLATION, OPERATIONAL_VIOLATION, PHYSICAL_VIOLATION, SECURITY_SERVICE_OR_MECHANISM_VIOLATION, TIME_DOMAIN_VIOLATION | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| probableCause | It qualifies alarm and provides further information than alarmType. This attribute value shall be single-value and of simple type such as integer or string. See Annex A for a complete listing. | type: string or integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| specificProblem | It provides further refinement to the probableCause. This attribute value shall be single-valued and of simple type such as integer or string. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.2. | type: string or integer multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| perceivedSeverity | It indicates the relative level of urgency for operator attention. allowedValues: CRITICAL, MAJOR, MINOR, WARNING, INDETERMINATE, CLEARED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| backedUpStatus | It indicates if an object (the MonitoredEntity) has a back up. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.4. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: False |
| backUpObject | Backup object of the alarmed object as defined in ITU-T Rec. X. 733 [8] | type: DN multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trendIndication | It indicates if some observed condition is getting better, worse, or not changing. AllowedValues: MORE_SEVERE, NO_CHANGE, LESS_SEVERE | type: ENUM multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| thresholdInfo | It indicates the crossed threshold information such as: - The identifier of the monitored attribute whose value has crossed a threshold, - The threshold settings, - The observed value that have crossed a threshold, etc. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.7. See also for information in 1 32.401 [12] clause 5.6. | type: ThresholdInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| stateChangeDefinition | It indicates attribute value changes associated with the alarm for state attributes of the monitored entity (state transitions). The change is reported with the name of the state attribute, the new value and an optional old value. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeNames and attributeValues. AttributeValues may be complex types. Beside the new value it may contain the old value as well. | type: AttributeValueChange multiplicity: 0..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| monitoredAttributes | It indicates attributes of the monitored entity and their values at the time the alarm occurred that are of interest for the alarm report. How these attributes are chosen is outside of the scope of the present document. See definition in ITU-T Recommendation X.733 [8] clause 8.1.2.11. The content of the attribute is a list of attributeName- attributeValue pairs. AttributeValues may be complex types. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| proposedRepairActions | Used if the cause is known and the system being managed can suggest one or more solutions to fix the problem causing the alarm as defined in ITU-T Recommendation X. 733 [8] | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalText | Allows a free form text description to be reported as defined in ITU-T Recommendation X. 733 [8]. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| additionalInformation | This attribute when present allows the inclusion of a set of vendor specific alarm information in the alarm. A specific condition for this optional population is when an alarm presented by the Management System (e.g. via the user interface) has different values of perceived severity, and / or alarm type, compared with the values presented to the Itf-N. Any other uses of additional information on the alarm and its semantics is outside the scope of the present document The content of the attribute is a list of attributeNames and string attributeValues. | type: NameValuePair multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| rootCauseIndicator | It indicates that this AlarmRecord is the root cause of the events captured by the notifications whose identifiers are in the related CorrelatedNotification instances. | type: boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackTime | It identifies the time when the alarm has been acknowledged or unacknowledged the last time, i.e. it registers the time when ackState changes. | type: DateTime multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackUserId | It identifies the last user who has changed the acknowledgement state. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackSystemId | It identifies the system that last changed the ackState of an alarm, i.e. acknowledged or unacknowledged the alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ackState | It identifies the acknowledgement state of an alarm. AllowedValues: ACKNOWLEDGED, UNACKNOWLEDGED | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearUserId | It carries the identity of the user who invokes the clearAlarms operation. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| clearSystemId | Identifier of a system clearing an alarm | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceUser | It identifies the service-user whose request for service provided by the serviceProvider led to the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceProvider | It identifies the service-provider whose service is requested by the serviceUser and the service request provokes the generation of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| securityAlarmDetector | It carries the identity of the detector of the security alarm. | type: string multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| comments | List of comments and data about the comments. | type: AlarmComment multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| correlatedNotifications | List of correlated notifications. | type: CorrelatedNotification multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| commentTime | Date and Time the comment was created. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentUserId | It carries the identification of the user who made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentSystemId | It carries the identification of the system (Management System) from which the comment is made. That system supports the user that made the comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| commentText | It carries the textual comment. | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.sourceObjectInstance | It identifies one MonitoredEntity. It is unique within a multivalue attribute based on the CorrelatedNotification data type. | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CorrelatedNotification.notificationIdSet | A list of correlated notificationIds. | type: integer multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTEs: none. |  |  |

Next change

### 8.3.2	Input parameters

Table 8.3.2-1: Input parameters for notifyClearedAlarm

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 8.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 8.2.1 |  |
| notificationType | M | "notifyClearedAlarm" |  |
| eventTime | M | alarmRecord.alarmClearedTime |  |
| systemDN | M | See clause 8.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| perceivedSeverity | M | alarmRecord.perceivedSeverity | Value shall be "CLEARED" |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| clearUserId | O | alarmRecord.clearUserId | This parameter shall be present if the AlarmRecord is cleared by the consumer. |
| clearSystemId | O | alarmRecord.clearSystemId | This parameter shall be present if clearUserId is present |

Next change

### 8.4.2	Input parameters

Table 8.4.2-1: Input parameters for notifyChangedAlarmGeneral

| Parameter Name | S | Matching Information/ Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| objectClass | M | See clause 8.2.1 |  |
| objectInstance | M | alarmRecord.objectInstance DN of the MonitoredEntity that is the source of the alarm |  |
| notificationId | M | See clause 8.2.1 |  |
| notificationType | M | "notifyChangedAlarmGeneral" |  |
| eventTime | M | alarmRecord.alarmChangedTime |  |
| systemDN | M | See clause 8.2.1 |  |
| alarmId | M | alarmRecord.alarmId |  |
| alarmType | M | alarmRecord.alarmType |  |
| probableCause | M | alarmRecord.probableCause |  |
| specificProblem | O | alarmRecord.specificProblem |  |
| perceivedSeverity | O | alarmRecord.perceivedSeverity |  |
| backedUpStatus | CO | alarmRecord.backedUpStatus | Used only in non-security notifications. |
| backUpObject | CO | alarmRecord.backUpObject | Used only in non-security notifications. |
| trendIndication | CO | alarmRecord.trendIndication | Used only in non-security notifications. |
| thresholdInfo | CO | alarmRecord.thresholdInfo | Used only in non-security notifications. |
| correlatedNotifications | O | alarmRecord.correlatedNotifications |  |
| stateChangeDefinition | CO | alarmRecord.stateChange | Used only in non-security notifications. |
| monitoredAttributes | CO | alarmRecord.monitoredAttributes | Used only in non-security notifications. |
| proposedRepairActions | CO | alarmRecord.proposedRepairActions | Used only in non-security notifications. |
| additionalText | O | alarmRecord.additionalText |  |
| additionalInformation | O | alarmRecord.additionalInformation |  |
| rootCauseIndicator | O | alarmRecord.rootCauseIndicator |  |
| serviceUser | CM | alarmRecord.securityServiceUser | Avaliable if security alarms are supported. Used only in security notifications. This may contain no information if the identify of the service-user (requesting the service) is not known. |
| serviceProvider | CM | alarmRecord.securityServiceProvider | Avaliable if security alarms are supported. Used only in security notifications. This shall always identify the service-provider receiving a service request, from serviceUser, that provokes the security alarm. |
| securityAlarmDetector | CM | alarmRecord.securityAlarmDetector | Avaliable if security alarms are supported. Used only in security notifications. This may contain no information if the detector of the security alarm is the serviceProvider. |
| changedAlarmAttributes | O | LIST OF SEQUENCE <AttributeName, OldAttributeValue> | The changed alarm attributes (name/value pairs) (with old values). |

Next change

### A.1.1	Mapping of the NRM

The mapping of object classes and attributes follows the general rules defined in TS 32.160 [15], clause 6.

Next change

### A.2.1	General

Mapping of Classes, attributes and notifications is identical to those described in clause A.1.

Next change

### A.3.1	General

The YANG-Netconf solution set uses the same notifications as OpenAPI, see clause A.1.2.

End of  changes