# 3GPP TSG-SA5 Meeting #156	S5-243774

# Maastricht, Netherlands, 19 – 23 August 2024

# Source:	Nokia, Nokia Shanghai Bell

# Title:	TR28.866 pCR add NF Scaling & dimensioning use case in MDA management data correlation analytics

# Document for:	Approval

# Agenda Item:	6.19.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.866-020 “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

The data from different aspects of the network are correlated, that in many cases real analytics value comes from leveraging the correlation among the data. This pCR is to add an MDA use cases on correlation of data for different uses and from different sources.

# 4	Detailed proposal

1st Change

## 5.3	Data correlation analytics

### 5.3.x 	Use case x: correlation analytics for NF Scaling and dimensioning

#### 5.3.x.1	Description

Performance metrics (performance measurements, KPIs) generated by a Network Function (NF) or related NFs may exhibit correlation and association patterns. These patterns can be leveraged to derive NF correlation or dependency relationships, which are valuable inputs for network performance optimization scenarios such as scaling and dimensioning.

-	For performance optimization scenarios (e.g., NF scaling, dimensioning), correlation/association analysis of NF measurement data can yield NF dependency/correlation patterns related to NF scaling and network resource utilization. The analysis output may include recommendations for performance opimization (e.g., optimizing NF scaling and dimensioning).

-	Correlation analytics may be refreshed regularly as correlation relationships can vary across different locations and time periods.

##### 5.3.x.2	Potential requirements

REQ-MDA-CORE-X:	MDA capability for correlation analytics for NF scaling and dimensioning should include the capability for NF dependency/correlation pattern recognition with respect to NF scaling or NF resource usage for dimensioning, and predict recommendations of NF scaling and dimensioning optimization in a coordinated manner.

#### 5.3.x.3	Potential solutions

- Introduce the data type nFScalingDimentionRecommendation to encapsulate recommendations for NF scaling and dimensioning optimization. This data type can be included within the analytics report generated by the MDA for a set of NFs that can be scaled or dimensioned in a coordinated manner.

  - The nFScalingDimentionRecommendation data type may contain a set of recommendations for both 3GPP and non-3GPP NFs.

#### 5.3.x.4	Evaluation of solutions

The solution outlined in clause 5.3.x.3 employs an NRM-based approach. It introduces a new data type, "Recommended3GPPAction" to augment existing information elements. Since the current NRM for recommended actions lacks support for coordination or dependency actions related to performance optimization, implementing the proposed solution on top of the existing NRM is straightforward. Therefore, the approach presented in clause 5.3.x.3 is considered feasible.

End Change