# 3GPP TSG SA WG5 Meeting #156	S5-243825

# Maastricht, The Netherlands 19 - 23 August 2024

# Source:	China Mobile, Huawei

# Title:	Rel-19 pCR 28.851 Support of standalone IMS Data Channel sessions

# Document for:	Approval

Agenda Item:	7.5.3

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.851: "Study on charging aspects of next generation real time communication services phase 2".

3	Rationale

This pCR proposes to add topics on support of standalone IMS Data Channel sessions to TR 28.851 [1].

4	Detailed proposal

The following changes are proposed to be incorporated into TR 28.851.

First change

## 5.x	Topic X: Support of standalone IMS Data Channel sessions

### 5.x.1	Use cases

5.x.1.1	Use case #Xa: Establishment and modification of standalone IMS Data Channel sessions

The standalone IMS data channel can be used for standalone applications that do not require a MMTel session or can be used without any voice or video communication between the connected parties.

The TR 23.700-77 [9] concludes that the following scenarios for IMS session with standalone bootstrap DC, or a combination of standalone bootstrap DC and application DC shall be supported.

a.	An IMS session with only standalone bootstrap DC is used for downloading application list and applications from DCSF;

b.	The UE initiate an IMS session to another UE with only standalone bootstrap DC and further updates the session with application DC;

c.	An IMS session with a combination of standalone bootstrap DC and application DC is used to establish application DC for a downloaded application to the remote UE, which allows the remote UE to download the application if not yet downloaded after accepting the bootstrap DC, and update the session to use the application in the same session.

The SDP offer/answer procedure is used to change an IMS session with audio/video/DC media components to a standalone IMS DC session and adding audio/video media components to an established standalone IMS DC session.

The potential charging requirements for this UC are: REQ-NG_RTC_CH_SIDCS-01, REQ-NG_RTC_CH_SIDCS-02.

### 5.x.2	Potential charging requirements

REQ-NG_RTC_CH_SIDCS-01: The IMS should support converged charging for establishment and modification of standalone IMS Data Channel sessions.

REQ-NG_RTC_CH_SIDCS-02: The 5G system should support converged charging for establishment and modification of standalone IMS Data Channel sessions.

### 5.x.3	Key issues

The key issues are for investigating how to support converged charging for establishment and modification of standalone IMS Data Channel sessions considering REQ-NG_RTC_CH_SIDCS-01 and REQ-NG_RTC_CH_SIDCS-02. This investigation covers the following:

-	Key Issue #xa: Identification of the charging information for converged charging for standalone IMS Data Channel sessions;

-	Key Issue #xb: Identification of the main interactions with the NFs to obtain the charging information.

End of changes