# 3GPP TSG SA WG5 Meeting #156	S5-243828

# Maastricht, The Netherlands 19 - 23 August 2024

# Source:	China Telecom

# Title:	Add a potential solution on measuring the amount of data collected by RE-NWDAF in roaming case

# Document for:	Approval

# Agenda Item:	6.19.18

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TS 28.877-000: "Study on Enhancement of the management aspects related to Network Data Analytics Functions (NWDAF) Phase 2".

[2]	3GPP TS 23.288-i40: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

3	Rationale

(With bullet points, describe the reasons for the proposed action. 
The objectives of the proposal should be clearly stated. 
Rejected alternative solutions should be mentioned if this aids understanding).

(For pseudo CR, the reason for change(s) and summary of change(s) must be clearly explained.)

It is agreed in SA5#154 that the potential solutions with respect to the following requirements needs to be studied [1]:

REQ-NWDAF-PM-DCVOLUME-01：the 3GPP management system shall have a capability to provide the total amount of data for NWDAF data collection from all data sources over a period of time.

REQ-NWDAF-PM-DCVOLUME-02：the 3GPP management system shall have a capability to provide the amount of data for NWDAF data collection from a specific data source over a period of time.

According to TS 23.288 [2] clause 6.2.10, H-RE-NWDAF in the HPLMN as service consumer may subscribe/unsubscribe to notifications about input data from the VPLMN for outbound roaming users (from the HPLMN perspective), and V-RE-NWDAF in the VPLMN may subscribe/unsubscribe to notifications about input data from the HPLMN for inbound roaming users (from the VPLMN perspective) using the Nnwdaf_RoamingData service.

--- Start of quote from TS 23.288---

This procedure may be used by the RE-NWDAF in the HPLMN as service consumer to subscribe/unsubscribe to notifications about input data from the VPLMN for outbound roaming users (from the HPLMN perspective). H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

This procedure may be used by an RE-NWDAF in the VPLMN to subscribe/unsubscribe to notifications about input data from the HPLMN for inbound roaming users (from the VPLMN perspective). H-RE-NWDAF and V-RE-NWDAF in the procedure are NWDAFs with roaming exchange capability.

---End quote---

Therefore, the total amount of data collected by H-RE-NWDAF from V-RE-NWDAF and the total amount of data collected by V-RE-NWDAF from H-RE-NWDAF can be used to monitor the performance of NWDAF performing data collection in roaming case and satisfy REQ-NWDAF-PM-DCVOLUME-01 and REQ-NWDAF-PM-DCVOLUME-02. They can also reflect how much data is exchanged between different PLMNs via NWDAF.

4	Detailed proposal

1st Change

#### 4.3.3.X	Potential solution #X: Amount of data collected by RE-NWDAF in roaming case

##### 4.3.3.X.1	Introduction

This potential solution is proposed to measure the amount of data collected by NWDAF with roaming exchange capability (RE-NWDAF) in roaming case. It reflects how much data is collected by an RE-NWDAF from another RE-NWDAF which acts as a data source and meets the requirements of REQ-NWDAF-PM-DCVOLUME-01 and REQ-NWDAF-PM-DCVOLUME-02. In this case, the RE-NWDAF providing the data is the data source.

##### 4.3.3.X.2	Description

In the roaming scenario, in the case of data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users or the case of data collection by V-RE-NWDAF from H-RE-NWDAF for inbound roaming users, the data is provided via the notification corresponding to the subscription of the Nnwdaf_RoamingData service, according to TS 23.288 [2] clause 6.2.10. In these cases, the RE-NWDAF providing the data can be considered as the data source. Over a given period of time, the total amount of data collected by one RE-NWDAF from the other RE-NWDAF can be measured.

More specifically, the measurement can be provided as:

Amount of data collected by H-RE-NWDAF from V-RE-NWDAF performed by H-RE-NWDAF:

In the case of data collection by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users, the measurement is provided by accumulating the amount of data collected by H-RE-NWDAF from V-RE-NWDAF over a period of time. When the H-RE-NWDAF receives a notification corresponding to the subscription of Nnwdaf_RoamingData service from the V-RE-NWDAF in VPLMAN, the amount of data is added to the relevant counter for the V-RE-NWDAF. This measurement reflects how much data is collected by H-RE-NWDAF from V-RE-NWDAF for outbound roaming users.

Amount of data collected by V-RE-NWDAF from H-RE-NWDAF performed by V-RE-NWDAF:

In the case of data collection by V-RE-NWDAF from H-RE-NWDAF for inbound roaming users, the measurement is provided by accumulating the amount of data collected by V-RE-NWDAF from H-RE-NWDAF over a period of time. When the V-RE-NWDAF receives a notification corresponding to the subscription of Nnwdaf_RoamingData service from the H-RE-NWDAF in HPLMAN, the amount of data is added to the relevant counter for the H-RE-NWDAF. This measurement reflects how much data is collected by V-RE-NWDAF from H-RE-NWDAF for inbound roaming users.

End of Changes