3GPP TSG-SA5 Meeting #156 	S5-243871

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson Hungary Ltd.

# Title:	Rel-19 pCR TR 28.871 Reliable notification transfer

# Document for:	Approval

Agenda Item:	6.19.8

# 1	Decision/action requested

Approve the proposal.

# 2	References

[1]	3GPP TR 28.871 Study on Service Based Management Architecture enhancement phase 3

[2]	3GPP TS 32.158 Design rules for REpresentational State Transfer (REST) Solution Sets (SS)

# 3	Rationale

Consumers use information received in notifications. Some consumers depend on receiving notifications reliably and in sequence. Notification delivery is not always reliable. Current specification doesn’t provide mechanisms for detecting missing or out-of-order notifications or a mechanism to recover the missed information.

We propose to improve the reliability of notification transfer.

# 4	Detailed proposal

First change

# 5.a Reliable notification transfer

### 5.a.1 Description

The handling of notifications has been identified as an area of improvement because there is no guarantee that notifications are always successfully sent to and received by consumers. Consumers might be unaware of the misalignment with the producer. There are multiple conditions that can cause the misalignment:

- Network issues: both temporary and permanent outages. These may be caused both by real connectivity issues or unavailability of the notification recipient.

- Producer internal issues while sending the notification, (e.g. overload situation) may result in notifications not being sent

- Consumer exceptions reported to the producer e.g. with HTTP response codes (4xx, 5xx).

- Notification may be received out of order e.g. if separate https connections are used to transfer subsequent notifications.

We need a reliable mechanism, to detect abnormal conditions and to recover from them. While Heartbeat checks the network connection, it does not check that the individual notifications are successfully transferred.

To introduce a reliable notification transfer, the following responsibilities have been identified:

- The Producer is responsible for detecting unsuccessful delivery of the notification (secured by HTTPS protocol and its response codes), storing those undelivered notifications, and providing them when requested.

- The consumer is responsible for discovering lost or out of order notifications, retrieving them from the producer, and managing them in the proper way.

The feature shall be optional to support, but if supported its use shall be mandatory according to the proposed solution below.

5.a.2 Potential requirements

REQ-rel-notif-1: Producer shall be capable of detecting unsuccessful delivery of the notification and its reason. Detection shall be based on detecting HTTPS connectivity problems and HTTPS response codes.

REQ-rel-notif-2: Producer shall be capable of storing undelivered notifications.

REQ-rel-notif-3: The producer shall provide a mechanism for the consumer that allows it to discover missing or out-of-order notifications.

REQ-rel-notif-4: The producer shall provide a mechanism for the consumer that allows it to retrieve undelivered notifications from the producer.

### 5.a.3 Potential solutions

#### 5.a.3.1 Part#1 Add sequence number

Delivery of notifications can be prevented by many reasons (bugs, firewalls, proxy or concentrator nodes, etc.) some of which cannot be foreseen at the moment. The smartest approach to ensure reliability has been identified as adding a sequence number to all notifications so that the consumer can identify ‘holes’ in the sequence of the notifications and out-of-order notifications. Sequence numbering is a robust mechanism that can detect problems both for the current HTTP 1.1 protocol (see [2]) or any past or future protocols (e.g. HTTP 3). If every notification carries a sequence number, the consumer can detect holes in the sequence and start a recovery procedure.

Furthermore, notifications can arrive out-of-order. The sequence number can be used to re-order the notifications. If the HTTP connection is closed after sending a notification and a new connection is opened for the subsequent notification, the order of delivery is not guaranteed.

Heartbeat notifications shall not carry an own sequence number, that is the sequence number shall not be increased when sending a notifyHeartbeat, but they shall carry the last used sequence number. This guarantees that if there is a failure and no more notifications are sent in the heartbeat interval, the consumer can identify the issue and start a recovery procedure. Heartbeat notifications shall also carry a new extra parameter lastEventTime that contains the eventTime paramter from the last (non-heartbeat) notification.

The new sequence number attribute shall be mandatory for all the notification types except the notifyHeartbeat. The sequence number shall be a monotonously increased integer that will be separate for each notification subscription.

Impacted specifications are: 28.532 and 28.111.

#### 5.a.3.2 Part#2 Request failed notifications

To ensure that notifications that failed to be delivered can be retrieved later, the producer shall store them in a temporary area, so that the consumer can ask for them as soon as it identifies the need for  recovery (e.g., checking the sequence number). The producer shall expose a new IOC FailedNotifications so that the consumer can retrieve missed notifications."

The IOC FailedNotifications shall contain the failed notifications and 3 additional attributes: lastSequenceNumber (equivalent to the notifyHeartbeat parameter), lastEventTime (equivalent to the notifyHeartbeat parameter) and numberOfFailedNotifications. The attributes of this IOC shall be isNotifiable=False, otherwise an inifinte number of value change notifications could be generated.

Impacted specifications are: 28.622 and 28.623.

### 5.a.4 Evaluation of potential solutions

Solution (part #1 and #2 together) fulfils the proposed requirements.

End of change