3GPP TSG-SA5 Meeting #156	S5-243920

# Maastricht, Netherlands, 19 – 23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.558 | CR | 0019 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.558 Replacing S-TMSI with TR/TRSR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The following observations are made wrt the UE identifier for the UE level measurements for NG-RAN. - In the current specification, S-TMSI is used as UE identifier. In the LS from RAN3, R3-243941, it has been mentioned that the S-TMSI is not available in the gNB-DU or in the gNB-CU-UP. - In case of logged MDT, the UE measures the neighboring cells in RRC_IDLE state and when connected back to the network, these reports are being shared with the network. When the UE moves from RRC_IDLE to RRC_Connected state, a new 5G-TMSI is allocated to the UE that is different than the old one. There will be two different 5G-TMSI identifiers for the same UE and the reports from this same UE will be interpreted as different UEs based on the different values of 5G-TMSI. Therefore, is a short lived identifier that may not survive during the course of an MDT session and cannot be identified in the OAM. - In case of Handovers, the 5G-TMSI information is not passed with the UE context and hence this cannot be used in such scenarios. The proposal is to use TR/TRSR (Trace Reference / Trace Recording Session Reference) instead of 5G-TMSI. The same TR/TRSR is used by the UE to share logged MDT reports and those reports can be inferred as the reports from the same UE and not from different UEs. TR/TRSR can uniquely identify a trace or MDT session in the OAM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | S-TMSI used as UE identifier for gNB related measurements have been replaced with TR/TRSR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification where in UE identifier that cannot identify a trace or MDT session uniquely is used |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.1, 6.3.1.1.2, 6.3.1.1.3, 6.3.1.1.4, 6.3.1.1.5, 6.3.1.1.6, 6.3.1.1.7, 6.3.1.1.8, 6.3.1.2.1, 6.3.1.2.2, 6.3.1.3.1, 6.3.1.3.2, 6.3.1.3.1, 6.3.1.4.1, 6.3.1.4.2, Annex A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of first Change

## 6.3	UE level measurements definitions for NG-RAN

### 6.3.1	UE level measurements definitions for gNB

#### 6.3.1.1	Packet delay

##### 6.3.1.1.1	Average delay DL air-interface

a)	This measurement provides the average (arithmetic mean) time it takes for packet transmission over the air-interface in the downlink direction. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained as: sum of (point in time when the last part of an RLC SDU packet was sent to the UE which was consequently confirmed by reception of HARQ ACK from UE for UM mode or point in time when the last part of an RLC SDU packet was sent to the UE which was consequently confirmed by reception of RLC ACK for AM mode, minus time when corresponding RLC SDU part arriving at MAC layer) divided by total number of RLC SDUs transmitted to UE successfully. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.AirIfDelayDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.2	Average delay DL in gNB-DU

a)	This measurement provides the average (arithmetic mean) RLC SDU delay on the downlink within the gNB-DU, for initial transmission of all RLC packets. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D2 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when the last part of an RLC SDU was scheduled and sent to the MAC layer for transmission over the air, minus time of arrival of the same packet at the RLC ingress F1-U termination) divided by total number of RLC SDUs arriving at the RLC ingress F1-U termination. If the RLC SDU needs retransmission (for Acknowledged Mode) the delay will still include only one contribution (the original one) to this measurement. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs. 
[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.RlcSduDelayDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.3	Average delay DL on F1-U

a)	This measurement provides the average (arithmetic mean) GTP packet delay DL on the F1-U interface. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D3 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained as: the time when receiving a GTP packet from the gNB-DU at the ingress GTP termination of GNBCUUPFunction, minus time when the same packet was sent to gNB-DU from the GTP egress termination of GNBCUUPFunction, minus feedback delay time (including queuing delay) in gNB-DU, obtained result is divided by two. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

Figure 6.3.1.1.3-1 Average delay DL on F1U

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs. 
 [Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpF1DelayDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	GNBCUUPFunction

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

NOTE : The NR RAN container (DL USER DATA/ DL DATA DELIVERY STATUS) carried in the GTP-U packet over the F1-U interface is used for the measurement.

##### 6.3.1.1.4	Average delay DL in CU-UP

a)	This measurement provides the average (arithmetic mean) PDCP SDU delay on the downlink within the gNB-CU-UP, for all PDCP packets. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D4 (see TS 38.314 [8]) as part of DL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained as: sum of (time when sending a PDCP SDU to the gNB-DU at the egress PDCP layer on F1-U/Xn-U, minus time of arrival of the same packet at NG-U ingress IP termination) divided by total number of PDCP SDUs arriving at NG-U ingress IP termination. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per S-NSSAI.

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of S-NSSAIs. 
[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpSduDelayDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	GNBCUUPFunction

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.5	UL PDCP packet average delay

a)	This measurement provides the average (arithmetic mean) UL PDCP packet average delay. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D1 (see TS 38.314 [8]) as part of UL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained according to the definition in clause 4.3.1.1 of TS 38.314 [8], named "UL PDCP Packet Average Delay per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpDelayUlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	GNBCUUPFunction

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.6	Average delay UL on over-the-air interface

a)	This measurement provides the average (arithmetic mean) over-the-air packet delay on the uplink. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D2.1 (see TS 38.314 [8]) as part of UL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained according to the definition in clause 4.2.1.2.2 of TS 38.314 [8], named "Average over-the-air interface packet delay in the UL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a real representing the mean delay in 0.1 millisecond. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.AirIfDelayUlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.7	Average RLC packet delay in the UL

a)	This measurement provides the average (arithmetic mean) RLC packet delay on the uplink, i.e., the delay within the gNB-DU. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D2.2 (see TS 38.314 [8]) as part of UL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained according to the definition in clause 4.2.1.2.3 of TS 38.314 [8], named "Average RLC packet delay in the UL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a real representing the mean delay in the unit 0.1 milliseconds. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.RlcDelayUlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU.

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

##### 6.3.1.1.8	Average PDCP re-ordering delay in the UL

a)	This measurement provides the average (arithmetic mean) PDCP re-ordering delay on the uplink, i.e., the delay within the gNB-CU-UP. The measurement is calculated per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as D2.4 (see TS 38.314 [8]) as part of UL M6 measurement (see TS 37.320 [9]).

b)	DER (n=1)

c)	This measurement is obtained according to the definition in clause 4.2.1.2.4 of TS 38.314 [8], named "Average PDCP re-ordering delay in the UL per DRB per UE”. The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is a real representing the mean delay in the unit 0.1 milliseconds. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

[Total No. of measurement instances] x [No. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form DRB.PdcpReordDelayUlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	GNBCUUPFunction

g)	TR/TRSR.

h)	One usage of this measurement is to support the end-to-end data volume transfer time analytics conducted by NWDAF (see TS 23.288 [7]).

#### 6.3.1.2	Packet Loss for all gNB deployment scenario

##### 6.3.1.2.1	DL Packet Loss Rate on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate on Uu interface. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.1 of TS 38.314 [8], named "Packet Uu Loss Rate in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateUuDlUe.Filter, 
Where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.2.2	DL Packet Loss Rate with delay threshold on Uu

a)	This measurement provides the DL Packet (i.e., RLC SDU) Loss rate including any packets not successfully transmitted or packets successfully received but delayed more than a delay threshold that can be used when the resource type of corresponding QoS Flow is Delay-critical GBR (clause 5.7.3.4 in TS 23.501[2]）on Uu. The measurement is split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. This measurement is also referred to as M7 in TS 37.320 [9].

b)	CC

c)	This measurement is obtained according to the definition in clause 4.2.1.5.2 of TS 38.314 [8], named "Packet Uu Loss Rate with delay threshold in the DL per DRB per UE". The measurement is performed per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value. The number of measurements is equal to the number of QoS levels multiplied by the number of supported S-NSSAIs.

e)	DRB.PacketLossRateDTUuDlUe.Filter, 
where Filter is a combination of QoS level and S-NSSAI, QoS level represents the mapped 5QI or QCI level, and SNSSAI represents S-NSSAI.

f)	NRCellDU

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

#### 6.3.1.3	Packet loss for split gNB deployment scenario

##### 6.3.1.3.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface. Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.PacketLossRateUlUe, and optionally
DRB.PacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.PacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction;

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.3.2	UL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing UL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the GTP sequence number of the first packet delivered by gNB-DU to gNB-CU-UP until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateUlUe, and optionally DRB.F1UPacketLossRateUlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateUlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	GNBCUUPFunction

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.3.3	DL F1-U Packet Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at the gNB-DU). It is a measure of the DL packet loss on the F1-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3), and subcounters per supported S-NSSAI. This measurement is also referred to as DL M7 in TS 37.320 [9].

b)	SI

c)	This measurement is obtained as: 1,000,000* Number of missing DL GTP sequence numbers (TS 29.281 [15]), representing packets that are not delivered to lower layers, of a data radio bearer, divided by Total number of UL GTP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by gNB-CU-UP to gNB-DU until the GTP sequence number of the last packet. Separate counters are optionally maintained for mapped 5QI (or QCI for NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form DRB.F1UpacketLossRateDlUe, and optionally DRB.F1UPacketLossRateDlUe.QoS where QoS identifies the target quality of service class, and DRB.F1UPacketLossRateDlUe.SNSSAI where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

#### 6.3.1.4	UE throughput

##### 6.3.1.4.1	Average DL UE throughput in gNB

a)	This measurement provides the average UE throughput in downlink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of if using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI. This measurement is also referred to as DL M5 in TS 37.320 [9].

b)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolDl" and "ThpTimeDl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for option 3) and for each supported S-NSSAI.

If ThpTimeDl>0, ThpVolDlThpTimeDl×1000 [kbit/s]

If ThpTimeDl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission,, otherwise

| ThpTimeDl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeDl" for each time the DL buffer for one Data Radio Bearer (DRB) is emptied. |
| --- | --- |
|  | The point in time after T2 when data up until the second last piece of data in the transmitted data burst which emptied the RLC SDU available for transmission for the particular DRB was successfully transmitted, as acknowledged by the UE. |
|  | The point in time when the first transmission begins after a RLC SDU becomes available for transmission, where previously no RLC SDUs were available for transmission for the particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolDl is the data volume, counted on RLC SDU level, in kbit successfully transmitted (acknowledged by UE) in DL for one DRB during a sample of ThpTimeDl. (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form 
DRB.UEThpDl, or optionally 
DRB.UEThpDl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpDl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

##### 6.3.1.4.2	Average UL UE throughput in gNB

a)	This measurement provides the average UE throughput in uplink. This measurement is intended for data bursts that are large enough to require transmissions to be split across multiple slots. The UE data volume refers to the total volume scheduled for each UE regardless of using only primary- or also supplemental aggregated carriers. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in NR option 3) and subcounters per supported S-NSSAI. This measurement is also referred to as UL M5 in TS 37.320[9].

B)	DER(N=1)

c)	This measurement is obtained according to the following formula based on the "ThpVolUl" and "ThpTimeUl" defined below. Separate counters are maintained for each mapped 5QI (or QCI for option 3) and for each supported S-NSSAI.

If ThpTimeUl>0, ThpVolUlThpTimeUl×1000 [kbit/s]

If ThpTimeUl=0, 0 [kbit/s]

For small data bursts, where all buffered data is included in one initial HARQ transmission otherwise

| ThpTimeUl | The time to transmit a data burst excluding the data transmitted in the slot when the buffer is emptied. A sample of "ThpTimeUl" for each time the UL buffer for one Data Radio Bearer (DRB) is emptied. |
| --- | --- |
|  | The point in time when the data up until the second last piece of data in data burst has been successfully received for a particular DRB |
|  | The point in time when transmission is started for the first data in data burst for a particular DRB. |
|  | The RLC level volume of a data burst, excluding the data transmitted in the slot when the buffer is emptied. A sample for ThpVolUl is the data volume counted on RLC SDU level in kbit received in UL for one DRB during a sample of ThpTimeUl, (It shall exclude the volume of the last piece of data emptying the buffer). |

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter and S-NSSAI subcounter are performed, the number of measurements is equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

e)	The measurement name has the form 
DRB.UEThpUl, or optionally 
DRB.UEThpUl.QoS, where QoS identifies the target quality of service class, and 
DRB.UEThpUl.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NRCellDU

g)	TR/TRSR

h)	One usage of this measurement is to support ML training and performance evaluation.

Annex A (normative):
Template for definitions of UE level measurements

Following is the template for defining the UE level measurements for 5G system.

Clause title: descriptive measurement name

This is a descriptive name of the measurement type that is specified as clause X of the present document.

a)	Description

This subclause contains an explanation of the measurement.

b)	Collection Method

This n contains the form in which this measurement data is obtained:

-	CC (Cumulative Counter);

-	GAUGE (dynamic variable), used when data being measured can vary up or down during the period of measurement;

-	DER (Discrete Event Registration), when data related to a particular event are captured every nth event is registered, where n can be 1 or larger;

-	SI (Status Inspection);

-	TF (Transparent Forwarding);

-	OM (Object Mapping).

c)	Condition

This subclause contains the condition which causes the measurement result data to be updated;
This will be defined by identifying protocol related trigger events for starting and stopping measurement processes, or updating the current measurement result value. Where it is not possible to give a precise condition, then the conditional circumstances leading to the update are stated.

If a measurement is related to “external” technologies, this subclause shall give a brief reference to other standard bodies.

d)	Measurement Result (measured value(s), Units)

This subclause contains a description of expected result value(s) (e.g. a single integer value). If a measurement is related to “external” technologies, this subclause shall also give a brief reference to other standard bodies.

The definition applies for each measurement result.

e)	Measurement Type

This subclause contains a short form of the measurement name specified in the header, which is used to identify the measurement type in the result files.

The measurement names are dotted sequences of items. The sequence of elements identifying a measurement is organised from the general to the particular:

-	The first item identifies the measurement family (e.g. RRC, DRB, HO). Note that this family may also be used for measurement administration purpose.

-	The second item identifies the name of the measurement itself, for which the following rules shall apply:

- The second item of the measurement name can be divided into <Operation>, <Reason/Result> and <Direction> (and in that order)

- Examples of Operation can be Establishment, Release, and Modification

- Examples of Reason/Result can be Attempt, Failure, Success, Throughput and Volume

- Examples of Direction can be Incoming, Outgoing, Uplink and Downlink

-	Depending on the measurement type, additional items may be present to specify subcounters (failure causes, traffic classes, min, max, avg, G, U, ...). In case of multiple additional items, they are also represented as a dotted sequence of items. When available, the template will describe to which standard it is referring to for these additional items (e.g. cause, traffic class). Otherwise, the additional item semantics need to be described in detail in the present document.

Standardised causes will be a number. This number shall be derived according to which of the following rules applies:

-	For the standardised causes with numeric values explicitly specified in the reference specification, the subcounter name will be the number assigned to this cause in this reference specification.

-	For the standardised causes without numeric values explicitly specified in the reference specification, but where the causes are identified, the subcounter name shall be a number from 1 to n mapped in an incremental sequence to each of the specified causes following top-down sequence in the order they are identified in the reference specification (e.g. RRC.ConnEstabAtt.1, 1 identifies the establishment cause "emergency", see TS 38.331 [6]).

-	For the standardised causes without numeric values explicitly specified in the reference specification and the causes identified and the causes have been divided into different cause groups, the subcounter should be defined as the form of ‘Cause group’.Cause, where:

-	the subcounter name of ‘Cause group’ shall be an incremental number from 1 to n to identify each cause group specified in a top-down sequence following the order they are identified in the reference specification;

-	the subcounter name of cause within this cause group shall be an incremental number from 1 to n to identify each cause within the group specified in a top-down sequence following the order they are identified within the cause group in the reference specification;

the non standardised causes should be a string (e.g. RRC.ConnEstab.NoReply).

f)	Measured Object Class

This subclause describes the measured object class (e.g., NRCellCU, NRCellDU, GNBCUCPFunction, UPFFunction). The object class used for this purpose shall be in accordance with NRMs defined in any NRM IRPs, such as those defined in 3GPP TS 28.541 [3].

For object classes currently not defined according to the above, the present document defines its own nomenclature.

It is possible to use the same measurement name for a standardized measurement type implemented at a different object class level than the one defined in the Standard. The same measurement type can apply to one or more measurements for which all fields of the measurement template are the same except the item f) "Measurement Object Class". For instance, a measurement which uses the same template as a given measurement type but relates to another or different object classes shall have the same name.

g)	Measured UE Identifier

This element indicates what type of the identifiers of the UE needs to be provided. The type of the UE identifier could be IMSI, IMEI, SUPI, N4 Session ID or RAN UE ID, TR/TRSR (Trace Reference/Trace Recording Session Reference, refer to clause 5.6 and 5.7 of [13]), etc., subject to the specific measurement. For MDT measurements, TR/TRSR can be used as this identifies the trace or MDT session.

h)	Purpose

This optional clause aims at providing additional information on the interest of the measurement but is purely indicative.

End of Changes