3GPP TSG-SA5 Meeting #156 	S5-243992

Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	ZTE Corporation

# Title:	Add potential solutions on resource load of RedCap network

# Document for:	Approval

Agenda Item:	6.19.17

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

# 3	Rationale

This contribution proposes to add potential solutions on resource load of RedCap network service.

# 4	Detailed proposal

1st Change

## 5.6	Use case #6:  Performance evaluation on resource load of RedCap network service

### 5.6.1	Description

The radio resource utilization measurements could provide operators the load information of the radio network during the measurement time period. Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) relative to eMBB (enhanced mobile broadband) devices due to their reduced capabilities and lower data rate requirements. PRBs related measurements may include total PRB usage, PDSCH PRB Usage, PUSCH PRB Usage etc.

In TS 28.552 [10], radio resource utilization metrics are used to evaluate the performance of the entire network, but the current approach combines the resource utilization of RedCap UEs and non-RedCap UEs, which may not accurately reflect the performance of the RedCap service.

RedCap UEs typically have reduced capabilities and different performance characteristics compared to standard UEs.

Compared to non-Redcap UEs, RedCap UEs use fewer network resources (e.g. PRBs) related to eMBB (enhanced mobile broadband) devices. By distinguishing between the two types of UEs, network operators can better identify and troubleshoot specific network resource issues related to RedCap UEs and non-RedCap UEs, thus can allocate resources more efficiently and ensure that RedCap UEs receive adequate resource.

Therefore, it is essential to investigate an approach for separately calculating the resource load of RedCap UEs and defining the resource load performance of networks that deliver RedCap service. This will ensure a more precise evaluation of the user experience for RedCap UEs, thus make the operator to be aware of whether a cell has ever experienced high load or not in the monitoring period, and is a key input to network capacity planning and load balancing for RecCap service.

Similarly, to distinguish RedCap UEs and non-RedCap UEs for metric of the number of the active UEs in each cell is helpful for operators to know how many DRBs are running with buffered data per cell for different kinds of UEs.

Through the metrics of radio resource utilization and active users of RedCap UEs and non-RedCap UEs, operators can better evaluate the overall consumption of network resources of RedCap UEs to facilitate adjustment of related network policies.

### 5.6.2	Potential requirements

REQ-RedCap-Perf-Resource: The 3GPP management system should have capability to provide measurements related to resource load of RedCap service.

### 5.6.3 Potential Solutions

#### 5.4.3.1 Potential solution #1

This solution proposes to reuse and enhance the existing measurements related to Radio resource utilization defined in TS28.552 [10] for this use case.

Current measurements about UE throughput includes DL Total PRB Usage, UL Total PRB Usage, PDSCH PRB Usage, PUSCH PRB Usage, PRB Usage per SSB(See clause 5.1.1.2 in TS 28.552 [10].), etc. In order to achieve the requirement in clause 5.4.2, most aspects of the existing measurements can be reused and some enhancements also need to be introduced.

Also, clause 5.1.1.23 in TS 28.552 [10] Number of Active UEs can also be reused to reflect the load of RedCap service including: Active UEs in the DL per cell, Active UEs in the UL per cell, Active UEs per cell, etc.

In radio access procedure in 5G system, UEs (no matter legacy UEs or RedCap UEs) need to report to gNB about the NR UE Radio Access Capability Parameter using UE-NR-Capability IE, which is specified in clause 6.3.3 in TS 38.331 [2]. RedCapParameters is part of UE-NR-Capability IE which can be delivered by the UE capability inquiry process.

UE-NR-Capability can be used to distinguish RedCap UEs and non-RedCap UEs. 
So it can be used as subcounters filtered by the new filter with the value of RedCap can be used as metrics of RedCap UE resource utilization.

End of change