3GPP TSG-SA5 Meeting #156 	S5-244093

Maastricht, Netherlands, 19 - 23 August 2024

# Source:	Huawei

# Title:	Add conclusion for service failure recovery

# Document for:	Approval

Agenda Item:	6.19.2

# 1	Decision/action requested

For approval

# 2	References

[1] 	3GPP TR 28.866: “Study on Management Data Analytics (MDA) – Phase 3”.

# 3	Rationale

This contribution proposes to add evaluation and conclusion for the use case on enhancing service failure recovery.

# 4	Detailed proposal

It is proposed to make the following changes to TR 28.866 [1].

1st change

### 5.7.3	Use case 3: Enhancing service failure recovery

#### 5.7.3.1	Description

TS 28.104 [2] clause 7.2.3.2 describes a use case for service failure recovery. To explore possible issues, this use case is elaborated in more detail as follows:

1. The network operator discovers that a service problem exists. This may be a failed service or a degraded service. The network operator may discover the issue because of an alarm or because of performance measurements.

2. The network operator requests MDA to analyse the service problem.

3. MDA produces a report, which includes recommendations to resolve the issue.

4. The network operator evaluates the recommendations.

5. If the network operator accepts the recommendations, the network operator implements the recommended actions.

The above use case raises the following issues:

It is not clear how the MDA MnS consumer should request analysis of a particular service problem. The MDARequest IOC contains an attribute analyticsScope which may be expressed as managedEntitiesScope. The definition of managedEntitiesScope says that it “carries the DN(s) of SubNetwork MOI(s), ManagedElement MOI(s), and/or the MOI(s) of the derivative IOCs of ManagedFunction”. It is not obvious how a service problem can be identified by any of the allowed DN types. For example, if the MDA MnS consumer wishes to use an alarm to identify a service failure, it is not possible to express an alarm identifier in managedEntitiesScope.

It is not clear which MDA type supports the use case for service failure recovery. The analytics output for MDA type MDAAssistedFaultManagement.FailurePrediction includes an information element recommendedActions which mentions “recovery”, but there is no description of what this means. Also, the related analytics output contains mandatory information elements which are not appropriate for service failure recovery.

MDA type MDAAssistedFaultManagement.FailurePrediction contains recommendations on how to resolve a predicted failure, but there is not enough information for the MDA MnS consumer to evaluate whether the recommendations are valid or not. Missing important information includes the root cause of the service problem and demarcation information. This information is essential to allow the MDA MnS consumer to trust the recommendations and to discover underlying issues that could cause a similar service problem to occur in the future.

#### 5.7.3.2	Potential requirements

REQ-FAILURE_RECORV_MDA-01: MDA capability for service failure recovery shall allow a consumer to select an alarm for analysis.

REQ-FAILURE_RECORV_MDA-02: MDA capability for service failure recovery shall be able to provide the analytics output including the probable cause of the failure and recommended actions to resolve the service failure.

#### 5.7.3.3	Potential solutions

Update TS 28.104 [2] to add a new MDA type MDAAssistedFaultManagement.FailureRecovery to allow an MnS consumer to request analytics related to service failure recovery. The attribute analyticsScope may need to be extended to allow the MDA MnS consumer to use an alarm identifier to indicate the scope of the analysis.

Update TS 28.104 [2] to add a new analytics output for service failure recovery analysis, including (but not limited to) information related to

- related alarm(s) which are likely to be correlated to the service failure

- recommended actions to resolve the service failure

Update TS 28.104 [2] to remove the term “recovery” in the failure prediction use case and MDA type MDAAssistedFaultManagement.FailurePrediction.

#### 5.7.3.4	Evaluation of solutions

There is one proposed solution which satisfies the requirements, and the solution is feasible.

2nd change

# 6	Conclusions

The MDA MnS producer needs to provide a new MDA capability for service failure recovery:

- analysis of the probable cause of the service failure

- recommended actions to resolve the service failure

End of changes