3GPP TSG-SA5 Meeting #156 	S5-244122

Maastricht , NL, 19 - 23 Aug 2024

# Source:	China Unicom, Huawei

# Title:	pCR TR 28.914 Add evaluation, conclusion and recommendation for Use Case #9

# Document for:	Approval

Agenda Item:	6.19.3

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	SP-231737 "Study on intent driven management services for mobile network phase 3"

[2]	3GPP TR 28.914: " Study on intent driven management service for mobile network phase 3 v0.2.0"

3	Rationale

It was approved to study new scenarios for intent driven management for mobile network related to WT-2 in SP-231737 [1]. The corresponding use case, requirement and potential solution on radio network expectation to support MOCN were approved in [2] during last two meetings.

This contribution proposes to add evaluation, conclusion and recommendation for Issue #9 enhancement of radio network expectation to support MOCN.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.9.4 Evaluation of potential solutions

The possible solution described in clause 5.9.3.1 enhances the existing RadioNetworkExpectation with two new ExpectationTargets: PRBsTarget and InterRAThandoverTarget. PRBsTarget describes total available number of physical resource blocks (PRBs). and InterRAThandoverTarget shows how often a handover from 5GS to EPS is successful. The change is lightweight and it is backward compatible, therefore it is a feasible solution.

2nd Change

6	Conclusion and Recommendation

6.X	Use case #9: Enhancement of Radio Network Expectation to support MOCN

This use case identifies that there will be differentiated radio service caused by a variety of factors in MOCN scenario, which will affect the user experience, especially the impact on the POP’s users. In order to ensure radio network for MOCN with undifferentiated radio service for MOP and POP, the potential solution proposes some enhancements on the existing radio network expectation to support radio network assurance for MOCN. Detailed description about this solution is in clause 5.9.3.1.

It is recommended to make some enhancement on the existing RadioNetworkExpectation according to this solution in the future normative work to satisfy the requirements for MOCN with undifferentiated radio service.

End of changes