3GPP TSG-SA5 Meeting #156 	S5-244131

Maastricht, Netherlands, 19 - 23 August 2024

# Source:CSCN

# Title: pCR TR28.846 NG-RAN on-board sharing charging between operators

# Document for:	Approval

Agenda Item:	7.5.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]		3GPP TS 22.101 V19.0.0 Service aspects; Service principles

[2]		3GPP TS 22.261 V19.7.0 Service requirements for the 5G system; Stage 1

[3]		3GPP TS 23.501 V19.0.0 System architecture for the 5G System (5GS)

[4]		3GPP TS 28.552 V19.0.0 5G performance measurements

3	Rationale

3.1Business scenario for NG-RAN sharing between multiple satellite operators.

In clause 5.18 of the TS 23.501V19.0.0, Network Sharing is supported, In this Release of the specification, the 5G Multi-Operator Core Network (5G MOCN) network sharing architecture, in which only the RAN is shared in 5G System, is supported. The 5G System may also support Indirect Network Sharing deployment between shared operator (i.e. shared network operator) and participating operator (see clause 6.21 of TS 22.261 [2], Figure 5.18.1-2 and Annex R), in which the RAN is shared. The communication between the shared RAN and the core network of the participating operator is routed through the core network of the shared operator that connects to the shared RAN.

In 6.21.2.1 of TS 22.261 V19.7.0, A 5G satellite access network shall support NG-RAN sharing.SA2 is studying gNB deployed on the satellite. As satellites are constantly orbiting in space, they spend 70% of their time flying over other countries. To improve the utilization of gNB, sharing them with other operators is a viable business model. Considering latency, the following shared mode is preferable for satellite operator.

Figure 3-1: NG-RAN on-board sharing

In the clause 28.2.3 of TS 22.101 V19.0.0, following requirements have been requested.

A shared NG-RAN Operator shall be able to collect events supporting the accounting of network resource usage separately for each Participating Operator. Collected events may be delivered to the subscriber's Participating Operator. This includes:

-	start of service in the Shared NG-RAN for a UE of the Participating Operator,

-	end of service in the Shared NG-RAN for a UE of the Participating Operator.

3.2Potential charging requirements

Participating Operator could be charged by shared RAN Operator based on the total data volume transferred via the satellite, or based on resource usage.

4	Detailed proposal

This document proposes the following changes in TR 28.846.

1st Change

5. X	Business scenario for NG-RAN sharing network is shared between multiple satellite operators

Business scenario#1x: NG-RAN on-board sharing. RAN is shared in 5G System. 5G MOCN for 5G System, including UE, RAN and AMF, shall support operators' ability to use more than one PLMN ID.The shared network operator allocates shared resources to the participating operators based on their planned and current needs and according to service level agreements. Participating Operator could be charged by shared RAN Operator based on the total data volume transferred via the satellite, or based on resource usage.

Next Change

6	Charging scenarios and key issues

6. X	Topic 1. Participating Operator could be charged by shared RAN Operator

6. X.1	Use cases

6. X.1.1	Use case #1x: Shared RAN operator charging participating operator for 5G data connectivity provided to the participating operator’s subscribers

This use case focuses on shared RAN Operator and Participating Operator business roles.

A CSC (UE) has a subscription with the Participating Operator which allows usage of 5G data connectivity. The CSC (UE) access network through the shared gNB.

The shared RAN Operator need to charge Participating Operator based on data volume or resource usage that the Participating Operator UE has used in the Shared RAN operator’s gNB.

For this case, the charging party and charged party can be:

-	Charged party: Participating Operator.

-	Charging party: Shared RAN Operator.

- X.2	Potential charging requirements

The following are potential high-level charging requirements for satellite in 5GS, derived from the requirements in TS 22.101 [2].

REQ-CH_ SATAC_RS-01: The charging mechanism in shared RAN Operator should support collecting charging information related to 5G data connectivity usage for each Participating Operator.

6. X.3	Key issues

6. X.3.1	Key issue#1.1: Charging information required

This key issue is for investigating how to support the NG-RAN sharing service charging considering REQ-CH_ SATAC_RS-01. This investigation covers the following:

-	Aggregation of charging information in shared RAN Operator for 5G data connectivity usage per Participating Operator for NG-RAN sharing.

6. X.3.2	Key issue #1.2: NF/Service suitable to provide charging information

This key issue is for investigating how to support the NG-RAN sharing service charging considering REQ-CH_ SATAC_RS-01. This investigation covers the following:

-	Determination of which NF in the 5G system are suitable to provide the charging information to support the NG-RAN sharing.

6. X.4	Possible solutions

6. X.4.1	Solution #1.1: eNB measure PRB Usage and PDCP PDU Data Volume for NG-RAN sharing charging

6. X.4.1.1	General description

According to TS28.552 (V19.0.0), following information can be measured by RAN.DL Total PRB Usage,UL Total PRB Usage,DL PDCP PDU Data Volume,UL PDCP PDU Data Volume can be used aggregation of charging information in shared RAN Operator for 5G data connectivity usage per Participating Operator for NG-RAN sharing.

5.1.1.2.1	DL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the downlink for any purpose. The measurement is optionally split into subcounters per PLMN ID and per supported S-NSSAI and per QoS resource type

5.1.1.2.2	UL Total PRB Usage

a)	This measurement provides the usage (in percentage) of physical resource blocks (PRBs) on the uplink for any purpose. The measurement is optionally split into subcounters per PLMN ID and per supported S-NSSAI and per QoS resource type

5.1.3.6.1.1	 DL PDCP PDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from GNB-CU to GNB-DU. The measurement is calculated per PLMN ID and per QoS level

5.1.3.6.1.2 UL PDCP PDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from GNB-DU to GNB-CU. The measurement is calculated per PLMN ID and per QoS level.

6. X.4.1.2	Charging procedures for Shared RAN operator charging participating operator

The message flow is used to collect wholesale charging data for Shared RAN operator to charge participating operator

Figure 6.X.4.1.2-1: Charging procedures for Shared RAN operator charging participating operator

1.	CEF subscribes to the notifications about RAN sharing charging from the MnS.

2.	The MnS consumer sends RAN sharing data to the MnS producer.

3.	The MnS producer sends RAN sharing data notification to the CEF.

4ch-a)	Charging Data Request: The CEF generates charging data related to RAN sharing and sends the charging data request including the RAN sharing information (e.g.	PLMN ID, gNB ID, data volume).

4ch-b)	Create CDR: The CHF stores received information and create a CDR.

4ch-c)	Charging Data Response: The CHF informs the CEF on the result of the request.

6. X.4.2	Solution #1.2: eNB measure start and end of service in the Shared NG-RAN for a UE of the Participating Operator

eNB measure the start and end of service in the Shared NG-RAN for a UE of the Participating Operator,and aggregated for each Participating Operator.The message flow is the same as Solution #1.1.

6. X.5	Evaluation

Solutions #1.1, #1.2 all solves Key issue#1.1 or Key issue#1.2.

-	Solution #1.1 can reuse the mechanism in TS28.552 to aggregated PRB Usage and PDCP PDU Data Volume for each PLMN ID.

-	Solution #1.2 need to measure the start and end of service in the Shared NG-RAN for a UE of the Participating Operator, which can be achieved through standardization.

6. X.6	Conclusion

End of change