3GPP TSG-SA5 Meeting #156 	S5-244309

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.879 Add solution for authorization of the external MnS consumer to access the management service API use case

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.879, " Study on OAM for service management and exposure to external consumers".

# 3	Rationale

This pCR proposes to add a solution for the authorization of the external MnS consumer to access the management service API use case in clause 5.1.4 of TR 28.879 [1].

# 4	Detailed proposal

It is proposed that the following changes be made in clause 5.1.4 of TR 28.879 [1].

First Change

### 5.1.4 Use case #4: Authorization of the external MnS consumer to access the management service API

#### 5.1.4.1	Description

After the external MnS consumer has discovered the available management services (now service APIs) at the CCF, the external MnS consumer proceeds to request the CCF for the authorization information to access the service API (see clause 8.11 of TS 23.222 [5] and clause 6.5.2.3 of TS 33.122 [14]) via the CAPIF-1e interface. This authorization information contains the actions (e.g., ALLOW, DENY) the external MnS consumer can perform on the discovered MnS producer. These actions might differ from what the external MnS consumer is authorized to discover from the CCF (depending on the configured discovery policy). However, currently, the CCF does not have this authorization information in order to generate the access token to grant external MnS consumers accessto the discovered service APIs (i.e., the management services).

Furthermore, after receiving the authorization information from the CCF, the external MnS consumer will proceed to access the management service at the MnS producer (the MnS producer is the CAPIF API exposing function (AEF)) via the CAPIF-2e interface. This implies that the MnS producer should be able to support the CAPIF-2e interface; however, it is safe to assume that this is already supported.

#### 5.1.4.2	Potential requirements

PREQ-FS_MExpo-Auth-01: The 3GPP management system shall provide the CCF with the authorization information for an external MnS consumer.

#### 5.1.4.3	Potential solutions

###### 5.1.4.3.i.1	Introduction

This potential solution describes how the CCF can authorize the external MnS consumer to access the management service API.

###### 5.1.4.3.i.2	Description

To authorize an external MnS consumer to consume management service(s) at the MnS producer, the CCF can use the AccessRule class (defined in clause 7.3.3 in TS 28.319 [29]) to define the access rule for authorization. This access rule is identified by the ruleName attribute. The dataNodeSelector attribute of the AccessRule class can be used to select the resources to which the access rule will apply. These resources are either the managed objects (IOCs), instances of managed objects (MOIs), or their corresponding attribute(s).

For the selected resources under the dataNodeSelector attribute, the Operations attribute is used to specify the operations (as specified in clause 11.1.1 in TS 28.532[17]) that can be applied to these resources.

The actions attribute authorizes the external MnS consumer to consume the management service. This attribute permits the external MnS consumer to either use all the allowable operations on the selected resources or deny access to all the allowable operations on the selected resources.

For a given access rule, the CCF can associate this access rule to a role using the Role class (defined in clause 7.3.2 in TS 28.319 [29]). This role identified by the roleName attribute could be associated with the external MnS consumer using the Identity class (defined in clause 7.3.1 in TS 28.319 [29]).

The access rule(s) associated with the external MnS consumer could be configured in the CCF beforehand. In addition, the role(s) associated with the external MnS consumer could also be configured in the CCF.

To request for authorization to consume a specific MnS API, the external MnS consumer sends a "service API authorization request" (see clause 8.11 of TS 23.222[5]) to the CCF. The request should include the information provided in Table 5.1.4.3.i.2-1.

Table 5.1.4.3.i.2-1: Access token request message parameters (Table extract from Table C.3.2-1 in TS 33.122 [14])

| Parameter | Values | Comments |
| --- | --- | --- |
| grant_type | REQUIRED. The value shall be set to "client_credentials or "authorization_code". | Should be set to "client_credentials" |
| client_id | REQUIRED. The identifier of the API Invoker making the request. It shall match the value that was assigned to the API Invoker during the onboarding process. | This is the same as the API invoker Id. |
| client_cred | OPTIONAL. The client credential that was provided to the API Invoker during the onboarding process. |  |
| Redirect_uri | OPTIONAL. The value shall be identical with the value in authorization request once authorization code grant or PKCE is used. |  |
| code | OPTIONAL. The authorization code received from the CCF for RNAA once authorization code grant or PKCE is used. |  |
| code_verifier | OPTIONAL. If the authorization code grant with PKCE flow is selected, the code verifier is used by the CCF to check the code_challenge according to IETF RFC 7636 [11] once PKCE is used. |  |
| scope | OPTIONAL. A string containing a space-delimited list, comprising of the following as scopes associated with this token: - List of Services per AEF (e.g. “AEF1:Service1,Service2,Service3,...,ServiceX; AEF2:Service1,Service2,Service3,...,ServiceZ”) |  |

When the CCF receives the "service API authorization request" (see clause 8.11 of TS 23.222[5]) from the external MnS consumer, the CCF first validates the request. If the request is valid, the CCF can:

1) Package the authorization information (i.e., the access rules) associated with the external MnS consumer to generate the token claims of the access token associated with the external MnS consumer.

2) Use the role associated with the external MnS consumer to generate the access token.

Upon generating the access token, the CCF sends the "service API authorization response" with the access token information to the external MnS consumer. The generated access token is structured according to the information provided in Table 5.1.4.3.i.2-2.

Table 5.1.4.3.i.2-2: Access token response message parameters (Table extract from Table C.3.3-1 in TS 33.122 [14])

| Parameter | Values | Comments |
| --- | --- | --- |
| access_token | REQUIRED. This is the issued access token. |  |
| Refresh_token | OPTIONAL. This is the issued refresh token. |  |
| token_type | REQUIRED. This field shall be "bearer" |  |
| expires_in | REQUIRED. The lifetime in seconds of the access token. |  |
| scope | OPTIONAL. The granted scope by the CAPIF core function. | This could be either the access rules or role(s) associated with the external MnS consumer. |

To consume the management service, the external MnS consumer provides the generated access token to the MnS producer via the CAPIF-2 interface (following the procedures described in clause 8.16 of TS 23.222[5]).

End of Change