3GPP TSG-SA5 Meeting #156 	S5-244310

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	Nokia

# Title:	Discussion on cloud-native design principles and their relevance to 3GPP OAM

# Document for:	Discussion and decision

Agenda Item:	6.19.6

# 1	Decision/action requested

The group is asked to discuss the presented observations and decide on the way forward

# 2	References

[1] 3GPP TR 28.869, " Study on cloud aspects for management and orchestration."

# 3	Rationale

TR 28.869[1] currently refers to the term cloud-native network functions throughout the document. The cloud-native network functions have been linked to cloud-native design principles without a proper definition of what these cloud-native design principles are. In addition, it remains unclear what the relevance of the cloud-native design principles is to the 3GPP management system.

Understanding the cloud-native design principles is relevant to the work done in clause 5.2 of TR 28.869[1], which focuses on the "Use of industry solutions for the management of cloud-native network functions." Documenting the relevance of cloud-native design principles to 3GPP OAM will aid the ongoing discussions on the definition of cloud-native NF.

# 4	General questions to the group

In addition, as a group, we need to consider the following questions in mind when discussing cloud-native network functions:

1) What is so special about cloud-native network functions, and why should 3GPP SA5 care? i.e., From the management system, what needs to be done to manage "cloud-native network functions"? The 3GPP management system can manage any network function today. Perhaps it's easier to manage and orchestrate a cloud-native network function than just any generic function?

2) According to the SBMA principles (TS28.533), any entity can produce a 3GPP MnS (aka a 3GPP management API), and any authorized (by the NOP) entity can consume 3GPP MnS. There is nothing special with regards to cloud-native or not that impacts those principles.

3) If (1) and (2) are true, perhaps there are opportunities for simplification in the 3GPP management system that deals only with cloud-native network functions. Do we want to extend the standard to leverage such opportunities in such a case?

4)  The implementor's decision whether to produce or consume a 3GPP MnS is orthogonal (i.e., completely independent) to the implementor's decision whether to follow cloud-native principles and make their function cloud-native. Do we want to tightly couple the 3GPP MnS with cloud-nativeness and eliminate the current flexibility?
Note: The 3GPP MnSs do not violate any cloud-native design principles.

# 5	Detailed proposal

Proposal 1: Agree that capturing cloud-native design principles in TR 28.869 as background information is useful.

Proposal 2: Consider evaluating cloud-native design principles from the perspective of relevance to 3GPP and 3GPP OAM and document the evaluation result in TR 28.869.

Proposal 3: Address the questions in clause 4 and capture the relevant answers in TR 28.869.

End of Change