3GPP TSG-SA5 Meeting #156 	S5-244315

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

# Source:	Nokia

# Title:	Rel-19 pCR TR 28.869 Add use case for scaling of cloud-native NFs

# Document for:	Approval

Agenda Item:	6.19.6

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1] 3GPP TR 28.869, " Study on cloud aspects for management and orchestration."

# 3	Rationale

This pCR proposes to add a use case to clause 5.2 of TR 28.869 [1] to enable the 3GPP management system to initiate scaling actions for specific cloud-native NF(s).

# 4	Detailed proposal

It is proposed that the following changes be made to clause 5.2 of TR 28.869 [1].

First Change

### 5.2.x	Use case #x: Scaling of cloud-native network functions

#### 5.2.x.1	Description

Currently, most management and orchestration entities support the auto-scaling functionality for cloud-native NF instances. The auto-scaling functionality supports the ability to automatically scale up or down a cloud-native network function instance depending on the load of the cloud-native NF.

Accordingly, there might be scenarios where the 3GPP management system needs to interact with the orchestration and management entity to request the scaling (up or down) of a specific cloud-native NF instance based on an already defined scaling descriptor.

NOTE:	The auto-scaling functionality referred to in this use case is focused on horizontal auto-scaling (i.e., increasing or decreasing the number of running cloud-native NF instance(s) depending on the defined auto-scaling parameters in the scaling descriptor).

#### 5.2.x.2	Potential requirements

REQ-AUTO-SCALING-1: The xyz reference point should have the capability enabling the 3GPP management system to request the scaling (up or down) of specific cloud-native NF instance(s).

REQ-AUTO-SCALING-2: The xyz reference point should have the capability enabling the orchestration and management entity to inform the 3GPP management system of the result of the scaling (up or down) of specific cloud-native NF instance(s) request.

REQ-AUTO-SCALING-3: The xyz reference point should have the capability enabling the 3GPP management system to disable the auto-scaling functionality for specific cloud-native network function(s).

REQ-AUTO-SCALING-4: The xyz reference point should have the capability enabling the orchestration and management entity to inform the 3GPP management system of the result of the request to disable the auto-scaling functionality for specific cloud-native network function(s).

End of Change