3GPP TSG-SA5 Meeting #156	S5-244370

# Maastricht, The Netherlands, 19 - 23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0086 | rev |  | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.538 Correct YAML with attribute properties isReadable and isWritable |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson-LG Co., LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2024-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Attribute properties "isReadable" and "isWritable" are specified in stage 2, however they are not mirrored in Open API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct YAML with attribute properties "isReadable" and "isWritable" values already specified in Stage 2 for the cases where their values is different than the default value as defined in TS 32.160 clause 6.1.11.8 solution set mapping rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | leads to incorrect and incomplete implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge file: TS28538_EdgeNrm.yaml |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... S5-242147 |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1245 at commit 333546e9a1fd3e6a8f3f1d7ab51f3bc8f2e2e3a5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

*** START OF CHANGE 1 ***

*** OpenAPI/TS28538_EdgeNrm.yaml ***

<CODE BEGINS>

openapi: 3.0.1

info:

title: 3GPP Edge NRM

version: 18.7.0

description: >-

OAS 3.0.1 specification of the Edge NRM

© 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.538; Edge NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.538/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

ServingLocation:

type: object

properties:

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

topologicalLocation:

$ref: '#/components/schemas/TopologicalServiceArea'

TopologicalServiceArea:

type: object

properties:

cellIdList:

type: array

items:

type: integer

trackingAreaIdList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

servingPLMN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/PlmnId'

GeoLoc:

type: object

properties:

geographicalCoordinates:

$ref: '#/components/schemas/GeographicalCoordinates'

civicLocation:

type: string

GeographicalCoordinates:

type: object

properties:

latitude:

type: integer

longitude:

type: integer

EDNConnectionInfo:

type: object

properties:

dNN:

type: string

eDNServiceArea:

$ref: '#/components/schemas/ServingLocation'

AffinityAntiAffinity:

type: object

properties:

affinityEAS:

type: array

items:

type: string

antiAffinityEAS:

type: array

items:

type: string

VirtualResource:

type: object

properties:

virtualMemory:

type: integer

virtualDisk:

type: integer

virtualCPU:

type: string

vnfdId:

type: string

SoftwareImageInfo:

type: object

properties:

minimumDisk:

type: integer

minimumRAM:

type: integer

discFormat:

type: string

operatingSystem:

type: string

swImageRef:

type: string

Duration:

type: object

properties:

startTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

EASServicePermission:

type: string

description: any of enumerated value

enum:

- TRIAL

- SILVER

- GOLD

EASFeature:

type: string

EASStatus:

type: string

description: any of enumerated value

enum:

- ENABLED

- DISABLED

ResourceReservationRequirement:

type: object

properties:

computeRequirement:

type: string

storageRequirement:

type: string

networkingRequirement:

type: integer

ResourceReservationStatus:

type: object

properties:

resourceId:

type: string

readOnly: true

reservationStatus:

type: string

readOnly: true

description: any of enumrated value

enum:

- RESERVED

- USEd

RelocationTriggerInfo:

type: object

properties:

triggerType:

type: string

description: any of enumrated value

enum:

- IMMEDIATE

- FUTURE

futuristicTriggerTime:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

AvailableEDNList:

type: object

properties:

resourceQuota:

$ref: '#/components/schemas/VirtualResource'

FederatedECSInfo:

type: object

properties:

federateECSIdentifier:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

federatedECSProfile:

type: integer

servedEASList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

servedEESList:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

#-------- Definition of types for name-containments ------

SubNetwork-ncO-EdgeNrm:

type: object

properties:

ECSFunction:

$ref: '#/components/schemas/ECSFunction-Multiple'

EdgeDataNetwork:

$ref: '#/components/schemas/EdgeDataNetwork-Multiple'

#-------- Definition of concrete IOCs --------------------------------------------

EdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

ednIdentifier:

type: string

eDNConnectionInfo:

$ref: '#/components/schemas/EDNConnectionInfo'

- type: object

properties:

EASFunction:

$ref: '#/components/schemas/EASFunction-Multiple'

EESFunction:

$ref: '#/components/schemas/EESFunction-Multiple'

availableEdgeVirtualResources:

type: string

readOnly: true

EASFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eASIdentifier:

type: string

readOnly: true

eESAddress:

type: array

items:

type: string

eASRequirementsRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

eASAddress:

type: array

items:

type: string

relocationTriggerInfo:

$ref: '#/components/schemas/RelocationTriggerInfo'

relocationRejectByASP:

type: boolean

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASProfile-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

aCID:

type: string

readOnly: true

eASProvider:

type: string

readOnly: true

eASdescription:

type: string

readOnly: true

eASSchedule:

$ref: '#/components/schemas/Duration'

readOnly: true

eASGeographicalServiceArea:

$ref: '#/components/schemas/GeoLoc'

readOnly: true

eASTopologicalServiceArea:

$ref: '#/components/schemas/TopologicalServiceArea'

readOnly: true

eASServicePermissionLevel:

$ref: '#/components/schemas/EASServicePermission'

readOnly: true

eASFeature:

$ref: '#/components/schemas/EASFeature'

readOnly: true

eASServiceContinuitySupport:

type: boolean

readOnly: true

eASDNAI:

type: string

readOnly: true

eASAvailabilityReportingPeriod:

type: integer

readOnly: true

eASStatus:

$ref: '#/components/schemas/EASStatus'

readOnly: true

EESFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eESIdentifier:

type: string

eESServingLocation:

type: array

items:

$ref: '#/components/schemas/ServingLocation'

eESAddress:

type: array

items:

type: string

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

serviceContinuitySupport:

type: boolean

eASFunctionRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ECSFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eCSAddress:

type: string

providerIdentifier:

type: string

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

eESFuncitonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

sharedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASRequirements-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

requiredEASservingLocation:

$ref: '#/components/schemas/ServingLocation'

affinityAntiAffinity:

$ref: '#/components/schemas/AffinityAntiAffinity'

serviceContinuity:

type: boolean

virtualResource:

$ref: '#/components/schemas/VirtualResource'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

eASSchedule:

$ref: '#/components/schemas/Duration'

eASFeature:

$ref: '#/components/schemas/EASFeature'

relocationPolicy:

type: string

description: any of enumrated value

enum:

- YES

- NO

- YESwNOTIFY

federationID:

type: string

eASDeploymentMonitor:

$ref: 'TS28623_GenericNrm.yaml#/components/schemas/ProcessMonitor'

reservationJobRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

EASResourceReservationJob-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

reservationLocation:

$ref: '#/components/schemas/ServingLocation'

resourceReservationRequirement:

type: array

items:

$ref: '#/components/schemas/ResourceReservationRequirement'

requestedReservationExpiration:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

resourceReservationStatus:

type: array

readOnly: true

items:

$ref: '#/components/schemas/ResourceReservationStatus'

EdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

participatingOPiD:

type: string

leadingOPiD:

type: string

federatedECSInfo:

$ref: '#/components/schemas/FederatedECSInfo'

OperatorEdgeFederation-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

federationID:

type: string

FederationExpiry:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DateTime'

leadingOPiD:

type: string

avaibleEDNList:

$ref: '#/components/schemas/AvailableEDNList'

acceptedEDN:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

OperatorEdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

availableEASResource:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

EASFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EASFunction-Single'

ECSFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ECSFunction-Single'

EESFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EESFunction-Single'

EdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/EdgeDataNetwork-Single'

EASProfile-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

EdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/EASProfile-Single'

OperatorEdgeFederation-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeFederation-Single'

OperatorEdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

#--------------------------------- Definition ------------------------------------

resources-edgeNrm:

oneOf:

- $ref: '#/components/schemas/EASFunction-Single'

- $ref: '#/components/schemas/ECSFunction-Single'

- $ref: '#/components/schemas/EESFunction-Single'

- $ref: '#/components/schemas/EdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASRequirements-Single'

- $ref: '#/components/schemas/EASProfile-Single'

- $ref: '#/components/schemas/EdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeFederation-Single'

- $ref: '#/components/schemas/OperatorEdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASResourceReservationJob-Single'

<CODE ENDS>

*** END OF CHANGE 1 ***

End of changes