3GPP TSG-SA5 Meeting #156 	S5-244426

Maastricht, The Netherlands, 19 - 23 August 2024

# Source:	Ericsson España S.A.

# Title:	pCR TR 28.879 Discovery policy configuration – update solution 3 description and add evaluation

# Document for:	Approval

Agenda Item:	6.19.21

# 1	Decision/action requested

The group is asked to discuss and approve the proposal.

# 2	References

[1] 3GPP TR 28.879: " Study on OAM for service management and exposure to external consumers".

[2] S5-243256: “pCR TR 28.879 Configuring discovery policy for an external MnS consumer”

# 3	Rationale

Clause 5.1.3 in TR 28.879 [1] focuses on configuring discovery policy use case. This policy, defined during the API invoker enrolment phase, allows to specify the visibility for an API invoker, and therefore filter which service API information this API invoker can i) subscribe to, during the onboarding phase; and ii) discover and subsequently access, once API invoker gets onboarded

Status: At SA5#155, a first draft of configuring discovery policy use case was agreed in S5-243256 [2]. This first draft contains:

- Description (clause 5.1.3.1)

- Potential requirements (clause 5.1.3.2)

- Potential solutions (clause 5.1.3.3), with three agreed solutions: “potential solution #1: using AccessRule class for discovery policy definition”, “potential solution #2: using Role class for discovery policy definition”, “potential solution #3: mapping policy definition into OAuth2.0 access token”.

- Evaluation of potential solutions (clause 5.1.3.4), with a first assessment of three agreed solutions.

Gaps:

- Through the entire use case, it is recurrently mentioned that defined discovery policy shall be mapped into access token. The current wording may read that this mapping is done in the discovery policy configuration use case, which is wrong; actually, this mapping is done into the authorization use case (see clause 5.1.4), which is when the access token is issued and enforced. What is in the scope of the discovery policy configuration is to define the discovery policy such that constituent rules can be then mapped to access token scope values. It is proposed to update the text to avoid misinterpretations.

- In clause 5.1.3.4, the evaluation done for solution #3 is poor and incomplete. If conclusions and recommendations are to be endorsed, further assessment is needed.

# 4	Detailed proposal

The present pCR aims to bridge the issues listed in the gap, by making the following changes in the latest version of TR 28.879 [1].

For gap #1:

- In the clause 5.1.3.1, improve the sentence “The policy configuration can be done through OAuth2.0 access token, by mapping discovery policy into token scope values”, to avoid misinterpretations.

- In the clause 5.1.3.2, update “PREQ-FS_MExpo-Disc-06”

- In the clause 5.1.3.3, update and name description for solution #3

For gap #2, add evaluation for solution #3 in clause 5.1.3.4.

Begin Change

### 5.1.3	Use case #3: Configuring discovery policy for an external MnS consumer.

#### 5.1.3.1		Description

The operator may not want that all service APIs published into CAPIF are discoverable to all API invokers; in fact, the operator may want to limit the visibility that certain API invokers have over published API information, according to the business agreements settled with the stakeholder owning the API invoker. To that end, the CCF shall be able to be configured with discovery policy on a per API invoker basis. This policy, defined during the API invoker enrolment phase, allows to specify the visibility for an API invoker, and therefore filter which service API information this API invoker can:

-	Subscribe to, during the API invoker onboarding. The CAPIF administrator uses the discovery policy and other enrolment information to generate an onboarding credential (OAuth2.0 access) of the API invoker. This credential and other CCF details (address, root CA certificate) are sent to the API invoker, so that it can reach out to CCF to initiate the onboarding (e.g., the API invoker becomes a valid CAPIF user).

-	Discover and subsequently access, once API invoker gets onboarded. Upon receiving a discovery request from the API invoker with certain query information (i.e., criteria for discovery matching service APIs), the CCF will apply the discovery policy on the search results matching the query criteria and filter them accordingly.

It is worth noting that 3GPP specifications do not provide details nor guidelines for discovery policy definition; actually, the policy format (information elements and constraints) is entirely to the operator discretion.

When using the CAPIF as the framework to expose MnSs, the external MnS consumer plays the role of API invoker, and the published API information corresponds to the MnS information published into CCF compliant with ServiceAPIDescription (clause 8.2.4.2.2 from [5]). Likewise, for the cases where CAPIF and 3GPP management system belong both to the same administrative domain, the 3GPP management system administrator [29] also acts as CAPIF administrator.

However, there are two issues that deserve further analysis.

On the one hand, it is needed to discuss how 3GPP management system can help define discovery policies for each external MnS consumer. The aim of these policies is to limit their visibility over the published MnS information, either at onboarding time (when the external MnS consumer becomes an onboarded API invoker) or at operation time (when the external MnS consumer issues discovery requests). The discovery policies can define filters at two levels:

-	First, the policy allows configuring which resources are made discoverable to the external MnS consumer. For each MOI which is under the management scope of the published MnS, the policy specifies whether the MOI is visible or not.

-	Secondly, for each visible MOI, the policy allows limiting what the external MnS consumer can see regarding:

-	Attributes supported by the MOI.

-	CRUD operations and/or notifications associated to the MOI.

-	Management data, e.g. performance and fault information, associated to the MOI.

On the other hand, it is needed to discuss how a defined discovery policy can be configured on the CCF, so that this policy can be initially associated to the API invoker during the enrolment phase. This association allows setting the visibility that this API invoker will have over its lifetime, from onboarding to offboarding, including all stages in between; if needed, the policy can be updated over the API invoker’s lifetime. The policy configuration can be done, for example by the CAPIF administrator, for the cases where this administrator is also administrator of the 3GPP management system (see TS 28.319 [29]). This configuration will allow CCF to be prepared for authorization use case (see clause 5.1.4), where OAuth2.0 access tokens will be issued.

2nd Change

#### 5.1.3.2	Potential requirements

PREQ-FS_MExpo-Disc-01: The 3GPP management system shall provide the capability to configure which MOIs, among those ones that are within the scope of a published MnSs, are visible to an external MnS consumer. The filtered set of MOIs is referred to as discoverable MOIs.

PREQ-FS_MExpo-Disc-02: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which attributes supported by this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-03: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which CRUD operations associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-04: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which notifications associated to this MOI are visible to an external MnS consumer.

PREQ-FS_MExpo-Disc-05: The 3GPP management system shall provide the capability to configure, for a discoverable MOI, which management data associated to this MOI are visible to an external MnS consumer.

NOTE: The above listed requirements are based on those originally defined in 3GPP TS 28.319 [29], but now applicable for external MnS consumers.

PREQ-FS_MExpo-Disc-06: The 3GPP management system shall make sure that granularity of discoverable MOIs is appropriate for generating OAuth2.0 access tokens.

NOTE: OAuth2.0 access tokens are generated and used during authorization use case (see clause 5.1.4). Authorization follows discovery, which means access token grant access to discoverable MOIs, i.e. discoverable resources. The granularity of discoverable MOIs shall be such that access tokens can be generated during authorization use case (see clause 5.1.4).

2nd Change

#### 5.1.3.3	Potential solutions

##### 5.1.3.3.1	Potential solution #1: Using AccessRule class for discovery policy definition

###### 5.1.3.3.1.1	Introduction

AccessRule class (TS 28.319 [29], clause 7.3.3) enables providing granular control on what actions are visible over which resources, when certain conditions (filter criteria) are met. The table below provides a summary of the class attributes.

| Attribute Name | S |
| --- | --- |
| ruleName | M |
| dataNodeSelector | M |
| operations | M |
| actions | O |
| componentCData | O |

The different AccessRule instances are defined at design time and stored in a database accessible by 3GPP management system. An example of this database can be the operator’s Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing AccessRule class to define discovery policies for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

###### 5.1.3.3.1.2	Description

It is proposed to use AccessRule to generate discovery policies as follows:

-	 “dataNodeSelector”: it can be used not only to configure which MOI are visible (first level of filter), but also the visibility over the attributes of these MOIs. This allows fulfilling PREQ-FS_MExpo-Disc-01 and PREQ-FS_MExpo-Disc-02.

-	 “operations”: it can be used to specify which CRUD operations and/or notifications are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-03 and PREQ-FS_MExpo-Disc-04.

-	 “componentCData”: it can be used to specify which management data are visible for each MOI listed in the dataNodeSelector. This allows fulfilling PREQ-FS_MExpo-Disc-05.

-	 “actions”: this attribute is not useful for the discovery policy configuration use case. The usage of this attribute is to grant authorization to the external MnS consumer when accessing service APIs over CAPIF-2/2e interface. However, this is a separate use case.

How “dataNodeSelector”, “operations” and “componentCData” are specifically mapped to generate the policy depends on the policy format decided by the operator. This format is not standardized.

##### 5.1.3.3.2	Potential solution #2: Using Role class for discovery policy definition

###### 5.1.3.3.2.1	Introduction

Role class (TS 28.319 [29], clause 7.3.2) enables capturing multiple access rules. The table below provides a summary of the class attributes.

| Attribute Name | S |
| --- | --- |
| roleName | M |
| Attribute related to role |  |
| accessRulesList | M |

The different Role instances are defined at design time and stored in a database accessible by 3GPP management system. Examples of this database can be the operator’s Authentication, Authorization and Accounting (AAA) server.

This solution proposes using the existing Role class to define discovery policies for external MnS consumer. With this solution, it is proposed to fulfil the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05.

###### 5.1.3.3.2.2	Description

This solution is similar as in clause 5.1.3.3.1, with the exception that the policy conveys information from the access rules associated to this role.

##### 5.1.3.3.3	Potential solution #3: Granularity on discovery policy definition.

###### 5.1.3.3.3.1	Introduction

This solution focuses on meeting the requirement PREQ-FS_MExpo-Disc-06.

###### 5.1.3.3.3.2	Description

The discovery policy is configured on the CCF, so the CCF can limit what the external MnS consumer can subscribe to (during the API invoker onboarding) and discover and subsequently access (once the API invoker gets onboarded). The rules contained in a discovery policy can be more or less granular; the more granularity, the more fine-grained the control on external MnS consumer’s visibility (i.e. what the consumer can discover), at the cost of increasing complexity of external MnS consumer’s authorization (i.e. what the consumer can access). The authorization is performed with an  OAuth 2.0 access token. The components of this token are shown in the table below (see Table C.2.2-1 from 3GPP TS 33.122 [14]).

| Parameter | Description |
| --- | --- |
| exp | REQUIRED. The expiration time of the access token. Implementers MAY provide for some small leeway, usually no more than a few minutes, to account for clock skew (not to exceed 30 seconds). |
| client_id | REQUIRED. The identifier of the API Invoker making the API request as previously established with the CAPIF Core Function through onboarding. |
| scope | REQUIRED. A string containing a space-delimited list, comprising of the following as scopes associated with this token: - List of Services per AEF (e.g. “AEF1:Service1,Service2,Service3,...,ServiceX; AEF2:Service1,Service2,Service3,...,ServiceZ”) |

The “scope” parameter contains OAuth2.0 scopes typically following the convention “scope value = <resource>:<access>”, where <resource> component refers to the object the scope grants access to, and <access> component refers to the operation(s) the scope authorized upon that resource. The <<resource>> component references discoverable MOIs, which means this component can be mapped from “dataNodeSelector” attribute. According to clause 7.3.3 from TS 28.319 [29], the “dataNodeSelector” can be used to represent either i) IOCs, i.e., whole classes; ii) MOIs, i.e. specific instances of classes with a known DN value; and iii) XPATH or JEX expressions.

The granularity used for “dataNodeSelector” definition determines the granularity over which authorization will be applied later on, and therefore how granular tokens should be emitted and enforced. A right level of granularity is fundamental to ensure fulfilment of PREQ-FS_MExpo-Disc-06; actually, too much granularity may risk this fulfilment, leading to noteworthy issues during authorization (see issues reported in clause 5.1.4).

3nd Change

#### 5.1.3.4	Evaluation of potential solutions

Solutions #1 and #2 allow fulfilling the requirements PREQ-FS_MExpo-Disc-01, PREQ-FS_MExpo-Disc-02, PREQ-FS_MExpo-Disc-03, PREQ-FS_MExpo-Disc-04 and PREQ-FS_MExpo-Disc-05, providing controllable granularity on which published MnS information is discoverable by an external MnS consumer.

###### 5.1.3.4.x	Evaluation of potential solution #3

Solution #3 reports that fulfilment of PRE-FS_MExpo-Disc-06 is subjected to granularity chosen for discovery policy definition, i.e. granularity of contained rules. The solution notes that the authorization is applied over discoverable resources, which means that “dataNodeSelector” attribute (used to specify discoverable resources) can be used to construct the scope’s <resource> component of OAuth2.0 access tokens (used to specify resources subjected to authorization). The more granular the “dataNodeSelector”, the more specific the OAuth2.0 scope, and the more the number of scopes needed to cover the same permission space. There are two aspects that are noteworthy.

First, there is a need to make sure that the granularity of resources specified with “dataNodeSelector” fits the granularity allowed by the authorization system; otherwise, access token needed for authorization phase may be compromised (see clause 5.1.4).

Secondly, there is a need to make sure that “dataNodeSelector” attribute is semantically mappable to the <resource> component of OAuth2.0 scope. For example, when using JEX for “dataNodeSelector”, the MnS producer needs to be able to map JEX expression components (see TS 32.161) into <resource> component  (string type). For examples, for JEX expressions containing predicates, the MnS producer needs to resolve expressions at run-time to get the targeted resources, and then mapping these resources into a string.